State of Washington

Current Contract Information (CCI)

Revised date:  April 09, 2009

Effective date:  April 09, 2009

| Contract number: | 00405 |  | Commodity code: |  |  |  | 9330 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contract title: | Lottery Scratch Ticket Dispensers |  |  |  |  |  |  |
| Purpose: | To extend contract for an additional period. |  |  |  |  |  |  |
| Original Award Date: | April 01, 2005 |  |  |  |  |  |  |
| Term: | April 09, 2009 | through |  |  | Implementation of new WSCA Contract |  |  |
| For use by: | Washington Lottery |  |  |  |  |  |  |
| Contract type: | This contract is designated as mandatory use. |  |  |  |  |  |  |
| # Scope of contract | This contract is awarded to one contractor. |  |  |  |  |  |  |
| Contractor : Address: | Take-A-Ticket, Inc. 130 NE Montgomery Street Albany, Oregon 97321-2723 |  |  | Contact: |  | Order Placement: Tami Dennis Contract Administrator: Calvin Tigner |  |
| Phone: |  |  |  |  |  | 800-253-4295 541-967-0433 |  |
| FAX: |  |  |  |  |  | 541-967-8415 |  |
| E-Mail: |  |  |  |  |  | info@tatinc.com calvin@tatinc.com |  |
| Fed. I.D. No.: |  |  |  |  |  | 93-0923470 |  |

This page contains key contract features.  Find detailed information on succeeding pages.  For more information on this contract, or if you have any questions, please contact your local agency Purchasing Office or you may contact our office at the numbers listed below.

| Contracts Specialist: | Tim Shay | Alternate Contact: | Shawna Pettitt |
| --- | --- | --- | --- |
| Phone Number: | (360) 902-7431 | Phone Number: | (360) 902-7342 |
| Fax Number: | (360) 586-2426 | Fax Number: | (360) 586-2426 |
| Email: | tshay@ga.wa.gov | Email: | spettit@ga.wa.gov |

Visit our Internet site: http://www.ga.wa.gov/purchase/

| Payment address: | 130 NE Montgomery Street, Albany OR 97321-2723 |  |  |  |
| --- | --- | --- | --- | --- |
| Order placement address: | 130 NE Montgomery Street, Albany OR 97321-2723 |  |  |  |
| Minimum orders: | None. |  |  |  |
| Delivery time: | 30 calendar days After Receipt of Order (ARO) or from approval of pre-production sample (first time orders). Thereafter, 20 days ARO |  |  |  |
| Payment terms: | Net 30 days. |  |  |  |
| Shipping destination: | Free On Board (FOB) Destination |  |  |  |
| Freight: | Prepaid and included in unit pricing |  |  |  |
| Credit Card acceptance: | Yes (VISA) |  |  |  |
| Contract pricing: | See attached |  |  |  |
| Term worth: | $163,351.00/$160,000.00/one year |  |  |  |
| Current participation: | $0.00 MBE | $0.00 WBE | $163,351.00/$160,000.00 OTHER | $0.00 EXEMPT |
|  | MBE 0% | WBE 0% | OTHER 100% | Exempt 0% |
| Recovered material : | ABS utility grade SP-321 Acrylonitrile Rutadiene Styrene contains 75-100% recycled content |  |  |  |

Notes:

- Ordering:  Washington Lottery: submit order directly to Contractor for processing.  Contractors shall not process state contract orders from unauthorized users.

Special Conditions:

1. Extend contract term for an additional period, until new WSCA contract has been implemented.

2. All existing terms and conditions as well as pricing will remain unchanged.

SPECIAL TERMS AND CONDITIONS

1. Pre-Production Samples:

Requests and Disposition:  the state reserves the right to request pre-production samples before or after contract award. The Washington Lottery must approve pre-production sample(s) prior to manufacture, especially for new or revised items.  If not destroyed in testing or required for quality control, contractor may request return of samples at their expense. If request for return is not received within sixty days, samples will be considered the property of the Washington Lottery.

2.  Packaging:

Shipments are to be carton packed, and each carton shall not exceed forty pounds in weight. Cartons shall equal or exceed federal specifications.

3.  Service Expectation:

Contractor’s manufacturing facility must be open during normal work hours and have an adequate number of employees to assist Washington Lottery.  Contractor employees shall be familiar with display dispensers. The contractor is expected to assist Washington Lottery in making cost effective purchases.  This assistance may include new product technology and economical order quantities.

4. AGENCY USER LOCATIONS

The following distribution centers have been identified as potential ship to locations for this contract.

10517 E Sprague Avenue					9 S 5th Avenue

Spokane WA  99206					Yakima WA  98902-3432

(No loading dock)						(No loading dock)

5963 Corson Avenue S, Suite 106			7860 29th Avenue NE Unit C

Seattle WA 98108-2611					Olympia WA 98516-7122

(No loading dock)

814 4th Avenue E						1503 NE 78th St Ste 4

Olympia WA 98506					Vancouver WA 98665-9668

(No loading dock)

11419 19th Avenue SE, Suite A106

Everett, WA  98208-5120

(No loading dock)

## SPECIFICATIONS

Items 1 & 2; Vertical Dispenser; Two and Three Game. Take-A-Ticket Model 100 Series (120 and 130)

Unit has been designed to display and dispense two games simultaneously in one ultra-low profile vertical dispenser.

The ultra-low design shall hold and dispense two, three or four complete packs of up to #500 instant tickets.

Dimensions: Item #1: length 14”; width 4-1/2”, and a sloping height to 8” at the back (Model 120) Item #2: length 14”; width 4-1/4”, and a sloping height to 10” at the back (Model 130) The sloped top shall be designed such as to prevent placement of any object which might obstruct the visibility of the display of tickets.

The apparent edges of all instant ticket dispensers shall be rounded and smooth to avoid any physical harm to retail employees or consumers.

The dispenser shall consist of two separate pieces; an outer shell and removable drawer:

The outer shell’s top, front, and base shall be fabricated of .150” clear Vivak (all one piece), and the sides shall be fabricated of 1/8” clear Vivak.

The removable drawer door and shelves shall be fabricated of .150” durable clear Vivak. The side plates on the removable drawer shall be fabricated of 1/8” clear Vivak. The removable drawer shall be fabricated of Vivak due to amount of use and accidental dropping that may occur in the loading process.

Double-sided, industrial strength adhesive tape; two (2) pieces of 2” x 4” x 1/32” on each end and one (1) piece of 1” x 4” x 1/32” in the center (Item #1) and three (3) pieces of 2” x 4” x 1/32” on each end and in the center (Items #2) shall be included for installation on the retail agent’s counter.

The removable drawer shall be located in back for easy loading and dispensing of tickets, and to prevent unauthorized ticket access and shall be secured in the dispenser with its bolt-action lock (includes two (2) keys.)

Loading and operating instructions shall be included with each unit.

Items 3, 4 & 5: Series Silent Salesman w/double glass top. Take-A-Ticket Model 400-LSG Series (Models 403-LSG, 404-LSG and 406-LSG)

Dispenser series has been designed for check-writing stands, and on-counter dispensing. With its styling, it proves to be an excellent point-of-purchase (P.O.P) display, while taking up very little counter space.

Ticket Capacity: Series shall hold #3 to #6 game bins wide and each bin easily holds a complete pack of #300+ tickets.

Unit shall be designed to give the optimum space efficiency while allowing a minimum of empty space remaining in each bin of the dispenser when holding full ticket packs.

DIMENSIONS: Dimensions on all units are 12-1/4” deep x 2-5/8” high x the following widths: Item #3: = 15 ¾” wide (3-game, Model 403-LSG) Item #4: = 20” wide (4-game, Model 404-LSG Item #5: = 28-1/2” wide (6-game, Model 406-LSG)

MATERIALS: The unit shall consist of two separate pieces; an outer shell and a removable drawer (tray).

A. OUTER SHELL: The sides and bottom of the shell shall be constructed of 3/16” black, ABS plastic. The frame top (houses the double-glass) shall be constructed of ¼” Black ABS plastic and holds the glass support strips which shall be constructed of 1/8” clear, PETG (Vivak) plastic. The shell lock piece shall be constructed of .150” clear PETG (Vivak) plastic and housed two (2) slots to accommodate the spring-load lock on the removable drawer.

DOUBLE-GLASS TOP: Each unit shall include two (2) pieces of tempered safety glass. Each piece shall be 5/32” thick and measures 10” deep x variety of widths (depending on size ordered.) The bottom piece shall be permanently sealed to the frame top & glass support strips on the outer shell with industrial-strength black double-sided glass tape. The top piece shall be removable for easy cleaning and replacement when necessary. Please note that each unit shall come with a small suction cup for safe and easy removable of top glass.

B. Removable Drawer (Tray): The removable drawer’s sides and bottom shall be constructed of 1/8” Black ABS plastic. The front of the unit (the customer’s view) shall be constructed of 1/8” clear PETG (Vivak) plastic. The ticket bin partitions shall be constructed of .150” clear PETG (Vivak) plastic; while the lock compartment (winning or void ticket may be housed in this compartment) shall have a partition constructed of 3/16” Black ABS plastic.

LOCK POSITIONS: The drawer contains the spring-load lock (two (2) keys provided) which secures the drawer into the outer shell. The spring-load lock allows the clerk to push in & lock the unit without a key, but the key is required to re-open the unit.

The unit shall have two (2) lock positions: - CLOSED LOCKED POSITION – in this position the unit is securely locked and tickets cannot be dispensed (the 2nd slot on the lock piece). 2) OPEN “DISPENSING” POSITION – in this position the unit is still locked (in the 1st slot on the lock piece) but the tickets may be dispensed by the clerk. The clerk can push the unit into the 2nd slot and securely lock the unit without the key.

TICKET TIGHTENER / TEAR BARS: On the drawer shall be a ticket tightener/tear bar assembly constructed of 3/16” & 1/8” black ABS plastic. Glued to the assembly piece shall be clear, injection-molded ticket tightener/tear bars. Each tear bar shall have two (2) ½” clear rollers for smooth ticket dispensing and to prevent any scratching of the latex surface of the tickets. There is NO PLASTIC LIP on the underside of the tear bar assembly to interfere with the dispensing of tickets.

SECURITY BAR: Each unit shall come with a security bar which eliminates any unauthorized access to the tickets. This unit shall be constructed of .090” black ABS plastic and fits securely over the tear bar assembly and can be housed under the tear bar assembly when not in use.

TAPE: For secure and simple installation on the retail agent’s counter, double-sided, industrial strength adhesive tape (1” X 4” X 1/32”) shall be included with each unit.

INSTRUCTIONS: Operation instructions shall be located in the drawer of each unit and shall be included in all units.

PACKAGING: Each unit shall be packaged in a bag and an individual box and then bulk packaged in a case box for delivery.

WARRANTY: The series shall be covered by a one (1) year warranty against defects in material and workmanship. This excludes the replaceable top piece of glass; which may need to be replaced anywhere from three (3) months to one (1) year after initial installation depending on the amount of traffic of the store.

Items 6 & 7: Silent Salesman (6 & 8 Game) – Polycarbonate, Take-A-Ticket Model 406-POLY and 4-8-POLY

This unit has been designed to display and dispense six games simultaneously in one ultra-low profile dispenser.

Dimensions: Item #6: length 11-1/2”; width 26 1/2”; and height 2-1/8” (Model 406-POLY) Item #7: length 11-1/2”; width 35 1/2”; and height 2-1/8” (Model 408-POLY)

It shall accommodate a full 4” x 10” pack of #500 tickets. The apparent edges of all instant ticket dispensers shall be rounded and smooth for safety and a pleasing finished appearance.

This unit shall consist of two separate pieces; an outer shell and a removable drawer:

The top, sides and front of the outer shell shall be fabricated of 3/16” clear unbreakable polycarbonate plastic, and the top shall have a 45 degree cutting edge for easy dispensing. The bottom of the shell shall be fabricated of 1/8” black ABS. The side shall also contain the ¼” x 7/8” slot for the locking bolt.

The removable drawer shall be fabricated of 1/8” black ABS (bottom, back and sides). The front shall be constructed of 1/8” Vivak. The five ticket partitions shall be fabricated of 3/16” clear Vivak. The side of the drawer shall have an integrated housing for the bolt-action lock (includes two keys); and a centered ¾” rounded finger pull.

For secure and simple installation on the retail agent’s counter six (6) pieces (6-game) and ten (10) pieces (8-game) of double-sided, industrial strength adhesive tape (2” x 4” x 1/32”) shall be located on the bottom of the dispenser.

The removable drawer shall be located in back for easy loading and dispensing of tickets, and to prevent unauthorized ticket access, and shall be secured in the dispenser with its bolt-action lock (includes two (2) keys).

Loading and operating instructions shall be include with each unit.

Items 8 & 9: In-Counter Dispensers (front window), Take-A-Ticket Model 400-FW Series (Models 412-FW-20” and 416-FW-20”)

This in-counter dispenser series has been designed to display and dispense twelve (12) or sixteen (16) games simultaneously

The feature on these units is the front window. The front window allows for “selling” tickets, due to the fact that the customers see the tickets through the “front window” as soon as they enter the store or approach the counter.

TICKET CAPACITY/LENGTH: The front window in-counter series shall be available in a #12 or #16 game style and each BIN shall easily hold a complete pack of up to #500 tickets. This series shall be available in the standard 20” size. This unit shall be designed to give the optimum space efficiency while allowing a minimum of empty space remaining in each bin of the dispenser when holding full ticket packs.

DIMENSIONS Item 8: Width = 19-3/4”, Height = 13 ¼”, Length = 22 ¾” Item 9: Width = 19-3/4”, Height = 15 ¾”, Length = 22 ¾” DIMENSIONS INCLUDE OUTSIDE FRAME SIZE & TEAR BAR ASSEMBLIES

MATERIALS: This series consists of: 1) a base unit that includes the double-glass/frame and front window; and 2) removable drawers.

MIRRORED BASE SIDES: All in-counter dispensers shall have approximately 2-1/2 square feet of mirrored plastic on the sides of the base. This shall be a .030"”plastic-backed, adhesive mirror that gives a more overall finished, attractive look and most importantly it enhances and shows off the tickets to their maximum potential.

LIGHT: All in-counter dispensers also shall have a light installed standard in the unit. The light shall be an all-white, 10-bulb electrical light with a 10’ (foot) cord. The light shall have a swivel shield so that it can be positioned to show off the tickets to their maximum. The light shall be installed in the inside top of the unit on the front window.

FRONT WINDOW (REPLACEABLE) All dispensers shall have a double-plastic front window. The window shall consist of two (2) pieces of plastic – a permanent piece of .150” clear Vivak shall be glued to the sides of the base and a front piece shall be a removable, replaceable piece of ¼” clear acrylic plastic that shall be attached with two (2) screws for easy replacement.

2-GLASS (REPLACEABLE) GLASS & FRAME: On this series, the double-glass & frame shall be attached with rivets and screws to the base. The aluminum frame shall hold two (2) pieces of tempered, approved safety glass.

The dispenser shall have a replaceable glass top. It shall consist of two (2) pieces of glass – a permanent piece of 5/32” clear tempered, approved safety glass is permanently sealed to the frame and the top piece is a removable, replaceable piece of 5/32” clear tempered approved safety glass.

Within six months to 1 year after installation, if the top glass or front window becomes marred, scratched or defaced in any way, the top piece of glass can be easily lifted and removed for replacement or cleaning.

A suction cup shall be provided with each unit to easily lift top piece of glass for replacement or cleaning.

The glass shall measure 18-1/8” x 18-1/8” square and shall be attached to a custom-designed aluminum frame that fits snug, flush and securely to the counter surface for easy coin exchange. The frame should then be siliconed (sealed) to the counter to prevent any spills from penetrating into the upper drawer.

REMOVABLE DRAWERS The removable, adjustable drawers, 11-3/4”H models = three (3) drawers and 14-1/2”H Models = four (4) drawers. Each shall be divided into four (4) ticket bins to hold and dispense complete packs of multiple-size tickets; the top drawer shall be designed to hold 4” x 6” bingo tickets.

The drawers shall be constructed of .150” & 3/16” durable clear PETG, 1/8” & 3/16” white ABS, and ¼” clear DR acrylic (unbreakable) rod plastic. The top drawer shall have three (3) Acrylic rods while the other drawers feature two (2) rods to keep the packs down and from interfering when loading and unloading drawers.

TICKET TIGHTENER/TEAR BARS: Each of the bins shall have a ticket tightener/tear bar injection-molded of clear PETG plastic and each shall have two (2) 3/8” white, nylon ROLLERS for smooth dispensing and to prevent tickets from scratching or snapping back into the dispenser.

REPLACEABLE TEAR BAR ASSEMBLY: The ticket tightener/tear bars shall be attached to a .150” clear PETG replaceable tear bar assembly. This assembly holds the four (4) tear bars for each drawer and shall be attached to a vertical brace with two (2) ½” pan-head sheet metal screws for easy replacement. This allows replacement of broken or damaged tear bars without the need to replace the entire drawer.

Each assembly shall have an integrated clear lock drawer clip located on each side of the tear bar assembly. These wrap around the base when the drawers are locked in the base to help prevent any unauthorized access to the unit and prevent the unit from bowing out on the sides when installed in the counter.

TICKET PACK ADJUSTMENT CLIPS (L-CLIPS): Each bin shall have a ticket pack adjustment clip (L-clip) injection-molded of .093” white ABS (break-resistant) plastic and attaches to the drawer with Velcro strips. This clip is used to keep the ticket pack forward (towards the customer) at all times for maximum visibility and to help prevent doubling up of tickets when dispensed.

LOCKS / KEYS: Each drawer shall be equipped with its own bolt-action lock; all drawers shall be keyed alike and each lock comes with two (2) keys. All drawers shall lock easily and securely lock into the lock slot on the base.

INSTRUCTIONS: Operation and loading instructions shall be located in each unit.

BASE EXTENSION: The base shall be constructed of 3/16” white ABS plastic. It shall consist of sides, bottom and a top strip measuring 1” wide. The base shall be length adjustable to accommodate both sizes.

One side shall have ¼” x 1-1/2” lock slots that house the locks when the drawers are installed

FRONT CABINET TRIM: Each unit shall include a front cabinet trim; this trim shall consist of three (3) thin pieces of 1/8” white ABS plastic with double sided adhesive tape. This trim shall be installed around the edge of the front window to cover any blemishes made on the cut-out hole.

PACKAGING: One unit per shipping box (box in a box). The unit shall be packaged in an individual inner box and then repackaged in an outer box with packaging material to prevent any damage in shipping.

WARRANTY: This series shall be covered by a one (1) year warranty against defects in material and workmanship.

This excludes the replaceable top piece of glass & plastic front window; which may need to be replaced anywhere from three (3) months to one (1) year after initial installation depending on the amount of traffic of the store.

ITEMS 13, 14, &15: SINGLE STACKABLE INTERLOCKING DISPENSER, Model 113

Is designed to be stacked in a vertical style. The unit is constructed in a manner that will promote the highest degree of point-of-purchase visibility to the consumer, while taking up very little counter space. Add or delete games with easy interlocking system

Game and Ticket capacity: single game dispenser that is equipped to hold one (1) pack of up to #500 instant scratch tickets. Units may be stacked up to five high to display up to five games vertically or may be stacked two to five high and side by side to display any number of games simultaneously. May also be used in conjunction with other dispensers to display the number of games required.

Dimensions: depth: 7 ½” (includes tear bar), width: 4 ½”, height: 6”

Materials: Cover: injection molded break-resistant clear polycarbonate PETG – DUROLON 2000 clear plastic. Thickness: .125” Door: injection molded clear polycarbonate PETG #6763 clear plastic. Thickness: .125”

Lock: equipped with a bolt action lock included two (2) keys and all locks are keyed alike

Stackable Interlocking Design: unique sliding rail system interlocks one stackable unit with another, and the door lock for each acts as a security lock for the upper dispenser

Installation: only the bottom dispenser requires tape for installation of the retail agent’s counter. Double sided industrial strength adhesive tape is included with every two models. Instruction label included on each unit.

Technical data: construction process including all glued joints, moving parts, materials, injection molds, seamless heat bends, etc are of the highest standard to enhance the longevity of the unit and withstand the heavy wear & tear anticipated in a high volume sales environment. All edges are void of any sharp edges to avoid any physical harm to retail employees and/or consumers and to provide a pleasing finish.

Shipping: two per plastic bag and box, then #20 boxes are placed in a master (case) box for a total of #40 units per master box

Warranty: one year replacement warranty

OPTIONAL BASE: Model CLB is designed to be counter mounted. Does not require adhesive tape. Dimensions: base = 6” wide x 6 ½” long x 2 ½” high. Counter lock is injection molded of .125” ABS black styrene plastic, unbreakable. Base is constructed of 3/16” ABS black styrene and measures 6” wide x 6 ½” long. Lock is located in back where it may prevent unauthorized ticket access. A bolt action lock secures the base to the stacking dispensers without any need for tape. Two keys are provided with each unit. One year warranty.

PEDESTAL: eight inches, constructed of 2” wide plastic pipe with top and bottom plate. Top plate can be flat or angled.

PRICE SHEETS

| Item | Description | Unit | Unit Price |
| --- | --- | --- | --- |
|  | - Dispenser; vertical (two-game), per attached specifications. Take-A-Ticket Model #120 | - EA | - $29.00 |
|  | - Dispenser; vertical (three-game), per attached specifications. Take-A-Ticket Model #130 | - EA | - $32.00 |
|  | - Dispenser; Silent Salesman w/double glass top – 3-Game, per attached specifications. Take-A-Ticket Model #403-LSG. | - EA | - $49.00 |
|  | Dispenser; Silent Salesman w/double glass top – 4-Game, per attached specifications. Take-A-Ticket Model #404-LSG | - EA | - $54.00 |
|  | Dispenser; Silent Salesman w/double glass top – 6-Game, per attached specifications. Take-A-Ticket Model #406-LSG | - EA | - $70.00 |
|  | Dispenser; Silent Salesman, 6-Game, polycarbonate, per attached specifications Take-A-Ticket Model #406 | - EA | - $49.00 |
|  | Dispenser; Silent Salesman, 8-Game, polycarbonate, per attached specifications. Take-A-Ticket Model #408 | - EA | - $135.00 |
|  | Dispenser; In-Counter w/front window, 20” size, per attached specifications. Take-A-Ticket Model #412-FW-20 | - EA | - $210.00 |
|  | Dispenser; In-Counter w/front window, 20” size, per attached specifications. Take-A-Ticket Model #416-FW-20 | - EA | - $235.00 |
|  | - Replacement parts for In-Counter Series: - Drawer Rods & Plugs - Rollers - Tear Bar Assembly - Ticket Pack Adjustment Clips (L-Clips) - Top Glass: - 20” - 18” | - EA - EA - EA - EA - EA - EA | - $.50 - $.00 - $10.00 - $.25 - $18.00 - $18.00 |
| - A | - Replacement drawers for in-counter series: - Drawers for front window in-counters, 12-game, 20”: - Top Drawer - Middle Drawer - Bottom Drawer | - EA - EA - EA | - $32.50 - $32.50 - $32.50 |
| 11.B | - Drawers for front window in-counters, 16-game, 20”: - Top Drawer - Top Middle Drawer - Bottom Middle Drawer - Bottom Drawer | - EA - EA - EA - EA | - $32.50 - $32.50 - $32.50 - $32.50 |
| 11.C | - Drawers for front window in-counters, 12-game, 18”: - Top Drawer - Middle Drawer - Bottom Drawer | - EA - EA - EA | - $32.50 - $32.50 - $32.50 |
| 11.D | - Drawers for front window in-counters, 16-game, 18”: - Top Drawer - Top Middle Drawer - Bottom Middle Drawer - Bottom Drawer | - EA - EA - EA - EA | - $32.50 - $32.50 - $32.50 - $32.50 |
| 11.E | - Drawers for in-counter series, 12-game, 20”: - Top Drawer - Middle Drawer - Bottom Drawer | - EA - EA - EA | - $25.00 - $25.00 - $25.00 |
| 11.F | - Drawers for in-counters, 16-game, 20”: - Top Drawer - Top Middle Drawer - Bottom Middle Drawer - Bottom Drawer | - EA - EA - EA - EA | - $25.00 - $25.00 - $25.00 - $25.00 |
| 11.G | - Drawers for in-counters, 12-game, 18”: - Top Drawer - Middle Drawer - Bottom Drawer | - EA - EA - EA | - $25.00 - $25.00 - $25.00 |
| 11.H | - Drawers for in-counters, 16-game, 18”: - Top Drawer - Top Middle Drawer - Bottom Middle Drawer - Bottom Drawer | - EA - EA - EA - EA | - $25.00 - $25.00 - $25.00 - $25.00 |
| 12. | Replacement Parts for On-Counter Series (items 3, 4 & 5) - Outer Shell, 2 5/8” high x 15 5/8” wide x 10 5/8” deep - #403-LBGOR Drawer, 1 ¾” high x 15” wide x 12 ½” deep | - EA - EA | - $25.00 - $25.00 |
| 13. | Dispenser, single stackable interlocking per attached specifications. Take-A-Ticket Model 113 | - Each | - $11.50 |
| 14. | Optional base for item 13 per attached specifications. Take-A-Ticket TAT CLB (Counter Base Lock) | - Each | - $9.50 |
| 15. | Optional pedestal, eight inches high, for item 13 per attached specifications. Take-A-Ticket 8” Pedestal | - Each | - $7.00 |
| 16. | Dispenser, 3 7/8” h x 27 ½” w x 15 1/8” d, TAT SSOC-6T | - Each | - $187.25 |
| 17. | Dispenser, 3 7/8” h x 36” w x 15 1/8” d, TAT SSOC-8T | - Each | - $208.65 |
| 18. | Dispenser, 7” h x 27 ½” w x 17 ½” d, AT SSOC-12T | - Each | - $214.00 |
| 19. | Dispenser, 10” h x 27 ½” w x 18 5/8” d, TAT SSOC-18T | - Each | - $315.65 |
| 20. | Dispenser, 13” h x 27 ½” x 18 5/8” d, TAT SSOC-24T | - Each | - $347.75 |
| 21. | Dispenser, 13 ¾” high x 19 ¾” wide x 23” deep, double glass metal frame, glass is safety tempered, top glass is removable for easy cleaning or replacement, two (2) removable drawers that individually lock, the U2W is one inch deeper (clerk to customer) than earlier in-counter units, the drawers have a variety of features to allow ticket packs of different length and thickness to be used, a spacer is used to hold the tickets up for best viewing, the dispenser brings all tickets to the surface for customer viewing, dispensers comes fully assembled for quick and easy installation. TAT 412U-2W | - Each | - $186.98 |
| 22. | Dispenser, 13 ¾” high x 19 ¾” wide x 23” deep, double glass metal frame, glass is safety tempered, top glass is removable for easy cleaning or replacement, two (2) removable drawers that individually lock, the U2W is one inch deeper (clerk to customers) than earlier in-counter units, the drawers have a variety of features to allow ticket packs of different length and thickness to be used, a spacer is used to hold the tickets up for best viewing, the dispenser brings all tickets to the surface for customer viewing, dispenser comes fully assembled for quick and easy installation TAT 416U-2W | - Each | - $213.47 |
| 23. | Tray, left, 12 1/8 high x 9 1/8” wide x 22 1/8” deep, TAT 416U Left Tray | - Each | - $60.00 |
| 24. | Tray, right, 12 1/8” high x 9 1/8” wide x 22 1/8” deep, TAT 416U Right Tray | - Each | - $60.00 |
| 25. | Inserts, 11 ¾” high x 8 7/8” wide x 22 ¼” deep, TAT 412U2W inserts | - Each | - $56.00 |
| 26. | Inserts, 11 ¾” high x 8 7/8” wide x 22 1/4” deep, TAT 416U2W inserts | - Each | - $72.50 |
| 27. | Dispenser, 6 ½” high x 9 7/8” wide x 18” deep, multiple on-counter game dispenser, side by side design makes for a great secure, compact, eye-catching dispenser, units are constructed of clear, break resistant PETG (Vivak) plastic and include an outer shell and locking removable drawer for easy loading and dispensing. TAT 804-D | - Each | - $48.15 |
| 28. | Dispenser, 9 ½” high x 9 7/8” wide x 20” deep, multiple on-counter game dispensers, side-by-side design makes for a great secure, compact, eye catching dispenser, units are constructed of clear, break resistant PETG (Vivak) plastic and include an outer shell and locking removable drawer for easy loading and dispensing. TAT 806-D | - Each | - $69.55 |
| 29. | Dispenser, 12 ½” high x 9 7/8” wide x 21 ½” deep, multiple on-counter game dispensers, side-by-side design makes for a great, secure, compact, eye-catching dispenser, units are constructed of clear, break resistant PETC (Vivak) plastic and include an outer shell and locking removable drawer for easy loading and dispensing. TAT 808-D | - Each | - $83.45 |
| 30. | Dispenser, TAT 140-D | - Each | - $46.00 |