About this document:

This document is a draft of a proposal for a new R&D project. The document will be presented during a special discussion session of the “1st Workshop on Radiation hard semiconductor devices for very high luminosity colliders” on Friday, the 30 November 2001 to colleagues that are interested in joining the R&D collaboration. Comments and suggestions as well as active participation in the further refinement of this document are not only highly welcome but also necessary, since not all paragraphs have been reviewed by the corresponding experts.

Cinzia Da Via, Christian Joram, Michael Moll (Editors)

---------------------------------------------------------------------------------------------------------------------

DEVELOPMENT OF RADIATION HARD SEMICONDUCTOR DEVICES FOR VERY HIGH LUMINOSITY COLLIDERS

- SMART* -

Structures and Materials

for Advanced Radiation Hard Trackers

Proposal

Authors, Institutes

Abstract

The requirements at the Large Hadron Collider (LHC) at CERN have pushed the present day silicon tracking detectors to the very edge of the current technology. Future very high luminosity colliders or a possible upgrade scenario of the LHC to a luminosity of 1035cm-2s-1 will require semiconductor detectors with substantially improved properties. Considering the expected fluences of fast hadrons above 1016 cm-2 and a possible reduced bunch-crossing interval of 10 ns, the detector must be ultra radiation hard, provide a fast and efficient charge collection and be as thin as possible.

We propose a research and development program to provide a detector technology, which is able to operate safely and efficiently in such an environment. Within this project we will optimize existing methods and evaluate new ways to engineer the silicon bulk material, the detector structure and the detector operational conditions. Furthermore, possibilities to use semiconductor materials other than silicon will be explored.

Table of contents

1	Summary	4

2	Introduction	5

3	Radiation Damage in Silicon Detectors	5

3.1	Radiation induced defects	5

3.2	Radiation damage in detectors	6

3.3	Present limits of operation	7

4	Objectives and Strategy	7

5	Defect Engineering	8

5.1	Oxygen enriched silicon	8

5.2	Oxygen dimer in silicon	10

6	New Detector Structures	11

6.1	3D detectors	12

6.2	Thin detectors	12

7	Operational Conditions	13

8	New Sensor Materials	13

8.1	Silicon Carbide	13

8.2	Amorphous Silicon	14

8.3	GaN-based materials	14

9	Basic Studies, Modeling and Simulations	14

9.1	Basic Studies	15

9.2	Modeling and Simulation	16

10	Work Plan, Time Scale and Milestones	16

# Summary

We propose an R&D program with the following main objective:

To develop radiation hard semiconductor detectors that can operate beyond the limits of present devices. These devices should withstand fast hadron fluences of the order of 1016 cm-2, as expected for example for a recently discussed luminosity upgrade of the LHC to 1035cm-2s-1.

Three strategies have been identified as fundamental:

- Material engineering

- Device engineering

- Detector operational conditions

While we expect each of the strategies to lead to a substantial improvement of the detector radiation hardness, the ultimate limit might be reached by an appropriate combination of two or more of the above mentioned strategies.

Moreover,

- Basic studies, defect analysis and device simulation

are vital to the success of the research program and are therefore treated as key tasks.

The proposed program covers the following research fields:

- Radiation damage basic studies

- Oxygenated silicon

- Oxygen dimered silicon

- 3D and thin devices

- Forward bias operation

- Other materials, like SiC, GaN and a-Si:H

- Defect modeling and device simulation

To evaluate the detector performance under realistic operational conditions, a substantial part of the tests will be performed on segmented devices.

Other important topics already covered are: cryogenic operation of silicon detectors (RD39), diamond detectors (RD42) and radiation tolerant electronics (RD49).

We plan to perform this research program keeping an active information exchange with the LHC experiments and the other R&D efforts on detector and electronics radiation hardness issues.

# Introduction

Future experiments at a high luminosity hadron collider will be confronted with a very harsh radiation environment and further increased requirements concerning speed and spatial resolution of the tracking detectors.

In the last decade advances in the field of sensor design and improved base material have pushed the radiation hardness of the current silicon detector technology to impressive performance [-, , ]. It should allow operation of the tracking systems of the Large Hadron Collider (LHC) experiments at nominal luminosity (1034cm-2s-1) for about 10 years. However, the predicted fluences of fast hadrons, ranging from 31015 cm-2 at R = 4 cm to 31013 cm-2 at R = 75 cm for an integrated luminosity of 500 fb-1, will lead to substantial radiation damage of the sensors and degradation of their performance. For the innermost silicon pixel layers a replacement of the detectors may become necessary before 500 fb-1 has been reached.

One option that has recently been discussed to extend the physics reach of the LHC, is a luminosity upgrade to 1035cm-2s-1, envisaged after the year 2010 []. An increase of the number of proton bunches, leading to a bunch crossing interval of the order of 10 – 15 ns is assumed to be one of the required changes. While present detector technology, applied at larger radius (e.g. R > 20 cm), may be a viable option, the full physics potential can only be exploited if the current b-tagging performance is maintained. This requires, however, to instrument also the inner most layers down to R  4 cm where one would face fast hadron fluences above 1016cm-2 (2500 fb-1).

The radiation hardness of the current silicon detector technology is unable to cope with such an environment. The necessity to separate individual interactions at a collision rate of the order of 100 MHz may also exceed the capability of available technology.

Several promising strategies and methods are under investigation to increase the radiation tolerance of semiconductor devices, both for particle sensors and electronics. To have a reliable sensor technology available for an LHC upgrade or a future high luminosity hadron collider a focused and coordinated research and development effort is mandatory. Moreover, any increase of the radiation hardness and improvement in the understanding of the radiation damage mechanisms achieved before the luminosity upgrade will be highly beneficial for the interpretation of the LHC detector parameters and a possible replacement of pixel layers.

While in a first phase of the R&D program emphasis may be put on the optimization of known radiation hardening mechanisms and exploration of new structures and materials, in the second phase, system and integration aspects must play a major role.

# Radiation Damage in Silicon Detectors

This paragraph gives a very brief overview about the present understanding of radiation damage in silicon detectors on the microscopic and macroscopic scale and outlines the resulting limits of detector operation in very intense radiation fields.

## Radiation induced defects

The interaction of traversing particles with the silicon lattice leads to the displacement of lattice atoms, which are called Primary Knock on Atoms (PKA’s). The spectrum of the kinetic energy transferred to the PKA’s depends strongly on the type and energy of the impinging particle []. A PKA loses its kinetic energy by further displacements of lattice atoms and ionization. While displaced silicon atoms with energies higher than about 35 keV can produce dense agglomerations of displacements (clusters or disordered regions), atoms with kinetic energies below this value can displace only a few further lattice atoms. A displaced lattice atom is called an Interstitial (I) and the remaining gap in the lattice a Vacancy (V). Both, vacancies and interstitials are mobile in the silicon lattice and perform numerous reactions with impurities present in the lattice or other radiation induced defects.

## Radiation damage in detectors

Three main macroscopic effects are seen in high-resistivity silicon detectors following energetic hadron irradiation (see e.g. [, ]). These are:

- Change of the doping concentration with severe consequences for the operating voltage needed for total depletion (see Figure 1).

- Fluence proportional increase in the leakage current, caused by creation of generation/recombination centers (see Figure 2).

- Deterioration of charge collection efficiency due to charge carrier trapping leading to a reduction of the effective drift length both for electrons and holes.

| Figure 1.: Example for the change of the depletion voltage with increasing particle fluence []. | Figure 2.: Increase of leakage current with fluence for different types of materials measured after an annealing of 80 min at 60 C []. |
| --- | --- |

The first effect is the most severe for present detectors at LHC. The depletion voltage Vdep necessary to fully extend the electric field throughout the depth of an asymmetric junction diode (i.e. silicon detector) is related with the effective doping concentration Neff of the bulk by

Vdep≈q02εε0Neffd2                                                                           (Eq. 1)

with q0 being the elementary charge and 0 the permittivity in vacuum. For a non irradiated n-type detector Neff , and therefore also Vdep, is determined by the concentration of shallow donors (usually phosphorus) and the sign of Neff is positive. Exposing the device to energetic hadron irradiation changes the depletion voltage as shown in Figure 1. With increasing fluence, Vdep first decreases (so-called donor removal) until the sign of the effective space charge changes from positive to negative (type inversion). Then, with further increasing fluence, the depletion voltage increases and eventually will exceed the operation voltage of the device. The detector has to work below full depletion. Consequently not all charge is collected and the signal produced by a minimum ionizing particle (mip) is smaller. After irradiation, Vdep shows a complex annealing behavior. Here, the most severe change is the so-called reverse annealing which leads to a drastic increase of Vdep in the long term which can only be avoided by constantly keeping the detector below about 0 C. This leads to strong restrictions during the maintenance of HEP detectors, which has to be performed either at reduced temperature or kept as short in time as possible. However, even when the reverse annealing can be avoided by keeping the detector cold, it is so far impossible to avoid the temperature and time independent part of the damage.

The second and third effects given in the list above have direct consequences for the signal-to-noise (S/N) ratio, increase in power dissipation and deterioration in the spatial resolution for the detection of mips. However, operating the detector in moderately low temperatures of about –10 C can largely reduce the leakage current and guarantees a sufficiently low noise and power dissipation. For the LHC experiments the trapping effects are also tolerable, however, for future very high luminosity colliders it might become the limiting factor for operation, as described in the next section.

## Present limits of operation

The recent research on radiation hard silicon detectors was focused on the understanding of the detector behavior after exposure to neutron or charged hadrons fluences of up to 1015 cm-2. At that fluence (1015 cm-2) several changes of the detector macroscopic parameters are observed to take place [Error: Reference source not found]:

- Reduction of the effective drift length for electrons ~150 m and for holes ~50 m [].

- Effective conduction type inversion of the material due to the presence of vacancy related radiation induced deep acceptors leading to a depletion starting from the n-contact.

- Fluence proportional increase of leakage current per unit volume due to the presence of radiation induced generation/recombination centers (I/V   mA/cm3 at 20 C).

- Negative space charge increases to 1012 cm-3, requiring ~1000 Vdep for 300 m full depletion.

- Presence of reverse annealing, or increase of the negative space charge after long term annealing at room temperature.

- Deterioration of the charge collection efficiency due to a combination of trapping and incomplete depletion, both for pixels and simple non-segmented pad structures.

# Objectives and Strategy

We propose an R&D program with the following main objective:

To develop radiation hard semiconductor detectors that can operate beyond the limits of present devices. These devices should withstand fast hadron fluences of the order of 1016 cm-2, as expected for example for a recently discussed luminosity upgrade of the LHC to 1035cm-2s-1.

Three strategies have been identified as fundamental:

- Material engineering

- Device engineering

- Detector operational conditions

While we expect each of the strategies to lead to a substantial improvement of the detector radiation hardness, the ultimate limit might be reached by an appropriate combination of two or more of the above mentioned strategies.

These are based on the successful achievements of the past and present CERN R&D projects [-, , , , , ] and recent discoveries in radiation hard semiconductor devices.

Moreover,

- Basic studies, defect analysis and device simulation

are vital to the success of the research program and are therefore treated as key tasks.

To evaluate the detector performance under realistic operational conditions, a substantial part of the tests will be carried out on segmented devices.

We plan to perform this research program keeping an active information exchange with the LHC experiments and the other R&D efforts on detector and electronics radiation hardness issues.

# Defect Engineering

The term “defect engineering” stands for the deliberate incorporation of impurities or defects into the silicon bulk material before, during or after the processing of the detector. The aim is to suppress the formation of microscopic defects with a detrimental effect on the macroscopic detector parameters during or after irradiation. In this sense defect engineering is coping with the radiation damage problem at its root.

## Oxygen enriched silicon

The CERN RD48 (ROSE) Collaboration introduced oxygen-enriched silicon as DOFZ (Diffusion Oxygenated Float Zone Silicon) to the HEP community [Error: Reference source not found]. The DOFZ technique was first employed by Zheng Li et al. on high resistivity FZ silicon [] and consists of diffusion of oxygen (e.g. for 24 hours at 1150C) into the silicon bulk from an oxide layer grown via a standard oxidation step. Figure 3 shows examples of oxygen depth profiles in different DOFZ samples as measured with the Secondary Ion Emission Spectroscopy (SIMS) method [Error: Reference source not found].

| Figure 3.: Oxygen depth profile as measured by SIMS after different oxygen diffusion processes [Error: Reference source not found]. | Figure 4.: Influence of Carbon and Oxygen on the depletion voltage Vdep [Error: Reference source not found]. |
| --- | --- |

RD48 demonstrated in 1998 that the oxygenated material is highly superior in radiation tolerance with respect to charged hadrons []. The main properties of oxygen-enriched silicon are described in [Error: Reference source not found, Error: Reference source not found] and are summarized in the following:

The increase of negative space charge (i.e. the increase of depletion voltage after type inversion) is reduced by about a factor of 3 for high energy charged hadrons. This is shown in Figure 4 for 23 GeV protons and was also observed for 192 MeV pions. Furthermore, for low energy protons in the energy range of 16-27 MeV a reduction of about 2 was observed []. However, after neutron irradiation no such improvement for this damage component was observed. The reverse annealing amplitude (i.e. the increase of the depletion voltage during the long term annealing) saturates at high fluences for the oxygenated silicon, amounting to a reduction factor of up to 3 for DOFZ diodes. The time constant for this process is at least a factor of 2 larger. Thus both improvements provide a substantial safety margin for the effects to be expected during the warm up maintenance periods. Furthermore, the leakage current is not influenced by the oxygen content [] and the DOFZ process is not influencing the surface and interface properties [].

The implementation of the DOFZ technique in the detector processes of manufacturing companies was initiated by RD48 and led meanwhile to considerable experience of many detector producers. The ATLAS-Pixel collaboration is using DOFZ silicon and the CMS-Pixel collaboration will most likely use this technology. However, there are still many open questions regarding the optimization of the technology and the understanding of the oxygen effect:

- Why does oxygen improve the radiation tolerance?

There are many ideas about the microscopic mechanism, which is underlying the oxygen-effect. However, so far the responsible microscopic defects have not been clearly identified. (basic studies, tasks to be detailed later).

- Quantitative correlation between oxygen content and radiation hardness?

Although the beneficial effect of O-enrichment on the radiation tolerance has been conclusively established in many experiments with detectors originating from many different producers no clear quantitative correlation between the oxygen content and the improvement of the radiation hardness could be established. It seems that there is an impact of the individual processes of the different manufacturers, which has so far not been understood. For the use of oxygenated silicon in future and present applications the clarification of this questions is regarded as very important.

- Reliable characterization of Oxygen and Carbon profiles and simulation of O-diffusion

For the measurement of Oxygen and Carbon profiles in high-resistivity-detector-grade silicon the SIMS (Secondary Ion Emission Spectroscopy) method has to be operated close to its detection limit. An absolute calibration is therefore absolutely necessary. Simulations of the oxygen diffusion profiles are necessary to understand the shape of the oxygen profile measured by SIMS.

- Optimization of DOFZ process

So far the DOFZ process has been studied in a range between 16h/1150C and 8d/1200C. SIMS measurements have shown that for the low in-diffusion process one gets a quite inhomogeneous O-distribution while in the latter case the depth profile is almost constant. The optimal process with respect to radiation hardness and cost effectiveness has to be found.

- More detailed characterization of oxygenated detectors

The oxygenation process significantly suppresses the reverse annealing. This needs to be understood, as more improvement may be possible. (basic studies, tasks to be detailed later).

- High resistivity Czochralski Silicon (CZ)

New developments in the silicon manufacturing technology make high resistivity CZ possible. This material might be cheaper than DOFZ and exhibit the same or even better radiation tolerance.

## Oxygen dimer in silicon

Recent work [] has shown that it is possible to convert oxygen interstitial Oi to oxygen interstitial dimer O2i in silicon. There are two main reasons why this is interesting for further developments of radiation tolerant detectors.

Firstly, O2i may be more effective than Oi at improving the radiation tolerance. For instance, V2O2 is electrically neutral unlike V2O, which is thought to be an acceptor close to mid-gap. Secondly, O2i is thought to diffuse more rapidly through silicon than Oi. Migration energies of Oi and O2i are 2.54 eV and 1.8 eV respectively. Thus a possible way to oxygenate silicon wafers in a short time is to introduce Oi into the surface of the wafer by a short high temperature diffusion, convert this Oi to O2i, and then thermally diffuse the O2i into the bulk of the wafer at a much reduced temperature. This would result in a shorter diffusion time and lower furnace temperature when preparing the oxygenated silicon material. Secondly, VO can be both an electron and hole trap, depending on its charge state, while VO2 is electrically neutral. In particular, VO is thought to be the main charge trap in cryogenic temperature forward bias operation, limiting the maximum charge collection efficiency at high fluences for this mode of operation.

Figure 5. DLTS spectra of high (309) and low (366) oxygen content silicon diodes. D indicates that the sample underwent dimerization process. In both 366D and 309D sample the VO (Vacancy-Oxygen) E(90) peak has disappeared [].

Oxygen dimer silicon diodes have been produced with 1015/cm3 carbon, low (1015/cm3) and high (1017/cm3) oxygen, n-type, 4 k-cm resistivity silicon diodes. For the dimerisation they were irradiated at 350C using a Cobalt-60 gamma source. Previously, a similar process has been tried using 2 MeV electrons [Error: Reference source not found]. The gamma source has the advantage of uniformly producing interstitial-vacancy pairs throughout the silicon. Moreover, divacancies V2 are produced a factor 50 less than single vacancies V [].

The quasi-chemical reactions that are thought to lead to Oxygen dimer formation are []:

V + O => VO,

VO + O => VO2,

I + VO2 => O2.

The success of the process was proven by the absence in both the low and high oxygen samples of the DLTS (Deep Level Transient Spectroscopy) VO (Vacancy Oxygen) peak (E(90)) after proton irradiation, as shown in Figure 5 [Error: Reference source not found]. The E(170) peak , which has been correlated with VOH, is present after the dimerisation process with a concentration of 51011cm-3. This concentration does not change after proton irradiation and it is too small to have any influence on the final concentration of radiation-induced defects.

Reverse annealed samples measured at –50 oC show a decrease of reverse annealed charge build-up, correlated with the intensity of the DLTS peak E(226) associated with the di-vacancy V2 cluster, for the low oxygen dimered sample [].

The potential of this material for radiation hardness applications has been discovered very recently. Systematic study is needed to understand the role of oxygen dimer in defect formation and device performance. Detailed measurements are required to understand:

- Optimization of dose rate and exposure time during material processing

- Defect formation (Infra Red Absorption, DLTS, EPR, positron lifetime)

- Space Charge and Reverse Annealing

- Charge Collection Efficiency

- Minority carrier lifetime

- Low temperature and forward bias behavior

# New Detector Structures

The signal formation in semiconductor devices is described by Ramo’s Theorem [], which relates the electrode’s induced current signal to the contribution of charge motion. The weighting potential Vw, is the solution of the Laplace’s Equations Vw=0. For a segmented detector the shape of the weighting potential depends on the ratio between the contact area and the depletion depth and results in an exponential increase of its value close to the segmented contact.

After irradiation, the movement of the carriers is limited by the effect of radiation-induced traps. The effective drift length Leff is therefore directly proportional to the effective trapping time t and the carrier drift velocity Vdrift, Leff = t  Vdrift, and has been measured and simulated to be ~150 m for electrons and ~50 m [Error: Reference source not found, Error: Reference source not found] for holes after a fluence of 11015 particles/cm2 and an electric field of 1V/m. The detectable charge, considering the charge loss due to trapping therefore can be approximately expressed as:

Signal ~ qVW(x)-VW(0) e-thτh+VW(d)-VW(x)e-teτe   Eq. 2

where VW(x) represents the weighting potential at the depth x within the depleted region of thickness d, th and te the drift times for holes and electrons and e and h the electron and holes trapping times, respectively. It follows that a segmented detector after 11015 particles/cm2 should:

- Collect electrons

- Optimize electrode configuration and detector thickness

## 3D detectors

3D radiation hard properties are geometric in nature and its improvement factors will generally multiply those coming from material improvements. The main characteristic of the 3D detector concept is shown in Figure 6 and consists in having the electrodes (p,n) implanted throughout the bulk in form of narrow columns instead of being deposited parallel to the detector surface. While in a conventional silicon sensor the depletion and charge collection across the full wafer thickness (usually 300 m) requires very high voltages and becomes incomplete after high radiation levels, the main advantage of this approach is the short distance between collecting electrodes. This allows at the same time very fast collection times, very low full depletion bias voltage (~10 V), low noise and the full 25 000 e/h provided by the 300 m detector active thickness.

| Figure 6.: Schematic, three-dimensional view of part of a sensor with 3D electrodes penetrating through the substrate. The front border of the figure is drawn through the center of three electrodes. | Figure 7. 3D detectors signal after irradiation with a fluence of 1·1015 55 MeV protons/cm2. The hardness factor corresponds to 1.7·1015 1 MeV equivalent neutron/cm2. []. |
| --- | --- |

After irradiation with protons up to 1·1015 55 MeV protons/cm2, see [Error: Reference source not found], a sensor with 100m n-n separation is fully depleted  at 105 V (see Figure 7) and has a plateau up to 150 V. Leakage currents for unirradiated sensors range from about ¼ to 1¼  nA/mm3 of depleted silicon. The increase of leakage current with irradiation is similar to those of similar planar detectors.

## Thin detectors

Similar considerations as for the 3D detectors can be applied to thin detectors. The basic advantage of thin devices relates to the optimized use of the effective drift length of the carriers while having a low full depletion voltage. Moreover, this leads to a significant reduction of the material budget, which would improve the overall particle momentum resolution.

The planar 300 µm silicon detectors have been so far a reasonable compromise between signal/noise, silicon availability and ease of mechanical handling. Thin, low mass semiconductor trackers would have many advantages in future experiments, as in some respects has been shown already by the use of CCDs at SLAC []: better tracking precision and momentum resolution, more precise timing (not compatible with CCD/monolithic devices), lower operating voltage, lower leakage currents and improved radiation hardness. As discussed above, even after a high dose, both the electrons and the holes still can be collected over 50 µm so that it may be feasible to retain a p+n segmented diode structure for a thin detector. However, the m.i.p. signal from such a thin, 50 µm, silicon sensor layer is only ~3500 e-h pairs, with a relatively broad Landau distribution towards higher values. Only with the small pixel concept readout electronics can one have sufficiently low noise at ns timing.

The problems related to this approach are purely technical since both, processing on thin devices is difficult, as well as thinning after processing. Industry has expressed high interest in thin silicon devices mainly for credit cards and smart cards. Work should be done, closely with industry, to process low cost, reliable samples to be tested with or without readout electronics. A precise cost estimate is very difficult at this stage, without R&D.

# Operational Conditions

The Charge Collection Efficiency (CCE) recovery of heavily irradiated planar standard silicon detector operated at a temperature around 130 K, known as the “Lazarus Effect”, is the subject of study of the RD39 collaboration [Error: Reference source not found]. The same collaboration is also studying effective ways to overcome space charge polarization effects at low temperatures, namely a reduction of the CCE with time due to charge trapping, by constant charge injection. The latter can be performed by forward bias operation, as previously demonstrated by the Lancaster group [], or by short wavelength illumination [Error: Reference source not found]. Operating the detector at low temperature can also control the space charge. Experimental results and simulations obtained by RD39, have shown that using the exponential dependence with temperature of energy levels occupancy (~exp(–Et/kT)) is an effective way to control the charge state of the radiation induced deep traps.

The operation of highly irradiated detectors under forward bias or by using other techniques to induce free charge into the detector bulk is not only a promising operational condition for low temperatures around 130 K but can also improve the detector performance at higher temperatures [Error: Reference source not found].  Therefore, it is foreseen to perform tests under such conditions on any of the new materials and devices whenever it is promising an improvement of the radiation tolerance. In cases where the temperature reaches the regime of 130 K we will strive for a close collaboration with RD39 in order to profit from their expertise and bundle resources.

# New Sensor Materials

The radiation hardness properties of diamond detectors for the LHC have been the subject of study of the RD42 collaboration [13]. Other materials, however, have been recently recognized as potentially radiation hard. Some of them are listed hereafter with their basic radiation hard properties. They will be only included into the final proposal in case institutions with expertise will express their interest in exploring their properties as radiation hard particle detectors.

## Silicon Carbide

Semi-insulating 4H-SiC has the intrinsic possibility of being a radiation hard particle detector. 4H-SiC has a large band gap (3.3 eV), e-h pair generation per 100 um per MIP (5100 e) and a low carrier density, which implies a low leakage current and high initial resistivity, as high as 1011 cm. at room temperature. The present wafer dimension is 30 mm, but the detector-processing yield is still limited due to high as-grown defect concentration present in the material. After irradiation with ~4·1014 cm-2 8MeV protons the measured charge was ~2000 e, with 500V bias voltage. Polarization was also observed with a time constant of ~14 min and a final charge of ~800 e [].

## Amorphous Silicon

Amorphous silicon has been extensively used for solar cells applications, flat panels displays and optical scanners. Its use is possible due to the hydrogenation process (a-Si:H) which allows the passivation of the intrinsic dangling bonds present in the material, due to missing atoms in the Si amorphous structure. The presence of the dangling bonds would prevent the use of such material as radiation detectors since they act as very effective recombination centers for electrons. At present Metal Insulator Semiconductor (MIS) and PIN structures have been fabricated up to thickness of tens of microns using Radio-frequency Plasma Enhanced Chemical Vapor deposition (PECVD). The Charge Collection Efficiency (CCE) for 0.8 MeV alpha particles was measured to be 3% [].

## GaN-based materials

GaN has been extensively studied for its optical properties and successfully employed in the fabrication of blue lasers. At present very pure growth processes like Molecular Beam Epitaxy and Chemical Vapour Deposition are available allowing the production of substrates with a low trap density. The large band gap (from 3.4 to 6.2) of AlGaAs provides a low leakage current and high intrinsic resistivity. The high breakdown voltage (300 V/m) and the possibility of internal gain due to electron avalanches would offer a novel interesting prospect for charge collection efficiency.

# Basic Studies, Modeling and Simulations

The radiation-induced changes of the macroscopic silicon detector properties – leakage current, depletion voltage, charge collection efficiency – are caused by radiation induced electrical active microscopic defects (see also Section 3). Therefore, a comprehensive understanding of the radiation induced detector degradation can only be achieved by studying the microscopic defects, their reaction and annealing kinetics and especially their relation to the macroscopic damage parameters. Furthermore, modeling of defect formation and device simulations are needed to understand the complicated defect formation mechanisms and the operation of irradiated structured devices.

These kinds of studies are of fundamental interest for all semiconductor-based devices (sensors and electronics) operating in an irradiation environment. In order to exploit the common interest of several groups working at CERN a close collaboration with the RD39 [Error: Reference source not found], RD42[13] and RD49 [Error: Reference source not found] collaborations is foreseen. We envisage joint research activities and common sessions in the respective collaboration meetings.

## Basic Studies

### Investigations on microscopic defects

In the last years many measurements on irradiation induced microscopic defects in high resistivity FZ silicon have been performed. However, the exact nature of the defects, which are responsible for the macroscopic radiation damage, are still not fully known. We propose therefore the following work:

- Defect characterization

Besides the techniques of Deep Level Transient Spectroscopy (DLTS), Thermally Stimulated Current (TSC) and Transient Charge Technique (TCT), which have been extensively used in the past years for defect characterization in detector silicon, also techniques like Electron Paramagnetic Resonance (EPR), Fourier Transform Infrared absorption (FTIR) and Photo Luminescence (PL) need to be used.

- Irradiations at different temperatures – online measurements

Both vacancies and interstitials are migrating very fast at room temperature. It is therefore impossible to directly investigate the formation of most of the defects since they are formed too quickly. Only by performing irradiations in the cold the migration process can be stopped (“frozen”) and a deeper insight into the defect formation process can be taken. Such measurements have either to be performed on the beam line (irradiation facility) or the samples have to be transported cold to the measurement setup.

### Investigations on irradiated detectors

Extensive experiments on the radiation induced changes of detector properties and their dependence of particle fluence, particle type and energy, temperature and annealing time are indispensable. They not only open the door for a profound understanding of the relationship between microscopic defects and detector properties but also are absolutely necessary to predict the radiation damage effects in the tracker experimental environment. The following topics need to be investigated in more detail:

- CCE (Charge Collection Efficiency)

Up to now most systematic investigations on the radiation-induced changes of the effective doping concentration have been based on depletion voltage measurements as extracted from Capacitance-Voltage (CV) measurements. However, systematic charge collection measurements, either performed with a laser (difficult for an absolute calibration) or with mips, have to be performed in systematic investigations to also parameterize the trapping behavior in more detail.

- Comparison between simple test structures and full-size or mini strip detectors.

Such measurements are closely related to investigations on the dependence of CCE and electric field distribution on the device structure.

- Irradiations under bias at operating temperatures (e.g. -10C)

So far, most irradiations have been performed without applied bias and in a room temperature ambient. It has been shown that the irradiation under bias has an influence on the changes of the depletion voltage. Since detectors are operated under bias and at temperatures below ambient temperature, these effects have to be investigated in more detail.

- Establishment of comparable measurement procedures

There exists no agreed common measurement procedure for irradiated detectors. Detector treatments after irradiation (annealing procedure) are differing strongly from community to community and are making inter-comparison very difficult.

- Systematic investigations on the particle and energy dependence (NIEL)

It has clearly been demonstrated by the ROSE collaboration that the so-called “NIEL-Hypothesis” is not valid for all damage parameters. This implies that irradiation tests with a much wider range of particle energies must be performed.

- Combined investigation with state of the art radiation hard electronics

Radiation hard electronics is fundamental for the proper functioning of a silicon tracker. The RD49 collaboration has already developed effective design strategies for the existing LHC experiments [Error: Reference source not found] and is planning new improvements for the high luminosity scenario. Close contacts are foreseen with the RD49 groups and combined tests are planned to evaluate small-scale radiation-hard modules.

## Modeling and Simulation

### Modeling of defect formation

Modeling of defect formation is indispensable for the understanding of the radiation damage process and the development of new defect-engineered materials.         []

### Device Simulation

Recent results achieved with commercial and in house software packages have helped crucially in understanding the present limitation of irradiated silicon devices [citations]. Furthermore, device simulators are a indispensable tool for the development of novel device structures.

# Work Plan, Time Scale and Milestones

3 years.

References:

[?] 	ATLAS – Inner Detector Technical Design Report; CERN/LHCC/97-16,17.

[?]	ATLAS – Pixel Detector Technical Design Report; CERN/LHCC/98-13.

[?]	CMS – Technical Proposal; CERN/LHCC 94-38

[?]	EP-TH Faculty Meeting, CERN, 17.01.2001

[?] 	M.Huhtinen, "Simulation of non-ionising energy loss and defect formation in silicon" ROSE/TN/2001-02; http://cern.ch/rd48/; to be published in NIMA.

[?]	G.Lindström et al. (The RD48 Collaboration); “Radiation Hard Silicon Detectors - Developments by the RD48 (ROSE) Collaboration –“; NIM A 466 (2001) 308-326.

[?] 	G.Lindström et al. (The RD48 Collaboration); “Developments for Radiation Hard Silicon Detectors by Defect Engineering - Results by the CERN RD48 (ROSE) Collaboration –“NIM A 465/1 (2001) 60-69.

[?]	Data taken from: R.Wunstorf, PhD thesis,”Systematische Untersuchungen zur Strahlenresistenz von Silizium-Detektoren für die Verwendung in Hochenergiephysik-Experimenten“, University of Hamburg, October 1992.

[?] 	M.Moll, “Radiation Damage in Silicon Particle Detectors – Microscopic Defects and Macroscopic Properties - ”,PhD-Thesis, DESY-THESIS-1999-040, December 1999.

[?] 	G. Kramberger et al. Effective carrier trapping times in irradiated silicon, Presented at 6-th ROSE Workshop RD48, October 2000, CERN (CERN/LEB 2000-006).

[?]  	The RD2 Collaboration

[?]	The RD20 Collaboration

[?] 	The RD39 Collaboration “Cryogenic tracking detectors”; http://www.cern.ch/RD39

[?]	The RD42 Collaboration http://www.cern.ch/RD42.

[?]	The RD48 (ROSE) Collaboration: http://cern.ch/rd48/.

[?] 	The RD49 Collaboration

[?] 	Z. Li et al.; “Investigation of the Oxygen-Vacancy (A-Center) Defect Complex Profile in Neutron Irradiated High Resistivity Silcon Junction Particle Detectors”; IEEE TNS, Vol 39, No 6, 1730-1738 (1992).

[?] 	A. Ruzin et al., IEEE Trans. on Nuclear Science, vol.46, no.5, 1310 (1999).

[?] 	J.Wyss et al.; “Observation of an energy dependence of the radiation damage on standard and oxygenated silicon diodes by 16, 21, and 27 MeV protons”; NIMA 457 (2001) 595-600.

[?]	M.Moll,  E.Fretwurst, G.Lindström; Investigation on the improved radiation hardness of silicon detectors  with  high oxygen concentration; NIMA 439 (2000) 282-292

[?] 	J.Wüstenfeld, Ph.D.thesis University of Dortmund, Internal Report, UniDo PH-E4 01-06, August 2001, see also ROSE/TN/2000-05 in [Error: Reference source not found].

[?] 	J.L. Lindström, T. Hallberg, J. Hermansson, L.I. Murin, V.P. Markevich, M. Kleverman and B.G. Svensson “Oxygen and Carbon Clustering in Cz-Si during Electron Irradiation at Elevated Temperatures”, Solid State Phenomena 70 (1999) 297-302.

[?] 	C. Da Via and S.J.Watts, “New results for a novel oxygenated silicon material”, paper submitted to the "European Materials Research Society 2001 Spring Meeting - Symposium B on Defect Engineering of Advanced Semiconductor Devices, Strasbourg, France, June 5-8, 2001, to be published in NIMB.

[?] 	M. Moll, H. Feick, E. Fretwurst, G. Lindstrom, T. Shultz, NIM A 338 (1997) 335.

[?] 	J. Coutinho, R. Jones, S. Öberg, and P. R. Briddon; “Oxygen and dioxygen centers in Si and Ge: Density-functional calculations”; Phys. Rev. B 62, 10824 (2001).

[?]	S.J. Watts and C. Da Via’, presented at the Vertex 2001 Conference, Brunnen Switzerland. To be published in NIMA.

[?] 	S. Ramo, Proc IRE 27 (1939) 584.

[?]	Sherwood Parker et al. Performance of 3D architecture, silicon sensors after intense proton irradiation, to be publishes in IEEE Trans. In Nucl. Scie. And references therein.

[?] 	K. Abe, A. Arodzero, C. Baltay, J. E. Brau, M. Breidenbach, P. N. Burrows, A. S. Chou, G. Crawford, C. J. S. Damerell, P. J. Dervan et al.; “Design and performance of the SLD vertex detector: a 307 Mpixel tracking system”, NIM A 400 (1997) 287-343.

[?] 	L. Beattie et al NIM A439 (2000) 293-300.

[?] 	M. Rogalla et al. Nucl Phys. B 78 (1999) 516-520.

[?]	C. Hordequin et al, NIMA 456 (2001) 284-289.

[?] 	B.C. MacEvoy, et al, Defect kinetics in Novel Detector Materials. Presented at the 1st 	ENDEASD Workshop, Santorini, April 21-22 1999. Accepted for publication in Materials 	Science in Semiconductor Processing.

* Preliminary name! This is just a proposal for a name. Further suggestions are very welcome.