Item No 1 - WALK-IN COOLER/FREEZER          	         1 Required

Kolpak Industries, Inc.  Unit to be a Refrigerator +35° F. of size and shape as shown on plan x  8' - 2 1/2" high. Wall and ceiling panels shall be 4" urethane, U.L.Flame Spread 25 insulation. Panels are without wooden structural members. Hinged entrance door(s) to be flush type, hinged as shown on plan. Thru ceiling door electrical assembly as shown on detail E-1-4. Provide one (1) lot removable closure panels with channels, the same finish as exterior panels to ceiling. (F.S.E. Contractor to verify ceiling height).  Closure to be louvered for top mounted condensing units (define) One (1) lot of trim strip, where required to close in wall gaps, of same finish as exterior panels to finished ceiling. (F.S.E. Contractor to verify height).

Options:

- Floorless refrigerator set on 4" vinyl screed on finished concrete slab.  Structural insulated panel floor (3/16 smooth aluminum plate) in freezer compartment.

- Built-in interior ramp at entrance to freezer.

- Embossed aluminum white interior and exposed exterior

- Unexposed exterior finish to be galvalume.

- Hinged doors to be 36" wide x 84" high with third hinge

- Dead bolt lock per Specific Conditions (on exterior doors only)

- Aluminum diamond tread kick plate, 3'-0" high on exterior and interior of door and jamb

- 14" x 14" vision panel and automatic door closer, Kason K-1094.

- Approved Lithonia fluorescent light fixtures and switches, installed and wired by the Electrical Division per detail E-1-4. (See electrical plan for quantity and location of lights and switches)

- Modularm 75B, recessed with thru ceiling door electrical as shown on detail E-1-4.

- Stainless steel corner guards at exposed corners.

- Wainscot on exposed exterior, 1/8" aluminum diamond plate.  Top to align with top of kickplate on door.

- Protect-All finished flooring on insulated panel floor w/ 6" high coved base at interior perimeter by General Contractor.  Color selected by Architect.

- Provide stainless steel backing plates (surface – mounted) on both sides of panel for through-bolt mounting of Item 6, Air Curtain.

- CCI Cool Curtain Clear Vu swinging Door on both compartments.

Refer to Kolpak preliminary shop drawing 65368.

Item No 2 - EVAPORATOR COIL, LOW TEMPERATURE   	   1 Required

Kolpak Industries, Inc. Model No. EL-090,  Unit to be adequately sized to operate Item 1, Freezer Compartment @ -10° F.  Electrical hook-up and interconnecting of system to be by the Electrical Contractor.

- Heat tape for low temp coil drain line furnished and installed by the FSE Contractor.  Final electrical connection to heat tape by Electrical Division.

Item No 3 - CONDENSING UNIT, AIR COOLED    	                1 Required

Kolpak Industries, Inc. Model No. PR204L,  F.S.E.C. to provide the necessary refrigeration lines required to operate and maintain the refrigeration system for Item 1, Freezer compartment.  Unit to be part of the remote refrigeration system for Item 2, Evaporator Coil.

F.S.E.C. to coordinate with the electrical/plumbing contractor to provide a complete and operable refrigeration system.

Provide start-up, five (5) year compressor warranty and one (1) year refrigeration service contract.

Division 16 to wire to safety disconnect switch.

- Provide outdoor ambient package.

- Unit to be located above fixture.

- Loose timeclock (freezer only) installed above walk-in freezer.

Item No 4 - CONDENSING UNIT, AIR COOLED        	                  1 Required

Kolpak Industries, Inc. Model No. PR150M,  F.S.E.C. to provide the necessary refrigeration lines required to operate and maintain the refrigeration system for Item 1, Refrigerator compartment.  Unit to be part of the remote refrigeration system for Item 5, Evaporator Coil.

F.S.E.C. to coordinate with the electrical/plumbing contractor to provide a complete and operable refrigeration system.

Provide start-up, five (5) year compressor warranty and one (1) year refrigeration service contract.

Division 16 to wire to safety disconnect switch.

- Provide outdoor ambient package.

- Unit to be located above fixture.

Item No 5 - EVAPORATOR COIL, MEDIUM TEMPERATURE   	1 Required

Kolpak Industries, Inc. Model No. AM-104,  Unit to be adequately sized to operate Item 1, Refrigerator Compartment @ +35° F.  Electrical hook-up and interconnecting of system to be by the Electrical Contractor.

Item No 6 - AIR CURTAIN                              	                            1 Required

Mars Air Doors Model No. WA36.

- Micro Switch, Plunger Style, for instant on/off control, use on any single swing hinged, sliding, track, lift, roll-up type doors

- Motor Control Panel for one motor, for any voltage and HP, supplied with NEMA 1 Cabinet with HOA selector switch on the cover and are remote mounted

- Housing Color: Gray, price per ft.

Item No 7 - AIR CURTAIN                                	                          1 Required

Mars Air Doors Model No. CH72-2

- Micro Switch, Roller Style, for instant on/off control, use on any double swing hinged, sliding, track, lift, roll-up type doors. Provide two micro switches for double door.

- Motor Control Panel for two motors, for any voltage and HP, supplied with NEMA 1 Cabinet with HOA selector switch on the cover and are remote mounted

- Housing Color: Gray, price per ft.

Item No 8 - RACK, UTILITY - UNIVERSAL ANGLE        	        LOT Required

Cres Cor Model No. 207-UA-13A,  Rack, Utility - Universal Angle.

- The racks shown are "By Owner" and meant to represent staging locations only.

Item No 9 - FLOOR TROUGH                                      	                  1 Required

IMC/Teddy Food Service Corp. Model No. SFT,  Unit of size and shape as shown on plan x 2" deep. This item to be set and installed by General Contractor. Quarry tile floor finish to be etched, if required, prior to setting food service equipment in place. Provide grating sections of equal size not to exceed 20" long. Provide shop drawings of size and scale outlined in specific conditions.

- Grating to be anti-slip, stainless steel.

- Shallow floor trough, 2" deep.

Item No 10 - CLEANING HOSE ASSEMBLY                    	             1 Required

IMC/Teddy Food Service Corp. Model No. M-5700,  Cleaning Hose Assembly.

- Provide 50' long red hose w/ stainless fittings.

- Nozzle w/ red renewable rubber cover.

- Globe valves.

Item No 11 - BULK CO2 SUPPLY & FILL STATION           	           1 Required

This item is not in contract. The item is shown on the drawing for informational purposes and generally represents the size/ capacity of unit to be provided by Product Supplier/ Vendor. No utility connections required.  All CO2 lines from surface mounted remote fill port to stationary tank are provided and installed by the Product  Supplier/ Vendor.

- Provide unit with Surface Mounted Fill Station

Item No 12 – COMPACTOR

This item is not in contract. The specification is for reference only. The item is shown  on the drawing for informational purposes and generally represents the size/ capacity of unit to be provided by Product Supplier/ Vendor. Any utility connections indicated on drawings are representative and intended to assist the Engineer in planning base building utility systems. Utility requirements for equipment provided by a product supplier will be confirmed by the FSE Contractor and represented on the dimensioned rough-in plans for use by the construction trades.

Item No. 13 - SPARE NUMBER-

Item No 14 - BALER                                                                          1 Required

This item is not in contract. The specification is for reference only. The item is shown  on the drawing for informational purposes and generally represents the size/ capacity of unit to be provided by Product Supplier/ Vendor. Any utility connections indicated on drawings are representative and intended to assist the Engineer in planning base building utility systems. Utility requirements for equipment provided by a product supplier will be confirmed by the FSE Contractor and represented on the dimensioned rough-in plans for use by the construction trades.

Item No 15 – SPARE NUMBER

Item No 50 - WALK-IN COOLER/FREEZER                                    1 Required

Kolpak Industries, Inc.  Unit to be a multi-compartment Refrigerator +35° F. /Freezer -10° F. of size and shape as shown on plan x  8'-2 1/2" high. Wall and ceiling panels shall be 4" urethane, U.L.Flame Spread 25 insulation. Panels are without wooden structural members. Hinged entrance door(s) to be flush type, hinged as shown on plan. Thru ceiling door electrical assembly as shown on detail E-1-4. Provide one (1) lot removable closure panels with channels, the same finish as exterior panels to ceiling. (F.S.E. Contractor to verify ceiling height). One (1) lot of trim strip, where required to close in wall gaps, of same finish as exterior panels to finished ceiling. (F.S.E. Contractor to verify height).

Options:

- Floorless with flat bottom wall panels set on slab. Slab urethane insulation, vapor barrier and quarry tile w/ minimum 2 1/2" setting bed by General Contractor.

- Exterior ramp to be quarry tile by General Contractor.  Ramp sufficient to achieve 2 1/2" mud and tile at door threshold.

- Embossed aluminum white interior in the freezer and exposed exterior of all compartments.

- Note: stainless steel finish on the interior of the refrigerated compartments.

- Unexposed exterior finish to be galvalume.

- Hinged doors to be 36" wide x 84" high with third hinge.

- Dead bolt lock per Specific Conditions (on exterior doors only).

- Anti -condensate heaters in full perimeter of door and frame (freezer only).

- Aluminum diamond tread kick plate, 3'-0" high on exterior and interior of door and jamb.

- 14" x 14" vision panel and automatic door closer, Kason K-1094.

- Factory cut door for tile floor conditions.

- Approved Lithonia fluorescent light fixtures and switches, installed and wired by the Electrical Division per detail E-1-4. (See electrical plan for quantity and location of lights and switches)

- Modularm 75B, recessed with thru ceiling door electrical as shown on detail E-1-4.

- Stainless steel corner guards at exposed corners.

- Wainscot on exposed exterior, 1/8" aluminum diamond plate.  Top to align with top of kickplate on door. FSE Contractor to coordinate w/ General Contractor on height of base tile as required.

- CCI Cool Curtain Clear Vu swinging Door.

- Suspended ceiling in largest compartment.

Refer to Kolpak preliminary shop drawing 65370.

Item No 51 - EVAPORATOR COIL, MEDIUM TEMPERATURE             2 Required

Kolpak Industries, Inc. Model No. AM-130,  Unit to be adequately sized to operate Item 50, Refrigerator Compartment (approximately 17' x 23') @ +35° F.  Electrical hook-up and interconnecting of system to be by the Electrical Contractor.

Item No 52 - CONDENSING UNIT, AIR COOLED REMOTE                  1 Required

Kolpak Industries, Inc. Model No. PR304M,  F.S.E.C. to provide the necessary refrigeration lines required to operate and maintain the refrigeration system for Item 50, refrigerated compartment.  Unit to be part of the remote refrigeration system for Item 51, Evaporator Coil.

F.S.E.C. to coordinate with the electrical/ plumbing contractor to provide a complete and operable refrigeration system.

Provide start-up, five (5) year compressor warranty and one (1) year refrigeration service contract.

Division 16 to wire to safety disconnect switch.

- Unit to be located on building roof.  See Architectural plan for location.

- Assume a 50' refrigeration line run.

Item No 53 - EVAPORATOR COIL, LOW TEMPERATURE                     2 Required

Kolpak Industries, Inc. Model No. EL-090,  Unit to be adequately sized to operate Item 50, Freezer Compartment @ -10° F.  Electrical hook-up and interconnecting of system to be by the Electrical Contractor.

- Heat tape for low temp coil drain line furnished and installed by the FSE Contractor.  Final electrical connection to heat tape by Electrical Division.

Item No 54 - CONDENSING UNIT, AIR COOLED REMOTE                   1 Required

Kolpak Industries, Inc. Model No. PR344L,  F.S.E.C. to provide the necessary refrigeration lines required to operate and maintain the refrigeration system for Item 50, refrigerated compartment.  Unit to be part of the remote refrigeration system for Item 53, Evaporator Coil.

F.S.E.C. to coordinate with the electrical/ plumbing contractor to provide a complete and operable refrigeration system.

Provide start-up, five (5) year compressor warranty and one (1) year refrigeration service contract.

Division 16 to wire to safety disconnect switch.

- Unit to be located on building roof.  See Architectural plan for location.

- Assume a 50' refrigeration line run.

- Loose timeclock (freezer only) installed above walk-in freezer.

Item No 55 - EVAPORATOR COIL, MEDIUM TEMPERATURE                1 Required

Kolpak Industries, Inc. Model No. AM-090,  Unit to be adequately sized to operate Item 50, Refrigerator Compartment (approximately 10'-6" x 23') @ +35° F.  Electrical hook-up and interconnecting of system to be by the Electrical Contractor.

Item No 56 - CONDENSING UNIT, AIR COOLED REMOTE                    1 Required

Kolpak Industries, Inc. Model No. PR204M,  F.S.E.C. to provide the necessary refrigeration lines required to operate and maintain the refrigeration system for Item 50, refrigerated compartment.  Unit to be part of the remote refrigeration system for Item 55, Evaporator Coil.

F.S.E.C. to coordinate with the electrical/ plumbing contractor to provide a complete and operable refrigeration system.

Provide start-up, five (5) year compressor warranty and one (1) year refrigeration service contract.

Division 16 to wire to safety disconnect switch.

- Unit to be located on building roof.  See Architectural plan for location.

- Assume a 50' refrigeration line run.

Item No. 57 - SPARE NUMBER -

Item No. 58 - SPARE NUMBER -

Item No 59 - RACK, UTILITY - UNIVERSAL ANGLE                       LOT Required

Cres Cor Model No. 207-UA-13A,  Rack, Utility - Universal Angle.

- Corner Bumpers

- 6" casters

Item No 60 - SHELVING UNIT                                                          LOT Required

Metro Equipment Corporation Model No. SUPER ERECTA,   Metroseal Construction

Size and shape as shown on plan x 4 tiers high. 74"  high posts to match shelf type.

- Unit to be mobile mounted on 5" diameter polyurethane swivel casters, two (2) swivel and two (2) with brakes.

Item No. 61 - SPARE NUMBER -

Item No. 62 - SPARE NUMBER -

Item No 63 - SHELVING UNIT                                                         LOT Required

Metro Equipment Corporation Model No. Super Erecta Chrome Construction

Size and shape as shown on plan x 5 tiers high. 86" high posts to match shelf type.

- Unit to be mobile mounted 5" diameter polyurethane swivel casters, two (2) swivel and two (2) with brakes.

Item No 64 - DUNNAGE PLATFORM                                             LOT Required

Metro Equipment Corporation Model No.   Metro Bow Tie dunnage racks. Of size and shape as shown on plan.

Item No 65 - RACK, CAN STORAGE                                             4 Required

Eastern Steel Rack Company Model No. CSR-FF-156,  Rack, Can Storage.

- Furnish per manufacturers standard specifications.

Item No. 66 - SPARE NUMBER -

Item No 67 - CENTRAL WATER FILTER W/SURGE TANK           1 Required

Everpure, Incorporated Model No. WELLMATE WM-35WB,  Central Water Filter W/Surge Tank.

- Dual high flow system with two 20" pre- filters and eight XC cartridges.

- Provide with one set of replacement cartridges.

- Plumbing Division to inter - plumb with the following Item Numbers # 117, 124, 167, 169, 174, 186, 187, 191, 192 and 309

- Wired for 120 Volt, single phase with cord and plug.  For low pressure alarm.

- Provide unit with Amtrol Model WX-302 Diaphragm -Type 86 gallon reservoir tank.

- F.S.E. Contractor to coordinate connections with appropriate trades to provide a complete and operable system.

Item No. 68 - SPARE NUMBER –

Item No 69 - REFUSE TILT TRUCK (CAP. 750 LBS)                          1 Required

Rubbermaid Commercial Products Model No. 1314,  Refuse Tilt Truck (cap. 750 Lbs).

- Furnish per manufacturers standard specifications.

Item No 70 - TRASH RECEPTACLE (44 GAL.)                                 LOT Required

This item is not in contract. The specification is for reference only. The item is shown on the drawing for informational purposes and generally represents the size/ capacity of unit to be provided by the Owner.

Rubbermaid Commercial Products Model No. 2643-06,  Round, 44 gal., 24"D x 31.50"H, High-impact plastic construction

Specify color: gray

Item No 71 - HAND SINK                                                                      4 Required

IMC/Teddy Food Service Corp. Model No. CSW,  Hand Sink.

- Gooseneck faucet w/ standard handles.

- Soap and towel dispenser by Owner.

- "P' trap.

Item No 72 - SHELVING UNIT                                                             1 Required

Metro Equipment Corporation Model No SUPER ERECTA,  Super Erecta Chrome Construction

Size and shape as shown on plan x 5 tiers high. 74"  high posts to match shelf type.

- Unit to be mobile mounted 5" diameter polyurethane swivel casters, two (2) swivel and two (2) with brakes.

Item No 73 - MOP BUCKET                                                                 1 Required

This item is not in contract. The specification is for reference only. The item is shown on the drawing for informational purposes and generally represents the size/ capacity of unit to be provided by the Owner.

Rubbermaid Commercial Products Model No. 6186,  Mopping Combo Pack, 6185 mop bucket/6127-01 wringer, foot activated emptying system, non-marking casters, yellow.

Item No 74 - MOP SINK, HEAVY DUTY                                                  1 Required

IMC/Teddy Food Service Corp. Model No. FS-H,  Mop Sink, Heavy Duty.

- Mop holder, 4 pole (MH4).

- Service sink wall faucet (SSF).

- Hose, 36" long (HA).

Item No 75 - CLEANING HOSE ASSEMBLY                                           1 Required

Strahman Valves, Inc. Model No. M-5700-RECESSED,  Globe valves.

- Provide with 50' long red hose w/ stainless fittings.

- Nozzle w/ red renewable rubber cover.

- Coordinate recess in wall w/ General Contractor.  Refer to detail C-8-22A.

Item No 76 - WASTE SYSTEM, PULPER/EXTRACTOR                            1 Required

SOMAT 7-1/2 High Tank Pulper - Close-Coupled, with:

- 7-1/2 HP TEFC (Totally Enclosed Fan Cooled) NEMA design B, direct drive motor.

- Vibration isolation system including (4) 165 lb. rated neoprene compression mount with pulping chamber fully isolated from Pulper shell and surrounding structure.

- HyPoint Cutting mechanism to consist of:

  - Investment cast rotating blade-one piece design, 17-4 stainless steel,

  - Investment cast security ring,  17-4 stainless steel, with

three replaceable stationary cutters, replaceable top cutters.

  - Impeller, 304 stainless steel, with 2 replaceable rotating cutters.

- Pulping chamber of one-piece investment cast 316 stainless steel construction mounted within a 30” diameter pulper shell.

- Completely enshrouded Pulper drive system, shrouding also to encase slurry, return water, and fresh water supply manifolds.  Shrouding to be 304 stainless steel.

- Six inch floor clearance.

- Removable stainless steel lid and associated proximity switch.

- Permanent trough magnet with mounting hardware.

- Spanish/English operating placards.

Hydra-Extractor: Shall consist of rigid stainless steel weldment with supporting frame, head assembly with 2 HP TEFC (Totally Enclosed Fan Cooled) drive motor mounted to a 20:1 gear reducer, removable stainless steel access cover and 17” long stainless steel discharge chute with limit switch.  Water extractor mechanism consists of a 6” diameter stainless steel screw with nylon brush edge, matching 6” diameter reinforced stainless steel screen and stainless steel plug cutter.

The Hydra-Extractor shall also include the following:

- Stainless steel recirculating pump, 3 HP TEFC direct drive, vertically mounted and completely enshrouded.

- Chemical additive pump.

- Auto Rinse System

Valve Package:  Prepiped bronze valve package enclosed in 304 stainless steel module.

Som-A-Trol:	(Electrical control panel) Wall mounted, UL approved, NEMA 4 stainless steel enclosure.

Includes all necessary power, control and water level components prewired to a terminal strip.  A push button station is provided.

Electrical Characteristics:    208 Volt, 3 Phase, 60 Hertz.

Capacity: 1250 lbs. per hour of foodservice waste mix.

# Footprint: 62” L x 34” W x 66” H

Refer to Somat preliminary shop drawing P-4322.

.

Item No. 77 - SPARE NUMBER -

Item No. 78 - SPARE NUMBER -

Item No 79 - TRAY ACCUMULATOR SYSTEM    	                              1 Required

Aerowerks Tray Accumulator

Provide unit of size and shape as shown on plan drawing.

## Pass Thru Window Frame – 1 required

Provide stainless steel full perimeter window frame integral with Tray Accumulator Conveyor.  Food Service Equipment Contractor to verify window opening with architectural drawings.  Provide 18 ga. stainless steel double wall hinged insulated doors for access to area below accumulator for house keeping.

## Security Roll Down Shutter – 1 required

Provide security roll down shutter custom built to size of tray drop opening.  Door shall be manually operated, push up and pull down type with built in counter balance mechanism.  Door, guides and hood shall be stainless steel type 304 construction with # 4 finish.

## Tray Accumulator – 1 required

Provide Tray Accumulator of size and shape as shown on plan. Unit shall be

Aerowerks model TA-243.

Sight and Sound Baffle

- Provide stainless steel sight and sound baffle with lights integral with the Tray Accumulator Conveyor as shown on plan.

- Tray Carriers

- The system shall include 24 columns of carriers. Each column shall include 3 tray carriers. Tray carrier size shall be determined by the size of trays to be used on the system.  The tray carriers shall be constructed of ¼” diameter stainless steel rods all welded and polished. Tray carriers shall be removable from carrier hanger without the use of tools.

- Carrier Hangers

- The carrier hangers shall be constructed of ½” diameter stainless steel rods all welded and polished. The carrier hangers shall be attached to a stainless steel ‘J’ bracket. The carriers shall be removable without the use of tools.

- ‘ J ’ Brackets

- Brackets shall be formed from 10 ga. stainless steel.  Brackets shall be attached to and driven by a conveyor chain.

- Drive Mechanism

- The tray carriers shall be driven by # 60 conveyor chain and sprockets. The drive mechanism shall be supported by stainless steel structural members fitted with stainless steel housing and access panels.

- Drive Motor

- Provide variable speed D.C. motor, with double reduction gearbox reducer. Speed shall be changed by turning speed control knob on control panel.

- Safety Switch

Provide safety switch to shut off conveyor in the event the trays are not fully loaded into the tray carriers.

## Soiled Dish Table – 1 required

Provide Soiled Dish Table, 14 ga. stainless steel top and trough as shown on plan.   Table shall be formed with 9” backsplash along all walls and vertical and horizontal corners coved to a ¾” radius.  Provide three (3) cantilevered pre-rinse sprays, three (3) perforated ledges, magnetic silver saver, and two (2) hose clean up stations as shown on plan.

Trough shall be 9” wide sloping from 3” to maximum with matching flange at Pulper.  Furnish trough with 1 ½” copper pipe connection for recirculating water line.  K.E.C. to provide recirculating water line mounted under Soiled Dish Table from end of trough to Pulper.

Provide pre-rinse sink w/ type 7 pre-rinse faucet per detail C-8-10 where shown on plan.

Support table on 1 5/8” diameter stainless steel legs with adjustable bullet feet.  Legs shall be positioned to align up front-to-back for maximum access for cleaning. Legs shall be supported with 14 gauge stainless steel leg channels.  All crossrails to be fully welded to legs with all welds polished and ground smooth.

Provide two hose stations w/ 30’ hose and heavy duty water gun.

## Main Control Center – 1 required

Provide Main Control Center containing main disconnect, start-stop, speed control, and indicating lights. All components shall be neatly contained in a stainless steel watertight enclosure.  All wiring shall conform to the latest UL standards.  The electrical contractor shall bring 208V/1phase/15A to the main of the panel, but wiring from the equipment to this panel shall be done by Aerowerks, and all wiring shall be carried in liquid-tight conduits, including conveyor motor and controls.

## Installation & Warranty – 1 required

Aerowerks shall be responsible for all interconnections of plumbing and electrical work required for conveyor system. Final connections to building services by others. Aerowerks shall install and adjust system to owner’s satisfaction and shall provide adequate instructions to operating personnel. System shall be guaranteed for a period of one year for parts and labor under normal operating conditions, from date system is turned over to owner.

Refer to Aerowerks preliminary shop drawing P061644 dated 12/05/06

Item No 80 - DISHWASHER, CONVEYOR W /BOOSTER       1 Required

Hobart Corporation Model No. CPW100A,  Unit to be complete with the following options and/or modifications:

- Provide with floor mounted steam booster heater wired for 208 volt, single phase. Provide removable s/s enclosure for floor mounted steam booster.

- Electrical Division to wire to safety disconnect switch.

- Provide with stainless steel vent cowls with 7" stacks and locking dampers at both ends.

- Right to left operation.

- Provide steam coil tank heat.

- Provide with table limit switch.

- Provide twelve dishracks.  Confirm rack type with Owner.

- Provide two vent hoods with locking dampers.

- Single point electrical connection.

- SL-30 Sideloader.

Refer to Hobart preliminary shop drawing Y-292104-827.

Item No 81 - SIDELOADER                                                    1 Required

Hobart Corporation Model No. SL30LR,  Sideloader.

- Specified as part of Item 80, Dishwasher.

Item No 82 - DISHWASHER VENT DUCT, LOAD END        1 Required

Custom fabrication. Construct per detail C-8-13. (inverted louvers).FSE Contractor to verify ceiling height.

Item No 83 - DISHWASHER VENT DUCT, UNLOAD END   1 Required

Custom fabrication,  Construct per detail C-8-13. (inverted louvers). FSE Contractor to verify ceiling height.

Item No 84 - VENT HOOD, LOAD END                                 1 Required

Hobart Corporation.  Vent Hood, Load End.

- Specified as part of Item 80, Dishwasher.

Item No 85 - VENT HOOD, UNLOAD END                             1 Required

Hobart Corporation. Vent Hood, Unload End.

- Specified as part of Item 80, Dishwasher.

Item No 86 - DISHTABLE, CLEAN                                        1 Required

Custom fabrication  Unit to be of size and shape as shown on plan x 34" high to drainboard level with 10" high splash per details C-1-1A Type "D" edge, C-1-1B backsplash and C-8-1.  Provide with the following:

- Open base type table, no undershelf.

- Front to back and rear crossrails only.

- Accommodate table limit switch as shown on electrical plan.

- Accommodate Steam Booster Heater, part of Item 80.

Item No 87 - DISH DOLLY                                                   LOT Required

Metro Equipment Corporation Model No. PCD9,  Dish Dolly.

- F.S.E.C. to confirm plate size with Owner.

Item No 88 - WALL SHELF                                                  1 Required

Custom fabrication,  Unit to be of size and shape as shown on plan per detail C-1-2.

Item No 89 - POT & PAN RACK, MOBILE                         LOT Required

Metro Equipment Corporation . Pot & Pan Rack, Mobile.

- Furnish per manufacturer's standard specifications.

- Three model PR60ES and one model PR48S.

Item No 90 – POWASH SINK W/ POWER SOAK, SCRAPPER & DISPOSER   	 1 Required

Custom Fabrication,  Power Soak W/ Scrapper & Disposer.  Unit to be U-shaped size and shape as shown on plan, 36" high to drainboard.

- Provide 10" high splash at rear with ends closed.

- Furnish with two (2) Type 2 faucets where shown on plan.

- Provide drop ledge on front of this item per CFL standard detail C-8-3A.

- Provide open base type table, no undershelves.

- Provide scrap sink where shown on plan and sink mount disposer manufactured by Insinkerator model #SS-300. Provide collar adapter and CC-101 control center.

- Ship disposer and related components to fabricator.

- Provide control panel mounting bracket per C-8-20 for mounting disposer control center.

- Furnish table with full length table mounted overshelf, 18" wide. Provide three (3) 1-5/8" O.D. posts with die stamped raised hole and thru table top for cross rail support. Refer to detail C-1-6  Overshelf to be mounted 20" above counter top.

- Return angled splash to Item 94, Pot Washing Machine as required.

- Construct similar to CFL Standard Details C-1-1A Type "D" edge, C-1-1B backsplash, C-1-6, C-8-1 and C-8-3A.

- (1) Wash (weld-in) tank 42” X 28.375” X 19” deep has wash jets spaced every 6” along back wall of the wash tank; average flow rate per wash jet approx. 40gpm.  All wetted parts of the pump assembly are stainless steel.  Continuous duty motor provided with all pump assemblies.  Motor size 2HP.  NO retained water in discharge manifold or pump housing when system is drained.  Integrated controls monitor liquid level sensors, water temperature sensor and all safety switches.  7000 watt heater located along the rear wall under the perforated intake guard.  Liquid level sensors mounted to detect low/high water levels within wash tank.

Open base construction with S/S sockets, 1-5/8” dia. s/s legs, 1-1/4” dia. s/s cross rails, s/s feet.

Item No. 91 - SPARE NUMBER -

Item No 92 – POT TABLE, CLEAN                   	                                    1 Required

Custom fabrication.  Unit to be of size and shape as shown on plan x 34" high to drainboard level with 10" high splash per details C-1-1A Type "D" edge, C-1-1B backsplash and C-8-1.  Provide with the following:

- Undershelf.

Item No 93 - VAPOR HOOD, WALL MOUNTED      	                            1 Required

Gaylord Industries, Incorporated Model No. VH2-W-54,  Unit to be Type II vapor hood of size and shape as shown on plan x 24" high and mounted 6'-8" AFF.

- Optional light.

Refer to Gaylord preliminary shop drawing 06-0050 sheet 1 of 13.

Item No 94 - POTWASHER                                  	                                1 Required

Champion Industries, Inc. Model No. PP-28,  Straight - through load operation. S/S steam closed coil w/thermostat & low-water cutoff.

- Rack, s/s, counter pan

- Rack, s/s, utility

- Rack, bake sheet

- Booster heater, steam, 40 degree rise built-in.

- Single point steam connection.

Item No 95 - FLOOR TROUGH W/GRATING    	                                   1 Required

IMC/Teddy. Model No. SFT,  Unit of size and shape as shown on plan.

This item to be set and installed by General Contractor. Quarry tile floor finish to be etched, if required, prior to setting food service equipment in place. Provide grating sections of equal size not to exceed 20" long. Provide shop drawings of size and scale outlined in specific conditions.

- Grating to be anti-slip, stainless steel.

- Shallow floor trough, 2" deep.

- Field dimension location of penetrations in floor for direct wastes. Fabricate waste connections on trough to suit floor penetrations.

Item No 96 - CART, UTILITY            	                                                 1 Required

Lakeside Manufacturing, Inc. Model No. 722,  Cart, Utility.

- Furnish per manufacturers standard specifications.

Item No. 97 - SPARE NUMBER -

Item No. 98 - SPARE NUMBER -

Item No. 99 - SPARE NUMBER -

Item No 100 - TABLE W/UTIL. CHASE, SINK & HAND SINK    	      1 Required

Custom fabrication. Unit to be size and shape as shown on plan x 36" high w/ 6" high splash per detail C-1-1A Type "A" edge, C-1-1B backsplash and C-7-1.  (Define) end splashes as shown on plan.  Provide with the following:

- Shelf, table mounted, single tier of size and shape as shown on plan per details C-7-8.

- Shelf to be turned up 2" at utility chase.

- Receptacle mounted in utility chase, to service Item 101.  Construct similar to detail C-14-1.  Refer to electrical plan.

- Utility chase of size and shape as shown on plan. Secure and seal chase to 2" coved turn up at fixture table top per detail C-2-12. Verify ceiling height.

- 9"x 12" x 6" deep hand sink per detail C-2-1E.  Provide with Type 5B faucet.

- Fabricator to provide an integral side/ rear splash, 6" high x 1" thick as shown on plan to isolate hand sink from table similar to C-2-1E. Soap and towel dispenser by Owner.

- Undershelf per C-7-1.

- Duplex receptacles on mounting plates below top per C-14-1. Refer to electrical plan.

Item No 101 - SLICER, FOOD                   	                                       1 Required

Hobart Corporation Model No. 2612+BUILDUP,  Slicer, Food.

- Furnish per manufacturers standard specifications.

Item No 102 - MIXER, 60-QUART          	                                          1 Required

Hobart Corporation Model No. HL600,  Mixer, 60-Quart.

- Stainless steel bowl.

- "B" beater.

- "D" wire whip.

- "ED" dough hook.

- Bowl scraper.

- Bowl truck.

Item No 103 - TABLE W/SINK & UTILITY CHASE             	          1 Required

Custom fabrication. Unit to be size and shape as shown on plan x 36" high w/ 6" high splash per detail C-1-1A Type "A" edge, C-1-1B backsplash and C-7-1.  Provide with the following:

- Shelf, table mounted, single tier of size and shape as shown on plan per details C-7-8.

- Shelf to be turned up 2" at utility chase.

- Utility chase of size and shape as shown on plan. Secure and seal chase to 2" coved turn up at fixture table top per detail C-2-12. Verify ceiling height.

- Undershelf per C-7-1.

- Duplex receptacles on mounting plates below top per C-14-1. Refer to electrical plan.

- One 20" x 20" x 10" deep sink.

Item No 104 - CHILLER, BLAST REACH-IN      	                          1 Required

This item is not in contract.  It is “FUTURE”. The specification is for reference only.  The FSE Contractor will show rough-in connections for this items on the dimensioned rough-in plans.

Traulsen & Company, Inc. Model No. RBC100,  Chiller, Blast Reach-In.

- Bottom mounted electric condensate evaporator.

- Set of four 6" high locking casters.

- Hinge door as shown on plan.

- 5 year compressor warranty and 1 year service contract.

- One day on-site blast chill training.

Item No 105 - TABLE, MOBILE                               	                       2 Required

Custom fabrication. Unit to be size and shape as shown on plan x 36" high w/ 6" high splash per detail C-1-1A Type "A" edge and C-7-1C.  Provide with the following:

- Undershelf per C-7-1.

- 5" diameter polyurethane casters.

Item No. 106 - SPARE NUMBER -

Item No. 107 - SPARE NUMBER -

Item No 108 - SHELVING                           	                                     LOT Required

Metro Equipment Corporation Model No. SUPER ERECTA chrome construction.  Of size and shape as shown on plan x 5 tiers high. 74" high posts to match shelf type.

- 5" diameter polyurethane swivel casters, two (2) with brakes.

Item No. 109 - SPARE NUMBER –

Item No 110 - CHARBROILER, GAS                        	                         1 Required

Magikitch'n Equipment Corp. Model No. FM-RMB-648,  Charbroiler, Gas.

- Richlite cutting board w/ 10" service shelf.

- Special cleaning brush

- Scraper

- 4 casters, 2 locking & 2 non-locking

Item No 111 - FLOOR PAN & TROUGH W/GRATING         	           1 Required

IMC/Teddy. Model No. FPT,  Unit of size and shape as shown on plan. This item to be set and installed by General Contractor. Quarry tile floor finish to be etched, if required, prior to setting food service equipment in place. Provide grating sections of equal size not to exceed 20" long. Provide shop drawings of size and scale outlined in specific conditions.

- Grating to be anti-slip, stainless steel.

- Provide floor pan of size and shape as shown on plan.

- Provide floor pan with plinths for level mounting of Items 112, 113 and 115.

- Floor trough, 3" deep set on the existing slab. Confirm depth of mud and tile w/ General Contractor.

- Optional wastes and locations. Confirm floor penetration locations for wastes and locate wastes on trough accordingly.

- 6” high splash, 2” thick w/ enclosed rear and ends, at rear of pan.

Item No 112 - TILT SKILLET, GAS, MANUAL TILT           	              1 Required

Cleveland Range Company Model No. SGL40T1,  Tilt Skillet, Gas, Manual Tilt.

- 2" tangent draw-off valve, front mounted left side

- Double pantry faucet w/ 3/4" swing spout (adds 2 1/4" to width) w/ mounting bracket.

Item No 113 - KETTLE, TILTING, STEAM                     	               1 Required

Cleveland Range Company Model No. KDL-40-T.

- 3" tangent draw-off valve with strainer

- Kettle gallon markings, (per each gallon mark)

- Spring-assisted cover

- 316 S/S liner

- Double pantry faucet w/ 3/4" spout w/ faucet bracket (req'd for mounting faucet)

- Measuring Strip

Item No 114 - CLEANING HOSE ASSEMBLY        	                      1 Required

Strahman Valves, Inc. Model No. M-5700 RECESSED,  Cleaning Hose Assembly.

- Provide 30' long red hose w/ stainless fittings.

- Nozzle w/ red renewable rubber cover.

Item No 115 - KETTLE, TILTING, STEAM               	                      1 Required

Cleveland Range Company Model No. KDL-60-T.

- 3" tangent draw-off valve with strainer

- Kettle gallon markings, (per each gallon mark)

- Spring-assisted cover

- 316 S/S liner

- Double pantry faucet w/ 3/4" spout w/ faucet bracket (req'd for mounting faucet)

- Measuring Strip

Item No. 116 - SPARE NUMBER

Item No 117 - COMBI OVEN-STEAMER                             	           1 Required

Rational Combi-Oven/Steamer Model No. CPC 202G,  Combi Oven-Steamer.

- Special grids for rotisserie chicken.

- The (10) 2 x 1 grids.

- Two units ea. of cleaning tabs

- Two units ea. of rinsing tabs

- One additional mobile oven rack.

- Connect to filtered cold water connection per manufacturer’s recommendations thru Item 121, Utility Distribution System, fed from Item 67, Central Water Filter w/ Surge Tank.

Item No. 118 - SPARE NUMBER -

Item No. 119 - SPARE NUMBER -

Item No. 120 - SPARE NUMBER –

Item No 121 - UTILITY DISTRIBUTION SYSTEM              	             1 Required

GAYLORD INDUSTRIES/DISTRIBUTOR Model DB-IM #304 Stainless steel, self contained load center Utility Distribution System (UDS) width and length as shown on plans.

System shall be completely pre-wired and pre-piped to a single connection point for electrical, cold water and hot water, gas, and steam as required by the equipment specified. All plumbing and electrical services shall include a minimum of 25% over-capacity for future changes in equipment. The UDS shall be UL listed under the category "Commercial Appliance Outlet Center for Damp Locations" and manufactured in accordance with the National Electric Code (NEC), National Fire Protection Association (NFPA 96), Uniform Plumbing Code (UPC), National Sanitation Foundation (NSF), and using only UL and AGA certified components. The UDS shall be completely pre-wired and pre-plumbed to one final connection point for the following services:

ELECTRICAL:  100 AMPS. 208Y/120/30/3 PHASE; 5 WIRE SERVICE TO MAIN DISCONNECT WITH SHUNT - TRIP BREAKER IN UDS. POWER SUPPLY MUST BE WYE STYLE

120/60/1 PHASE; 3 WIRES FROM VENTILATOR LIGHTS TO TERMINAL BLOCKS IN UDS.

120/60/1 PHASE, 3 WIRES FROM FIRE SUPPRESSION SYSTEM TO TERMINAL BLOCK IN UDS.

120/60/1 PHASE, 2 WIRES FROM TERMINAL BLOCK IN UDS TO SUPPLY FAN STARTER COIL.

120/60/1 PHASE, 2 WIRES FROM TERMINAL BLOCK IN UDS TO EXHAUST FAN STARTER COIL.

4 WIRES FROM UDS TO EACH DAMPER CONTROL J-BOX ON EACH VENTILATOR SECTION.

NATURAL GAS:  2.00” LOOPED SERVICE, 1000 MBH TOTAL.

COLD WATER: 1” DIAMETER.

HOT WATER: 3/4" DIAMETER.

STEAM: 2.00” 525 LBS/HR AT 25 PSI.

CONDENSATE RETURN: 1.00”

The UDS shall be constructed entirely of stainless steel type 304, #4 finish, and not less than 18 gauge. This includes the vertical risers, peaked top, end caps, horizontal raceway, hinged and removable panels. Unexposed items such as the main electrical chase, load center for individual breaker housing, and the inside raceway section shall also be fabricated from 18 gauge stainless steel. The insulating and supporting materials for the plumbing and electrical components are the only exception to this. The main electrical panel, electrical raceway, and all electrical distribution components shall be isolated from the plumbing chamber and plumbing raceway in accordance with national electric codes. Individual electrical compartments for electrical components in the main electrical riser for separation from the plumbing section will not be accepted.  All hardware for utility interconnects for multiple section raceways and risers to be provided by the manufacturer for field re-assembly.

The UDS shall be sized as shown on plans having a peaked cap on the horizontal raceway. The UDS shall be supplied with two full height vertical riser/base combinations and raceway as shown on plans. The raceway system shall be mounted 20" above finished floor and shall not exceed 51" in height.

The riser shall enclose the main circuit breaker with disconnect handle and shunt trip coil for fire/fuel shut-off. The main riser shall also enclose all switches, relays, solenoid valves, and terminal blocks necessary for fire/fuel shut down of the equipment and shall be factory wired in accordance with the national electric codes. The main riser shall feed the ventilator lights and the internal power connections. A power interruption device, (PID) shall be installed to keep solenoid gas valve open in the event of a momentary power loss.  The 120/208 volt 3 phase service is to be distributed through the insulated copper busbar system to individual point of use circuit breakers wired to the equipment receptacle. The riser end access door shall be a full height door mounted on a full length stainless steel hinge and shall be secured with a locking corrosion resistant twist latch-closing device. The main breaker, emergency kill switch, gas valve reset button with PID, ventilator light switch, ventilator fan switch, convenience outlet and equipment status lights shall be mounted on the riser end access door. The removal of access plates to individual electrical compartments to service these items will not be acceptable. The main breaker, emergency kill switch, gas valve reset button with PID, ventilator light switch, ventilator fan switch, convenience outlet and equipment status lights shall be clearly engraved on plastic laminated labeling which is mounted to the front of the access door. The access door on the end of the main riser shall be recessed 1" to protect all exposed electrical components from damage. The ventilator light switch and fan switch shall have a spring hinged weatherproof protected cover with. Main access door shall be designed so it cannot open unless the main power for the unit is turned off. Access to the primary plumbing utility connection inlets shall be by way of latching side lift-out panels on the side of the riser.

The secondary riser to be similar to the main riser except there is no electrical. There is to be a general use GFI convenience duplex outlet mounted in a weatherproof box. Access to primary utilities to be by way of latching side lift-out panels on both sides.

The electrical distribution system shall be sized and installed as per the NEC and pre-wired for easy interconnection. The busbar shall be 100% copper, insulated, and pre-punched every three inches for future connections. The use of load centers or hardwired systems will not be accepted. All breakers are to be Square D brand, thermal magnetic style, with 10,000 A/C ampere inrush capacity. All point of use circuit breakers shall be sized according to equipment load and shall be clearly labeled to identify volts, phase, amps and connection number (per UDS equipment schedule). A corresponding status light shall be located in the main riser door for each circuit breaker. Point of use circuit breakers and receptacles connection plates shall be located on the side of the horizontal raceway, which are individually grounded to the system main frame. Each connection plate shall be common and interchangeable and shall be furnished with a moisture resistant clear plastic cover and grounding type receptacle having a specific NEMA polarized configuration. All plugs and receptacles to be straight blade type. Ground fault interruption (GFI) for personnel protection (4 to 6 milliamperes) to be provided for all outlets as required by N.E.C.

Field joints at risers and raceways shall be provided with quick disconnect snap plug-ins for easy reconnection of control wiring and include wiring required for reconnection of buss bar system. The reconnection of the field joints shall be by the job site electrical contractor. Cord and plug sets with strain relief devices shall be furnished for all equipment items not provided with cord and plug sets. The job site electrical contractor shall be responsible for the installation of the cord strain relief devices for the equipment that requires these items.

Connection points for the surface fire protection shall be provided for fire/fuel shutdown of the electrical system and electric gas valves as required by NFPA 96. The job site electrical contractor shall be responsible for connections required between the fire system microswitches and the UDS.

Access to the plumbing compartment shall be by way of side lift-out panels on the side of the plumbing compartment. Gas system shall consist of a black iron gas manifold, provided with pre-wired electric gas valves. The main connections shall include a quarter turn ball valve in each riser. Unions shall be provided at the end of each field joint. The field joint reconnection in the field for the unions shall be the responsibility of the job site plumber. Each outlet connection shall include an AGA approved ¼ turn shut off valve and quick disconnect coupling. AGA approved plastic covered stainless steel flexible gas hoses shall be provided and shall include one safety restraining cable device per each mobile appliance for attachment from the UDS to the appliance. All gas outlets shall be clearly labeled for identification. Steam system shall consist of a black iron steam manifold and condensate return manifold, insulated with 1” thick fiberglass insulation. The main connections shall include a quarter turn ball valve in the riser. The end riser shall also be equipped with an inverted bucket trap with “Y” strainer for condensate return and line protection.  Unions shall be provided at the end of each field joint. The field joint reconnection in the field for the unions shall be the responsibility of the job site plumber. Each outlet connection shall include an approved ¼ turn shut off valve and quick disconnect coupling. Approved plastic covered stainless steel flexible hoses shall be provided for steam connections and condensate return connections for attachment from the UDS to the appliance. All steam and condensate return outlets shall be clearly labeled for identification.  Hot and cold water supply lines shall be hard drawn type "L" copper insulated with 1/2" thick cellular foam insulation throughout. Also include a main quarter-turn manual shut-off valve in one vertical end riser. For each piece of equipment, provide hose connections as required. Each hose connection shall be provided with a quarter-turn ball valve and two-way disconnect coupling. Each field joint shall have unions for easy re-assembly at the job site. The job site plumber shall be responsible for the reconnection of the unions. A combination pressure/temperature gauge shall be provided for the hot water line. Shock arrestor shall be provided and installed at the hot and cold water supply inlets to the UDS. For each water connection to the equipment, provide an approved plastic covered stainless steel flexible hose. All plumbing outlets shall be clearly labeled for identification at each access location.

Interface all Utility Distribution System construction with ITEMS 116/122 VENTILATOR and  FIRE SUPPRESSION SYSTEM and actual field measurements at job site. Provide inspection of installation at initial equipment start-up by qualified factory trained personal to insure that connections have been correctly made and that the unit is functioning properly.

This item and item NO 116/122 - VENTILATOR shall be by the same manufacturer for coordination purposes.

Refer to Gaylord preliminary shop drawing 06-0050 sheets 5 & 6 13.

Item No 122 - EXHAUST HOOD                        	                                   1 Required

Gaylord Industries, Incorporated Model No. GX2-BDL-BB-CA-136.  Unit to be U.L. listed grease extractor, high velocity island canopy of size and shape as shown on plan x  30" high, built in four section(s). and mounted at 6’-8” AFF.

One (1) lot stainless steel enclosure paneling, at all open sides, to finished ceiling (F.S.E. Contractor to verify height). Paneling must be supplied by Exhaust Hood Manufacturer and installed by F.S.E. Contractor. Adhere to Specific Conditions for installation of fabricated equipment.

Ventilator to be provided w/ full length fire damper located in the ventilator extractor plenum between the extractor inserts and the ventilator duct collar.  The damper shall be controlled by an electrically driven actuator specifically listed for such use and shall be activated by fail safe thermostats located at the duct collar.  The damper shall close each time the exhaust fan is turned off, to prevent conditioned air from escaping the occupied space via thermal drafts and then re-open when the exhaust fan is started.

Ventilator to be U.L. and NSF listed and be in full accordance with NFPA 96 and all state/local codes/ordinances applicable.

F.S.E. Contractor to supply the necessary mounting rods for hanging ventilator.

Supply and/ or exhaust air requirements as shown on plans.

Options:

- Recessed incandescent light fixtures as indicated on electrical plan.

- Pre-piping of the Item No. 156, Fire Protection System is to be provided by the Exhaust Hood manufacturer at the time of hood fabrication.

- Pre-piping to include fusible-link detector bracket assemblies and cable runs, and wet-chemical manifold piping, duct collar and plenum protection piping with termination nozzles, and cooking appliance nozzle drops with termination nozzles.

- Location of cooking appliance nozzle drops are to be fully dimensioned on hood submission drawings.

- Fusible link detector and wet chemical piping terminations to be coordinated with the FSE Contractor and clearly indicated on hood submission drawings.

- Access to the fusible links for inspection and change-outs to be provided.  The hood manufacturer and FSE Contractor to coordinate their efforts to assure the necessary access.

- Supply air plenum boxes w/ full length internal removable baffle, mounted at finished ceiling height.

FSE Contractor is responsible for fit of equipment. Prior to releasing hoods for fabrication, FSE Contractor needs to verify field conditions (existing &/or proposed) and to determine clearances to all structural items, obstructions, etc. The FSE Contractor will coordinate with other trades to confirm that the hood can be mounted as proposed and that the ductwork and final connection can be accommodated without conflict.  Failure to perform this step may result in modifications to the exhaust hood at the FSE Contractor's expense.

Refer to Gaylord preliminary shop drawing 06-0050 sheets 2, 3 & 4 of 13.

Item No 123 - HOLDING CABINET                                                          1 Required

Winston Industries Model No. HA4022-HL-5,  Holding Cabinet.

- Caster size, 5"

- 4 wire rack/ trays.

Item No 124 - STEAMER, 10 PAN, DIRECT STEAM                                1 Required

Cleveland Range Company Model No. 24CDP10

- Right hand door hinging, controls on right

- ON/OFF compartment controls only, no timer (NC)

- 10" adjustable legs, shipped loose (floor models only)

- Descaling solution, 6-one gallon containers.

- Connect to filtered cold water connection per manufacturer’s recommendations thru Item 121, Utility Distribution System, fed from Item 67, Central Water Filter w/ Surge Tank.

Item No 125 - OVEN, CONVECTION GAS                                               2 Required

Blodgett Oven Company Model No. DFG100 DOUBLE,  Oven, Convection Gas.

- Solid-State Manual control.

- Stainless steel draft diverter.

- 4" casters(set).

- Gas manifold (double section)

Item No 126 - FRYER, DEEP FAT GAS                                                   1 Required

Keating of Chicago, Inc. Model No. 18BB,  Fryer, Deep Fat Gas.

- Battery w/ Items  127, Prep Station on a common frame

- Polyurethane casters with brakes

Item No 127 - FRYER DRAIN CABINET                                                  1 Required

Keating of Chicago, Inc. Model No. 18-003908,  Fryer Drain Cabinet.

- Battery on common frame w/ Item 126, Fryer.

Item No. 128 - SPARE NUMBER -

Item No. 129 - SPARE NUMBER -

Item No 130 - RANGE, FRENCH TOP                                                    1 Required

Jade Range, Incorporated Model No. JTRH-2FHT-36,  Range, French Top.

- Stainless steel left, right or both sides

- Additional oven rack.

- Rear gas connection.

- End caps and cover.

- Swivel casters (4) w/ front brakes.

- Single deck stainless steel tubular high shelf.

- Two 30,000 BTU/hr. burners

Item No 131 - RANGE, HEAVY DUTY GAS                                          2 Required

Jade Range, Incorporated Model No. JTRH-4-36,  Range, Heavy Duty Gas.

- Stainless steel left, right or both sides

- Additional oven rack.

- Rear gas connection.

- End caps and cover.

- Swivel casters (4) w/ front brakes.

- Single deck stainless steel tubular high shelf.

- Four 30,000 BTU/hr. burners

Item No. 132 - SPARE NUMBER -

Item No. 133 - SPARE NUMBER -

Item No 134 - TABLE WITH UTILITY CHASE                                     1 Required

Custom fabrication. Unit to be size and shape as shown on plan x 36" high w/ 6" high splash per detail C-1-1A Type "A" edge, C-1-1B backsplash and C-7-1.  Provide with the following:

- Junction box mounted on bracket below table top to service Item 102, Mixer. Construct similar to detail C-14-1.  Refer to electrical plan.

- Utility chase of size and shape as shown on plan. Secure and seal chase to  2" coved turn up at fixture table top per detail C-2-12. Verify ceiling height.

- Undershelf per C-7-1.

- Duplex receptacles on mounting plates below top per C-14-1. Refer to electrical plan.

Item No 135 - POT & PAN RACK, CEILING MOUNTED                  1 Required

Custom fabrication. Unit to be of size and shape as shown on plan per detail C-7-4A.  Anti-sway bracing above finished ceiling to be provided by F.S.E. Contractor.

Item No 136 - TABLE W/ HAND SINK & UTILITY CHASE                1 Required

Custom fabrication  Unit to be size and shape as shown on plan x 36" high w/ 6" high splash per detail C-1-1A Type "A" edge, C-1-1B backsplash and C-7-1.  Provide with the following:

- Receptacle mounted in utility chase, to service Item 101.  Construct similar to detail C-14-1.  Refer to electrical plan.

- Utility chase of size and shape as shown on plan. Secure and seal chase to  2" coved turn up at fixture table top per detail C-2-12. Verify ceiling height.

- 9"x 12" x 6" deep hand sink per detail C-2-1E.  Provide with Type 5B faucet.

- Fabricator to provide an integral side/ rear splash, 6" high x 1" thick as shown on plan to isolate hand sink from table similar to C-2-1E. Soap and towel dispenser by Owner.

- Undershelf per C-7-1.

- Duplex receptacles on mounting plates below top per C-14-1. Refer to electrical plan.

Item No 137 - INGREDIENT BIN                                                          3 Required

Rubbermaid Commercial Products Model No. 3600,  Ingredient Bin.

- Furnish per manufacturers standard specifications.

Item No 138 - SLICER, FOOD                                                             1 Required

Hobart Corporation Model No. 2612  Slicer, Food.

- Furnish per manufacturers standard specifications.

Item No. 139 - SPARE NUMBER -

Item No. 140 - SPARE NUMBER -

Item No. 141 - SPARE NUMBER -

Item No. 142 - SPARE NUMBER –

Item No 143 - TABLE W/SINKS & UTILITY CHASE                        1 Required

Custom fabrication This worktable to consist of two tables “back to back” tack welded and sealed at common back splash per C-7-1 and C-1-1B (back to back).

Front:

- Unit to be of size and shape as shown on plan x 36" high with 6" high splash at rear with ends and back closed.

- Provide one (1) 24" x 24" x 12" deep sink with rotary waste where shown on plan.

- Drain handles to be provided with a # 14 ga. stainless steel valve bracket located at sink bowl front.  Refer to Detail C-8-5 for construction.

- F.S.E. Contractor to furnish sink with one (1) type 1 faucet.

- Provide apron at sink fronts.

- Provide an undershelf to the left of sink bowl.

- At center of table, provide a 20" x 20" x 5" deep utility drawer constructed similar to C-1-3A with C-1-3C flush horizontal drawer pull with C-1-3D drawer hasp.

- Furnish table with full length table over shelf, 24" wide. Provide four (4) 1-5/8" O.D. posts thru splash and cross rail supported.  Refer to detail C-1-6  Overshelf to be mounted 20" above counter top. Notch shelf at utility chase.

- Where shown on plan, provide one (1) 4" x 36" utility chase with access panels for utility services.  Construct similar to CFL Standard Detail C-2-12  F.S.E. Contractor  to verify ceiling height.  Utility chase to support shelf at end.

- Construct similar to CFL Standard Details C-1-1A Type "A" edge, C-1-1B backsplash,  C-1-3A, C-1-3C, C-1-3D, C-1-6, C-2-12, C-7-1, C-8-5 and C-8-17.

Back:

- Unit to be of size and shape as shown on plan x 36" high with 6" high splash at rear with ends and rear closed.

- Provide one (1) 20" x 20" x 12" deep sink with rotary waste where shown on plan.

- Drain handles to be provided with a # 14 ga. stainless steel valve bracket located at sink bowl front.  Refer to Detail C-8-5 for construction.

- F.S.E. Contractor to furnish sink with one (1) type 1 faucet.

- Provide apron at sink fronts.

- Provide an undershelf to the right of sink bowl.

- At the center of the table, provide a 20" x 20" x 5" deep utility drawer constructed similar to C-1-3A with C-1-3C flush horizontal drawer pull with C-1-3D drawer hasp.

- Construct similar to CFL Standard Details C-1-1A Type "A" edge, C-1-1B backsplash,  C-1-3A, C-1-3C, C-1-3D, C-1-6, C-2-12 and C-7-1.

- Provide recessed splash mounted receptacles, recessed chase mounted receptacles and J- boxes and table-mounted receptacles per CFL standard detail C-14-1 as indicated on plan KB-3.

Item No 144 - FOOD CUTTER                                                                   1 Required

Hobart Corporation Model No. 84186+BUILDUP,  Food Cutter.

12 attachment hub.

Item No. 145 - SPARE NUMBER –

Item No 146 - MIXER, 20-QUART                                                            1 Required

Hobart Corporation Model No. HL200+BUILDUP,  Mixer, 20-Quart.

- Stainless bowl.

- Stainless steel flat beater.

- Stainless wire whip.

- Bowl splash cover.

Item No. 147 - SPARE NUMBER -

Item No. 148 - SPARE NUMBER -

Item No. 149 - SPARE NUMBER –

Item No 150 - TABLE W/ SINKS & UTILITY CHASE                               1 Required

Custom fabrication. This worktable to consist of two tables “back to back” tack welded and sealed at common back splash per details C-7-1 and C-1-1B (back to back). 	 Front:

- Unit to be of size and shape as shown on plan x 36" high with 6" high splash at rear with ends and back closed.

- Provide one (2) 24" x 24" x 12" deep sinks with rotary wastes where shown on plan.

- 9"x 12" x 6" deep hand sink per detail C-2-1E.  Provide with Type 5B faucet.

- Fabricator to provide an integral side splash, 6" high x 1" thick as shown on plan to isolate hand sink from table similar to C-2-1E. Soap and towel dispenser by Owner.

- Drain handles to be provided with a # 14 ga. stainless steel valve bracket located at sink bowl front.  Refer to Detail C-8-5 for construction.

- F.S.E. Contractor to furnish sinks with one (1) type 1 faucet.

- Provide apron at sink fronts.

- Provide an undershelf to the right of the 2 compartment sink.

- At center of table, provide a 20" x 20" x 5" deep utility drawer constructed similar to CFL Standard Details C-1-3A with C-1-3C flush horizontal drawer pull with C-1-3D drawer hasp.

- Furnish table with full length table over shelves, 24" wide. Provide four (4) 1-5/8" O.D. posts  thru splash and cross rail supported.  Refer to detail C-1-6  Overshelf to be mounted 20" above counter top.

- Where shown on plan, provide one (1) 4" x 36" utility chase with access panels for utility services.  Construct similar to CFL Standard Detail C-2-12  F.S.E. Contractor  to verify ceiling height.  Utility chase to support shelf at end.

- Construct similar to CFL Standard Details C-1-1A Type "A" edge, C-1-1B backsplash,  C-1-3A, C-1-3C, C-1-3D, C-1-6, C-2-12, C-7-1, C-8-5 and C-8-17.

Back:

- Unit to be of size and shape as shown on plan x 36" high with 6" high splash at rear with ends and back closed.

- At the center of the table, provide a 20" x 20" x 5" deep utility drawer constructed similar to CFL Standard Details C-1-3A with C-1-3C flush horizontal drawer pull with C-1-3D drawer hasp.

- Offset top at Item 146, Mixer 6".  Elevation of top at Item 146 to be 30" AFF.

- Construct similar to CFL Standard Details C-1-1A Type "A" edge, C-1-1B backsplash, C-1-3A, C-1-3C, C-1-3D, C-1-6, C-2-12 and C-7-1.

- Provide recessed splash mounted receptacles, recessed chase mounted receptacles and J- boxes and table-mounted receptacles per CFL standard detail C-14-1 as indicated on plan KB-3.

Item No 151 - SALAD DRYER                                                                 1 Required

Mannhart Inc. Model No. SD-PE,  Salad Dryer.

- Furnish per manufacturers standard specifications.

Item No 152 - FOOD PROCESSOR                                                        1 Required

Mannhart Inc. Model No. M3000,  Food Processor.

- Provide starter kit of eight (8) cutting blades. Confirm requirements w/ Operator prior to ordering.

Item No. 153 - SPARE NUMBER -

Item No 154 - PEELER, VEGETABLE W/ DISPOSER                           1 Required

Hobart Corporation Model No. 6430,  Peeler, Vegetable W/ Disposer.

- Provide with Model # FD2-50 with long upper housing Disposer.

- Switch to be set at location B.

- Provide unit with disposer stand.

Item No 155 - TABLE WITH UTILITY CHASE & O-SHELVES               2 Required

Custom fabrication. Unit to be size and shape as shown on plan x 36" high w/ 6" high splash per detail C-1-1A Type "A" edge, C-1-1B backsplash and C-7-1.    Provide with the following:

- Shelf, table mounted, two tiers of size and shape as shown on plan per details C-7-8.

- Undershelf per C-7-1.

- Where shown on plan, provide one (1) 4" x 18" utility chase with access panels for utility services.  Construct similar to CFL Standard Detail C-2-12  F.S.E. Contractor  to verify ceiling height.  Utility chase to support shelf at end.

- One (1) 20" x 20" x 5" deep utility drawer constructed per details C-1-3A, C-1-3B, C-1-3C and C-1-3D. Lock hasp.

- Angle slides at bottom of drawer w/one (1) Richlite 1/2” thick cutting board per detail C-1-3C.

- Duplex receptacle(s) on mounting plate below top per C-14-1. Refer to electrical plan.

Item No 156 - FIRE SUPPRESSION SYSTEM                                          1 Required

Ansul Fire Protection Model No. PIRANHA,  Furnish a complete wet chemical fire suppression system model Piranha as manufactured by “Ansul” or equal in compliance with U.L. 300 standards. The system shall include all permits and test as required by the authority having jurisdiction.

Automatic actuation shall be by means of fusible links mounted in the exhaust plenum of the ventilator with no visible conduit.

System shall be furnished and installed by an Ansul certified distributor in accordance with manufacturer's instructions and the authority having jurisdiction.

System to include a power interruption device, (PID) to insure the solenoid gas valve will stay open in the event of a momentary power loss.

(4) Microswitches shall be furnished as part of the fire protection system for tie in of building alarm and for fire/fuel shut down.

All access openings, holes, sleeves, chases, etc., in building structure necessary to permit piping and control tubing to be run between system	unit, ventilator and duct work are to be provided by the General Contractor.

The Building Alarm System Contractor is to furnish a control relay to detect operation of the system by connection to the microswitches supplied.

The Electrical Contractor is to furnish and install all wiring required for the system specified and include shunt trip devices required.

Notes:

(1)	All exposed piping and nozzles of fire protection system shall be chrome or Stainless steel sleeved.

(2)All horizontal piping is to be done on the top of the ventilator unless otherwise specified.

Refer to Gaylord preliminary shop drawings 06-0050, sheets 4 & 7 of 13.

Item No 157 - COFFEE MAKER                                                        1 Required

Fetco, Inc Model No. CBS-52H-15,  Coffee Maker.

- This item is not in contract. The specification is for reference only. The item is shown on the drawing for informational purposes and generally represents the size/ capacity of unit to be provided by Product Supplier/ Vendor.

- Any utility connections indicated on drawings are representative and intended to assist the Engineer in planning base building utility systems.

- Utility requirements for existing equipment or equipment provided by a product supplier will be confirmed by the FSE Contractor and represented on the dimensioned rough-in plans for use by the construction trades.

- Stainless steel brew baskets.

- Provide w/ 500 filter papers.

- Provide with half batch brew capability.

- F.S.E. Contractor to provide unit with Dormont Model SWIRL water connector hose, appropriately sized, with quick disconnect 2-way coupling per specific conditions.

- Connect to Item 67, Central Water Treatment System

- Tea brewing package.

- Drip tray accessory.

- Twelve gravity flow type thermos w/ sight gauges.

Item No 158 - REFRIGERATOR, 2-SECTION                                    1 Required

Traulsen & Company, Inc. Model No. AHT232WUTHHS,  Refrigerator, 2-Section.

- Furnish w/ cord & plug.

- Doors w/ cylinder locks, common keyed.

- Provide five (5) year compressor warranty and one (1) year refrigeration service contract.

- Half doors.

- Provide two additional chrome plated wire shelves.

Item No 159 - TABLE WITH OPEN BASE                                        1 Required

Custom fabrication. Unit to be size and shape as shown on plan x 38" high w/ 6" high splash per detail C-1-1A Type "A" edge, C-1-1B backsplash and C-7-1.  Rear and left end splashes as shown on plan.  Provide with the following:

- Open base table with rear and side cross bars to accommodate placement of Item 160, Utility Cart.

- As part of this Item provide Kason bumper guard approx. 12’ long mounted to wall at +24” AFF to protect wall finish from cart abuse.

Item No 160 - UTILITY CART                                                             6 Required

Metro Equipment Corporation Model No. BCE2030-34G,  Three shelf enclosed end unit.  Furnish per manufacturer's standard specifications.

- Confirm color w/ Architect.

Item No 161 - SHELVING UNIT                                                          1 Required

Metro Equipment Corporation Model No. Super Erecta Chrome Construction.   Size and shape as shown on plan x 4 tiers high. 74" high posts to match shelf type.

- Unit to be mobile mounted 5" diameter polyurethane swivel casters, two (2) swivel and two (2) with brakes.

Item No 162 - REFRIGERATOR, 2-SECTION                                    1 Required

Traulsen & Company, Inc. Model No. AHT232WUTHHS,  Refrigerator, 2-Section.

- Furnish w/ cord & plug.

- Doors w/ cylinder locks, common keyed.

- Provide five (5) year compressor warranty and one (1) year refrigeration service contract.

- Half doors.

- Provide two additional chrome plated wire shelves.

Item No. 163 - SPARE NUMBER -

Item No. 164 - SPARE NUMBER –

Item No 165 - FLOOR TROUGH                                                                  1 Required

IMC/Teddy Food Service Corp. Model No. SFT,  Unit of size and shape as shown on plan. This item to be set and installed by General Contractor. Quarry tile floor finish to be etched, if required, prior to setting food service equipment in place. Provide grating sections of equal size not to exceed 20" long. Provide shop drawings of size and scale outlined in specific conditions.

- Grating to be anti-slip, stainless steel.

- Shallow floor trough, 2" deep.

Item No 166 - ICE BIN & TRANSPORT SYSTEM                                       1 Required

Follett Corporation Model No. 1350-56,  Ice Bin & Transport System.

- Provide six accessory totes for each cart

- Provide plastic shovel

- Reinforced top required if ice makers weight exceed 1000 lbs.

Item No 167 - ICE MACHINE                                                                       2 Required

Hoshizaki Model No. KM-900MAH,  Ice Machine.

- Provide flexible water disconnects for installation by Plumbing Division

- This item to be connected to Item 67, central water filtration system by Division 15.

- Locate side by side on Item 166.

Item No 168 - ICE STORAGE BIN                                                              1 Required

Manitowoc Equipment Works, Inc. Model No. B-970,  Ice Storage Bin.

- Provide any adapter, mounting kits or ice deflectors required

- Stainless legs included in w/ B series

Item No 169 - ICE MAKER, FLAKER                                                         1 Required

Hoshizaki Model No. F-1001MAH Ice Maker, Flaker.

- Flake cube size

- Provide any bin mounting kits necessary

- Provide flexible water disconnects for installation by Plumbing Division

- This item to be connected to Item 67, central water filtration system by Division 15

Item No. 170 - SPARE NUMBER -

Item No. 171 - SPARE NUMBER -

Item No. 172 - SPARE NUMBER -

Item No 173 - BAG N' BOX RACK                                                              2 Required

By Vendor Model No. BAGNBOXRACK,  This item is not in contract. The item is shown on the drawing for informational purposes and generally represents the size/ capacity of unit to be provided by Product Supplier/ Vendor.

Item No 174 - CARBONATOR                                                                    1 Required

By Vendor Model No. CARBONATOR,  This item is not in contract. The item is shown on the drawing for informational purposes and generally represents the size/ capacity of unit to be provided by Product Supplier/ Vendor.

Any utility connections indicated on drawings are representative and intended to assist the Engineer in planning base building utility systems.

Actual utility requirements will be confirmed by the Foodservice Equipment Contractor and represented on the dimensioned rough-in plans for use by the construction trades.

The Foodservice Equipment Contractor will provide flexible utility disconnects for this item per the specific conditions section of the contract.

This item to be connected to Item 67, central water filtration system by Division 15

Item No 175 - SHELVING                                                                    LOT Required

Metro Equipment Corporation Model No. SUPER ERECTA,  Of size and shape as shown on plan x 5 tiers high.  74" high posts to match shelf type.

- 5" diameter polyurethane swivel casters, two (2) with brakes.

Item No. 176 - SPARE NUMBER -

Item No. 177 - SPARE NUMBER -

Item No 178 - OFFICE CHAIR                                                            LOT Required

This item is not in contract. It is specified by the Architect.

Item No 179 - WALK-IN COOLER                                                         1 Required

Kolpak Industries, Inc.  WALK-IN COOLER,  Unit to be a Refrigerator +35° F. of size and shape as shown on plan x  8' - 2 1/2" high. Wall and ceiling panels shall be 4" urethane, U.L.Flame Spread 25 insulation. Panels are without wooden structural members. Hinged entrance door(s) to be flush type, hinged as shown on plan. Thru ceiling door electrical assembly as shown on detail E-1-4. Provide one (1) lot removable closure panels with channels, the same finish as exterior panels to ceiling. (F.S.E. Contractor to verify ceiling height).  Closure to be louvered for top mounted condensing units (define) One (1) lot of trim strip, where required to close in wall gaps, of same finish as exterior panels to finished ceiling. (F.S.E. Contractor to verify height).

Options:

- Floorless with flat bottom wall panels anchored to slab on grade.. No insulation.  General Contractor to provide mud and tile including base tile at interior and exterior perimeter of fixture.

- Embossed aluminum white finish on exposed exterior.

- Stainless steel finish on the interior.

- Unexposed exterior finish to be galvalume.

- Hinged doors to be 36" wide x 84" high with third hinge

- Dead bolt lock per Specific Conditions (on exterior doors only)

- Aluminum diamond tread kick plate, 3'-0" high on exterior and interior of door and jamb

- 14" x 14" vision panel and automatic door closer, Kason K-1094.

- Factory cut door for tile floor conditions.

- Approved Lithonia fluorescent light fixtures and switches, installed and wired by the Electrical Division per detail E-1-4. (See electrical plan for quantity and location of lights and switches)

- Modularm 75B, recessed with thru ceiling door electrical as shown on detail E-1-4.

- Stainless steel corner guards at exposed corners.

- Wainscot on exposed exterior, 1/8" aluminum diamond plate.  Top to align with top of kickplate on door.  FSE Contractor to coordinate w/ General Contractor on height of base tile as required.

Refer to Kolpak preliminary shop drawing 65369.

Item No 180 - EVAPORATOR COIL, MEDIUM TEMPERATURE              1 Required

Kolpak Industries, Inc. Model No. AM-052,  Unit to be adequately sized to operate Item 179, Refrigerator Compartment @ +35° F.  Electrical hook-up and interconnecting of system to be by the Electrical Contractor.

Item No 181 - CONDENSING UNIT, AIR COOLED REMOTE                    1 Required

Kolpak Industries, Inc. Model No. PR100MW,  F.S.E.C. to provide the necessary refrigeration lines required to operate and maintain the refrigeration system for Item 179, refrigerated compartment.  Unit to be part of the remote refrigeration system for Item 180, Evaporator Coil.

F.S.E.C. to coordinate with the electrical/ plumbing contractor to provide a complete and operable refrigeration system.

Provide start-up, five (5) year compressor warranty and one (1) year refrigeration service contract.

Division 16 to wire to safety disconnect switch.

- Unit to be located on building roof.  See Architectural plan for location.

- Assume a 50' refrigeration line run.

Item No 182 - SHELVING                                                                   LOT Required

Metro Equipment Corporation Model No. SUPER ERECTA METRO SEAL,  Of size and shape as shown on plan x 5 tiers high.  74" high posts to match shelf type.

- 5" diameter polyurethane swivel casters, two (2) with brakes.

Item No 185 - BEVERAGE COUNTER                                                      1 Required

Beverage Counter.

- This item is not in contract. It is specified by the Architect.

Item No 186 - COFFEE MAKER                                                               1 Required

Fetco, Inc Model No. CBS-62H,  Coffee Maker.

- This item is not in contract. The specification is for reference only. The item is shown on the drawing for informational purposes and generally represents the size/ capacity of unit to be provided by Product Supplier/ Vendor.

- Any utility connections indicated on drawings are representative and intended to assist the Engineer in planning base building utility systems.

- Utility requirements for existing equipment or equipment provided by a product supplier will be confirmed by the FSE Contractor and represented on the dimensioned rough-in plans for use by the construction trades.

- Stainless steel brew baskets.

- Provide w/ 500 filter papers.

- Provide with half batch brew capability.

- F.S.E. Contractor to provide unit with Dormont Model SWIRL water connector hose, appropriately sized, with quick disconnect 2-way coupling per specific conditions.

- Connect to Item 67, Central Water Treatment System

- F.S.E. Contractor to confirm manufacturer/ model number of unit to be provided by Product Supplier and reflect utility requirements on dimensioned rough-in plans.

- Drip tray accessory.

- Six 3 gallon Luxus dispensers.

- Connect to filtered cold water connection from Item 67, Central Water Filter w/ Surge Tank.

Item No 187 - AIRPOT BREWER, TWIN                                                     1 Required

Fetco, Inc Model No. CBS-32AAP,  Airpot Brewer, Twin.

- This item is not in contract. The specification is for reference only. The item is shown on the drawing for informational purposes and generally represents the size/ capacity of unit to be provided by Product Supplier/ Vendor.

- Any utility connections indicated on drawings are representative and intended to assist the Engineer in planning base building utility systems.

- Utility requirements for existing equipment or equipment provided by a product supplier will be confirmed by the FSE Contractor and represented on the dimensioned rough-in plans for use by the construction trades.

- Provide w/ 500 filter papers.

- F.S.E. Contractor to provide unit with Dormont Model SWIRL water connector hose, appropriately sized, with quick disconnect 2-way coupling per specific conditions.

- Connect to Item 67, Central Water Treatment System

- Twelve airpot 3 liter, lever top, s/s lined.

- Gourmet (metal) brew basket in place of standard plastic.

- Dual portion controlled timer.

- Airpot rack 6 station.

- Round drip tray.

- Connect to filtered cold water connection from Item 67, Central Water Filter w/ Surge Tank.

Item No 188 - COFFEE SERVER                                                             2 Required

Fetco, Inc Model No. LS-15,  Coffee Server.

- This item is not in contract. The specification is for reference only. The item is shown on the drawing for informational purposes and generally represents the size/ capacity of unit to be provided by Product Supplier/ Vendor.

- Specified as part of Item 186.

Item No 189 - COFFEE, AIRPOT W/STAND/BASE                                  2 Required

Bloomfield Industries, Inc. Model No. 3013 W/(3)7759,  Coffee, Airpot W/Stand/Base.

- This item is not in contract. The specification is for reference only. The item is shown on the drawing for informational purposes and generally represents the size/ capacity of unit to be provided by Product Supplier/ Vendor.

Item No 190 - MILK DISPENSER                                                               1 Required

Silver King Model No. SK2-SERIES,  Milk Dispenser.

- This item is not in contract. The specification is for reference only. The item is shown on the drawing for informational purposes and generally represents the size/ capacity of unit to be provided by Product Supplier/ Vendor.

- Any utility connections indicated on drawings are representative and intended to assist the Engineer in planning base building utility systems.

- Actual utility requirements will be confirmed by the Foodservice Equipment Contractor and represented on the dimensioned rough-in plans for use by the construction trades.

Item No 191 - CAPPUCCINO DISPENSER                                              1 Required

Cecilware Corporation Model No. GB3M,  Cappuccino Dispenser.

- This item is not in contract. The specification is for reference only. The item is shown on the drawing for informational purposes and generally represents the size/ capacity of unit to be provided by Product Supplier/ Vendor.

- Any utility connections indicated on drawings are representative and intended to assist the Engineer in planning base building utility systems.

- Actual utility requirements will be confirmed by the Foodservice Equipment Contractor and represented on the dimensioned rough-in plans for use by the construction trades.

- Connect to filtered cold water connection from Item 67, Central Water Filter w/ Surge Tank.

Item No 192 - ICED TEA BREWER                                                          1 Required

LIPTON,  Iced Tea Brewer.

- This item is not in contract. The specification is for reference only. The item is shown on the drawing for informational purposes and generally represents the size/ capacity of unit to be provided by Product Supplier/ Vendor.

- Any utility connections indicated on drawings are representative and intended to assist the Engineer in planning base building utility systems.

- Actual utility requirements will be confirmed by the Foodservice Equipment Contractor and represented on the dimensioned rough-in plans for use by the construction trades.

- The Foodservice Equipment Contractor will provide flexible utility disconnects for this item per the specific conditions section of the contract.

- Connect to filtered cold water connection from Item 67, Central Water Filter w/ Surge Tank.

Item No 193 - SODA & ICE DISPENSER                                                  1 Required

Cornelius Model No. ENDURO 300,  Soda & Ice Dispenser.

- Foodservice Equipment Contractor to provide as part of this Item, one “swirl”-type quick water disconnect per specific conditions.

- This item is not in contract. The specification is for reference only. The item is shown on the drawing for informational purposes and generally represents the size/ capacity of unit to be provided by Product Supplier/ Vendor.

- Any utility connections indicated on drawings are representative and intended to assist the Engineer in planning base building utility systems.

- Actual utility requirements will be confirmed by the Foodservice Equipment Contractor and represented on the dimensioned rough-in plans for use by the construction trades.

Item No. 194 - SPARE NUMBER -

Item No. 195 – ACESS PANEL –

This item is not in contract.  It is specified by the Architect.

The purpose of the access panel is to enable service service access to gas connections on Item 198, Pizza Oven.

Item No 196 - MOBILE RACK, U/C HEIGHT                                         1 Required

United Epco Model No. K1826-5ST,  Mobile Rack, U/C Height.

- Corner bumpers.

- Casters w/ polyurethane wheels.

- Angles on 1 ½” centers (for pizza).

Item No 197 - REFRIGERATED PREP TABLE                                     1 Required

Kairak Innovations Model No. KRP-65S,  Refrigerated Prep Table.

- Provide 1/2 “ thick Richlite cutting board in lieu of standard.

- Provide on 4" casters.

- One compartment removable tray slides with spacing at 2 1/2".

- Door locks.

- Telescoping night cover.

- Compressor housing on left.

Item No 198 - PIZZA OVEN                            	1 Required

Marsal And Sons, Inc. Model No. MB60 STACKED,  Pizza Oven.

- Finishing kit including limestone tile.  Kit to include tile for exposed sides.

- Provide w/ flat top.

Item No 199 - EXHAUST HOOD                                                            1 Required

Gaylord Industries, Incorporated Model No. GX2-BDL-CA-66. Unit to be U.L. listed grease extractor, high velocity island canopy of size and shape as shown on plan x  30" high, built in one section.

One (1) lot stainless steel enclosure paneling, at all open sides, to finished ceiling (F.S.E. Contractor to verify height). Paneling must be supplied by Exhaust Hood Manufacturer and installed by F.S.E. Contractor. Adhere to Specific Conditions for installation of fabricated equipment.

Ventilator to be provided w/ full length fire damper located in the ventilator extractor plenum between the extractor inserts and the ventilator duct collar.  The damper shall be controlled by an electrically driven actuator specifically listed for such use and shall be activated by fail safe thermostats located at the duct collar.  The damper shall close each time the exhaust fan is turned off, to prevent conditioned air from escaping the occupied space via thermal drafts and then re-open when the exhaust fan is started.

Ventilator to be U.L. and NSF listed and be in full accordance with NFPA 96 and all state/local codes/ordinances applicable.

F.S.E. Contractor to supply the necessary mounting rods for hanging ventilator.

Supply and/ or exhaust air requirements as shown on plans.

Options:

- Recessed incandescent light fixtures as indicated on electrical plan.

- Pre-piping of the Item No. 156, Fire Protection System is to be provided by the Exhaust Hood manufacturer at the time of hood fabrication.

- Pre-piping to include fusible-link detector bracket assemblies and cable runs, and wet-chemical manifold piping, duct collar and plenum protection piping with termination nozzles, and cooking appliance nozzle drops with termination nozzles.

- Location of cooking appliance nozzle drops are to be fully dimensioned on hood submission drawings.

- Fusible link detector and wet chemical piping terminations to be coordinated with the FSE Contractor and clearly indicated on hood submission drawings.

- Access to the fusible links for inspection and change-outs to be provided.  The hood manufacturer and FSE Contractor to coordinate their efforts to assure the necessary access.

- Supply air plenum boxes w/ full length internal removable baffle, mounted at finished ceiling height.

FSE Contractor is responsible for fit of equipment. Prior to releasing hoods for fabrication, FSE Contractor needs to verify field conditions (existing &/or proposed) and to determine clearances to all structural items, obstructions, etc. The FSE Contractor will coordinate with other trades to confirm that the hood can be mounted as proposed and that the ductwork and final connection can be accommodated without conflict.  Failure to perform this step may result in modifications to the exhaust hood at the FSE Contractor's expense.

Refer to Gaylord preliminary shop drawing 06-0050 sheets 8 & 10 of 13.

Item No 200 - FOOD SHIELD                                                                   1 Required

Brass Smith Inc Model No. Z9915,  Finish to be brushed aluminum.  . Of size and shape as shown on plan.

- Undercounter mount, millwork counter MWU5 heavy duty flange/inverted

- 3/8” tempered glass (standard for shelves) max. 66" long.

- Lights linear halogen.  Wire lights to remote switch in counter apron. Coordinate w/ Millwork Contractor.

- Stealth warmer brushed aluminum finish

- Switch option: remote infinite (standard).  Switch mounted in counter apron.  Coordinate w/ Millwork Contractor.

- Wiring exit point option: right side (customer view)

- Provide shop drawings of size and scale outlined in specific conditions

Note:  Lights to be provided w/ concealed wiring of sufficient length through protector case support posts to remote control located below counter.

Item No 201 - HEAT LAMP                                                                    3 Required

Merco Savory Model No. 500-R-T,  Heat Lamp.

- Shade number to be selected by Architect.

- Retractable cord mount, black.

- Trac heat connection color to be selected by Architect.

- Confirm shade color w/ Architect

- Bulb color: white

- Confirm mounting height.

- Counter height to be at 34" AFF

Item No 202 - FOOD WARMER, DROP IN                                             1 Required

Wells Manufacturing Company Model No. MOD-400TDM,  Food Warmer, Drop In.

- Plumbing Division to provide gate valve on indirect waste.

- Ship unit to fabricator for installation in counter, Item 215.

- Provide Type 6 single pantry faucet.

- Drain Screen

- Wellslok Extension Kit, for UL listed 12" x 20" warmers only, to adapt wood countertop up to 1-1/2" thick (1 well requires one Kit, 2 to 3 well units require two kits, 4 to 5 well units require three kits)

- 72" Wiring (thermostatically controlled warmers) per well.

Item No 203 - FOOD SHIELD                                                                  1 Required

Brass Smith Inc Model No. Z9915,  Finish to be brushed aluminum.  Of size and shape as shown on plan.

- Undercounter mount, millwork counter MWU5 heavy duty flange/inverted.

- 3/8” tempered glass (standard for shelves) max. 66" long.

- Lights linear halogen.  Wire lights to remote switch in counter apron. Coordinate w/ Millwork Contractor.

- Stealth warmer brushed aluminum finish

- Switch option: remote infinite (standard).  Switch mounted in counter apron.  Coordinate w/ Millwork Contractor.

- Wiring exit point option: right side (customer view)

- Provide shop drawings of size and scale outlined in specific conditions

Note:  Lights to be provided w/ concealed wiring of sufficient length through protector case support posts to remote control located below counter.

Item No 204 - COUNTER WITH SINK                                                      1 Required

Custom fabrication. Unit to be of size and shape as shown on plan x 36” high w/ 6” splash per detail C-1-1A Type “A” edge, C-1-1B backsplash and C-2-1. Provide with the following:

- 20" x 20" x 10" deep sink.

- Type 5 faucet.

- Equal double pan doors, hinged per plan and constructed per detail C-2-2.

- Padlock hasp(s) similar to detail C-1-4.

- Fixed intermediate shelf behind each door.

- Removable stainless steel kickplates.

Item No 205 - WASTE CONTAINER                                                          1 Required

By Owner Model No. SLIM JIM,  Waste Container.

- This item is not in contract. The specification is for reference only. The item is shown on the drawing for informational purposes and generally represents the size/ capacity of unit to be provided by Product Supplier/ Vendor.

Item No. 206 - SPARE NUMBER -

Item No. 207 - SPARE NUMBER -

Item No 208 - ROTISSERIE OVEN, COUNTERTOP                                   1 Required

Hickory Industries, Inc. Model No. 5.5E,  Rotisserie Oven, Countertop.

- Provide w/ cord & plug.

- Standard spits and skewers.

- Rotating basket spits.

Item No. 209 - SPARE NUMBER -

Item No 210 - HOT DELI/DISPLAY SYSTEM                                             1 Required

Alto-Shaam, Incorporated Model No. EU2SYS-72,  Hot Deli/Display System.

- Two 120/208 volt, single phase connections.

- The display system to include one (1) model ED-72 Display Case mounted directly on top of a BU-72 Base.  The base includes two (2) Model CH-72/DM Cooking and Holding Ovens.

- Decorator panels finish to be selected by Architect.

- Provide optional mirropane doors and end panels.

- Cutting board.

Item No 211 - FOOD WARMER, DROP IN, 5 WELL                                 1 Required

Wells Manufacturing Company Model No. MOD-500TDM,  Food Warmer, Drop In, 5 Well.

- Plumbing Division to provide gate valve on indirect waste.

- Ship unit to fabricator for installation in counter, Item 215.

- Provide type 6 single pantry faucet.

- Drain Screen

- Wellslok Extension Kit, for UL listed 12" x 20" warmers only, to adapt wood countertop up to 1-1/2" thick (1 well requires one Kit, 2 to 3 well units require two kits, 4 to 5 well units require three kits)

- 72" Wiring (thermostatically controlled warmers) per well.

Item No 212 - FOOD SHIELD                                                                     1 Required

Brass Smith Inc Model No. Z9915,  Finish to be brushed aluminum.  Of size and shape as shown on plan.

- Undercounter mount, millwork counter MWU5 heavy duty flange/inverted

- 3/8” tempered glass (standard for shelves) max. 66" long.

- Lights linear halogen.  Wire lights to remote switch in counter apron. Coordinate w/ Millwork Contractor.

- Stealth warmer brushed aluminum finish

- Switch option: remote infinite (standard)

- Wiring exit point option: right side (customer view).

- Provide shop drawings of size and scale outlined in specific conditions

Note:  Lights to be provided w/ concealed wiring of sufficient length through protector case support posts to remote control located below counter.

Item No 213 - HEAT LAMP                                                                      3 Required

Merco Savory Model No. 500-R-T,  Heat Lamp.

- Plug in style.

- Shade number to be selected by Architect.

- Retractable cord mount, black.

- Trac heat connection color to be selected by Architect.

- Confirm shade color w/ Architect

- Bulb color: white

- Confirm mounting height.

- Counter height to be at 34" AFF

Item No 214 - COUNTER WITH SINK                                                         1 Required

Custom fabrication. Unit to be of size and shape as shown on plan x 36” high w/ 6” splash per detail C-1-1A Type “A” edge, C-1-1B backsplash and C-2-1. Provide with the following:

- 20" x 20" x 10" deep sink.

- Type 5 faucet.

- Equal double pan doors, hinged per plan and constructed per detail C-2-2.

- Padlock hasp(s) similar to detail C-1-4.

- Fixed intermediate shelf behind each door.

- Removable stainless steel kickplates.

- Compartment open to floor for Slim Jim waste receptacle w/ hinged doors similar to detail C-2-2. Height of doors w/integral toe base to be approximately 27" and clear floor by ½". Provide hinged flap at top of door to underside of counter top for trash access.

- Accommodate undercounter placement of Item 87 where shown on plan.

Item No 215 - SERVICE COUNTER                                                           1 Required

Service Counter.

- This item is not in contract. It is specified by the Architect.

Item No 216 - FIRE SUPPRESSION SYSTEM                                           1 Required

Ansul Fire Protection Model No. PIRANHA,  Furnish a complete wet chemical fire suppression system model Piranha as manufactured by “Ansul” or equal in compliance with U.L. 300 standards. The system shall include all permits and test as required by the authority having jurisdiction.

Automatic actuation shall be by means of fusible links mounted in the exhaust plenum of the ventilator with no visible conduit.

System shall be furnished and installed by an Ansul certified distributor in accordance with manufacturer's instructions and the authority having jurisdiction.

System to include a power interruption device, (PID) to insure the solenoid gas valve will stay open in the event of a momentary power loss.

(4) Microswitches shall be furnished as part of the fire protection system for tie in of building alarm and for fire/fuel shut down.

All access openings, holes, sleeves, chases, etc., in building structure necessary to permit piping and control tubing to be run between system 	unit, ventilator and duct work are to be provided by the

General Contractor.

The Building Alarm System Contractor is to furnish a control 	relay to detect operation of the system by

connection to the microswitches supplied.

The Electrical Contractor is to furnish and install all wiring required for the system specified and include shunt trip devices required.

Notes:

(1)	All exposed piping and nozzles of fire protection system shall be chrome or Stainless steel sleeved.

(2)	All horizontal piping is to be done on the top of the ventilator unless otherwise specified.

Refer to Gaylord preliminary shop drawings 06-0050, sheets 8 & 10 of 13.

Item No. 217 - SPARE NUMBER –

Item No 218 - HOLDING CABINET, FULL HEIGHT                                 2 Required

Winston Industries Model No. HA4022-HR-5,  Holding Cabinet, Full Height.

- Without windows.

- Full perimeter bumpers w/ 5" casters.

Item No. 219 - SPARE NUMBER -

Item No. 220 - SPARE NUMBER -

Item No 221 - SERVICE COUNTER                                                           1 Required

Service Counter.

- This item is not in contract. It is specified by the Architect.

Item No 222 - SOFT-SERVE FREEZER,FLOOR MODEL                          1 Required

Soft-Serve Freezer, floor model.

- This item is not in contract.  It is “By Owner”

- Any utility connections indicated on drawings are representative and intended to assist the Engineer in planning base building utility systems.

- Utility requirements for existing equipment or equipment provided by a product supplier will be confirmed by the FSE Contractor and represented on the dimensioned rough-in plans for use by the construction trades.

Item No 223 - ICE PAN                                                                                1 Required

Atlas Metal Ind. Model No. WIC-1,  Ice Pan.

- Adapter bars for sixth size pans.

Item No 224 - DESSERT COUNTER                                                           1 Required

Service Counter.

- This item is not in contract. It is specified by the Architect.

Item No 225 - FOOD SHIELD                                                                      1 Required

Brass Smith Inc Model No. Z9915,  Finish to be brushed aluminum. Of size and shape as shown on plan.

- Undercounter mount, millwork counter MWU5 heavy duty flange/inverted

- 3/8” tempered glass (standard for shelves) max. 66" long.

- Lights linear halogen. Wire lights to remote switch in counter apron. Coordinate w/ Millwork Contractor.

- Wiring exit point option: left side (customer view).

- Provide shop drawings of size and scale outlined in specific conditions

Note:  Lights to be provided w/ concealed wiring of sufficient length through protector case support posts to remote control located below counter.

Item No 226 - REFRIGERATED MERCHANDISER                                    1 Required

This item is not in contract. The specification is for reference only. The item is shown on the drawing for informational purposes and generally represents the size/ capacity of unit to be provided by Product Supplier/ Vendor.

Utility connections indicated on drawings are representative and intended to assist the Engineer in planning base building utility systems.

Actual utility requirements will be confirmed by the Foodservice Equipment Contractor and represented on the dimensioned rough-in plans for use by construction trades.

True Food Service Equipment, Inc Model No. GDM-26,  Refrigerated Merchandiser.

- Gravity-feed True Trac shelving

- Sign panels

Item No 227 - REFRIG. MERCHANDISER, GLASS DOOR                       2 Required

This item is not in contract. The specification is for reference only. The item is shown on the drawing for informational purposes and generally represents the size/ capacity of unit to be provided by Product Supplier/ Vendor.

Utility connections indicated on drawings are representative and intended to assist the Engineer in planning base building utility systems.

Actual utility requirements will be confirmed by the Foodservice Equipment Contractor and represented on the dimensioned rough-in plans for use by construction trades.

True Food Service Equipment, Inc Model No. GDM-35,  Refrig. Merchandiser, Glass Door.

Item No 228 - AIR SCREEN REFRIGERATOR                                         1 Required

Barker Company Model No. S95-24TM,  Air Screen Refrigerator.

- Stainless interior and exterior finish.

- Ends to be solid.

- Roller shutter w/ lock.

- Light below each shelf.

- Five stainless steel shelves.

- Provide five (5) year compressor warranty and one (1) year refrigeration service contract

- Remote refrigeration specified as Item 239.

- Unit to be connected to Item 239, Condensing Unit, Remote.  Manufacturer to provide fixture complete with built - in defrost time clock, solenoid valve thermostat and expansion valve.  Food Service Equipment Contractor to provide complete and operable system.

- Ambient conditions in serving area  not to exceed 86 degrees F. or 55% relative humidity.

Item No 229 - COUNTER                                                                           2 Required

This item is not in contract. It is specified by the Architect.

Item No 230 - MERCHANDISER, ICE CREAM                                         1 Required

Beverage Air Model No. CF5-1W-G,  Merchandiser, Ice Cream.

Item No 231 - SERVICE COUNTER                                                          1 Required

This item is not in contract. It is specified by the Architect.

Item No 232 - COUNTER W/SINK & HAND SINK                                    1 Required

Custom fabrication. Unit to be of size and shape as shown on plan x 36” high w/ 6” splash per detail C-1-1A Type “A” edge, C-1-1B backsplash and C-2-1. Right & left end splashes as shown on plan.  Provide with the following:

- 20" x 20" x 10" deep sink.

- Type 5 faucet.

- Equal double pan doors, hinged per plan and constructed per detail C-2-2.

- Padlock hasp(s) similar to detail C-1-4.

- Fixed intermediate shelf behind each door.

- Removable stainless steel kickplates.

- 9" x 12" x 6" deep hand sink per detail C-2-1E. Provide w/type 5B faucet.

- Fabricator to provide an integral side splash, 6" high x 1" thick x depth of counter top, to isolate hand sink from counter similar to C-2-1E. Soap & towel dispenser by Owner.

- Open base to floor to accommodate Item 243.

- Compartment open to floor for “Slim Jim” waste receptacle w/ hinged doors similar to detail C-2-2. Height of doors w/integral toe base to be approximately 27" and clear floor by 1/2”. Provide open front at top of door to underside of counter top for trash access.

Item No 233 - SLICER, FOOD                                                                  1 Required

Hobart Corporation Model No. 2612, Slicer, Food.

- Furnish per manufacturer's standard specifications.

Item No 234 - HOLDING CABINET, UNDERCOUNTER                             1 Required

Winston Industries Model No. HC4009-HR-1,  Holding Cabinet, Undercounter.

- Doors hinged left.

- Caster size: 2"

- Fit below Item 232, Counter.

Item No 235 - FOOD WARMER, DROP IN                                                 1 Required

Wells Manufacturing Company Model No. MOD-100TD,  Food Warmer, Drop In.

- Plumbing Division to provide gate valve on indirect waste.

- Ship unit to fabricator for installation in counter, Item 229.

- Adapter Top, to convert 12" x 20" round corner warmer to hold one 7 qt. & one 11 qt. insert

- Adapter Top, to convert 12" x 20" warmers to hold two 7 quart insets

- Provide type 6 single pantry faucet.

- Drain Screen

- Wellslok Extension Kit, for UL listed 12" x 20" warmers only, to adapt wood countertop up to 1-1/2" thick (1 well requires one Kit, 2 to 3 well units require two kits, 4 to 5 well units require three kits)

- 72" Wiring (thermostatically controlled warmers) per well

Item No 236 - REFRIGERATED PREP TABLE                                         1 Required

Kairak Innovations Model No. KRP-48S,  Refrigerated Prep Table.

- Stainless steel rear where exposed.

- Hinge doors per plan drawing.

- Provide 1/2 “ thick Richlite cutting board in lieu of standard.

- Provide on 4" casters.

- Removable tray slides with spacing at 2 1/2".

- Door locks.

- Telescoping night cover.

- Compressor housing on right.

Item No 237 - FOOD SHIELD                                                                    1 Required

Brass Smith Inc Model No. Z9915,  Finish to be brushed aluminum.  Of size and shape as shown on plan.

- Undercounter mount, millwork counter MWU5 heavy duty flange/inverted

- 3/8” tempered glass (standard for shelves) max. 66" long.

- Lights linear halogen. . Wire lights to remote switch in counter apron. Coordinate w/ Millwork Contractor.

- Wiring exit point option: left side (customer view).

- Provide shop drawings of size and scale outlined in specific conditions

Note:  Lights to be provided w/ concealed wiring of sufficient length through protector case support posts to remote control located below counter.

Item No 238 - FOOD SHIELD, COMBINATION                                       1 Required

Brass Smith Inc Model No. Z9500,  Finish to be brushed aluminum. Of size and shape as shown on plan.

- Undercounter mount, millwork counter MWU5 heavy duty flange/inverted

- 3/8” tempered glass (standard for shelves) max. 66" long.

- Provide shop drawings of size and scale outlined in specific conditions

Item No. 239 –PANELBOARD

This item is not in contract.  It is in the millwork contract.

Item No 240 - CLAM SHELL GRILL                                                          2 Required

SODIR, Equipex, Ltd. Model No. MAJESTIC,  Clam Shell Grill.

- Confirm platen types w/ Owner.

Item No 241 - REFRIGERATED FOOD PREP TABLE                              1 Required

Kairak Innovations Model No. KRP-72S,  Refrigerated Food Prep Table.

- Stainless steel rear where exposed.

- Hinge doors per plan drawing.

- Provide 1/2 “ thick Richlite cutting board in lieu of standard.

- Provide on 4" casters.

- Removable tray slides with spacing at 2 1/2".

- Door locks.

- Telescoping night cover.

- Compressor housing on right.

Item No 242 - FOOD SHIELD                                                                    1 Required

Brass Smith Inc Model No. Z9915,  Finish to be brushed aluminum.  Of size and shape as shown on plan.

- Undercounter mount, millwork counter MWU5 heavy duty flange/inverted

- 3/8” tempered glass (standard for shelves) max. 66" long.

- Lights linear halogen. . Wire lights to remote switch in counter apron. Coordinate w/ Millwork Contractor.

- Wiring exit point option: left side (customer view).

- Provide shop drawings of size and scale outlined in specific conditions

Note:  Lights to be provided w/ concealed wiring of sufficient length through protector case support posts to remote control located below counter.

Item No 243 - REFRIGERATOR, UNDERCOUNTER                                 1 Required

Kairak Innovations Model No. KRUC-96S,  Refrigerator, Undercounter.

- Hinge doors per plan drawing.

- Provide five (5) year compressor warranty and one (1) year refrigeration service contract.

- Provide on 4” casters.

- Removable tray slides.

- Door locks.

Item No 244 - TOASTER, CONVEYOR TYPE                                           1 Required

Merco Savory Model No. RT-2VSE,  Toaster, Conveyor Type.

- Loading ramp.

Item No 245 - REFRIGERATED FOOD PREP TABLE                              1 Required

Kairak Innovations Model No. KRP-72S,  Refrigerated Food Prep Table.

- Stainless steel rear where exposed.

- Hinge doors per plan drawing.

- Provide 1/2 “ thick Richlite cutting board in lieu of standard.

- Provide on 4" casters.

- Removable tray slides with spacing at 2 1/2".

- Door locks.

- Telescoping night cover.

- Compressor housing on right.

Item No 246 - FOOD SHIELD                                                                     1 Required

Brass Smith Inc Model No. Z9915,  Finish to be brushed aluminum.  Of size and shape as shown on plan.

- Undercounter mount, millwork counter MWU5 heavy duty flange/inverted

- 3/8” tempered glass (standard for shelves) max. 66" long.

- Lights linear halogen. . Wire lights to remote switch in counter apron. Coordinate w/ Millwork Contractor.

- Wiring exit point option: left side (customer view).

- Provide shop drawings of size and scale outlined in specific conditions

Note:  Lights to be provided w/ concealed wiring of sufficient length through protector case support posts to remote control located below counter.

Item No. 247 - SPARE NUMBER -

Item No. 248 –PANELBOARD

This item is not in contract.  It is in the millwork contract.

Item No. 249 PANELBOARD

This item is not in contract.  It is in the millwork contract.

Item No 250 - COUNTER WITH SINK                                                       1 Required

Custom fabrication. Unit to be of size and shape as shown on plan x 36” high w/ 6” splash per detail C-1-1A Type “A” edge, C-1-1B backsplash and C-2-1. End splashes at left side as shown on plan.  Provide with the following:

- 20" x 20" x 10" deep sink.

- Type 5 faucet.

- Equal double pan doors, hinged per plan and constructed per detail C-2-2.

- Padlock hasp(s) similar to detail C-1-4.

- Fixed intermediate shelf behind each door.

- Removable stainless steel kickplates.

Item No 251 - RANGE, 24-INCH GAS W/OVEN                                  1 Required

Garland Commercial Ind., Inc. Model No. ST28,  Range, 24-Inch Gas W/Oven.

- Stainless steel both sides

- End caps and cover.

- Swivel casters (4) w/ front brakes.

- 10" high stainless steel backguard.

- Flexible gas disconnect per specific conditions.

- Polyurethane non-marking swivel casters (4) w/ front brakes.

Item No 252 - FRYER DRAIN CABINET                                                1 Required

Keating of Chicago, Inc. Model No. 14-007686,  Fryer Drain Cabinet.

- Stainless steel cabinet.

- Standard height riser.

- Casters.

- Pan slides.

- Provide "Keep Krisp" overhead mounted food warmer (2LCF). Provide with cord and plug.

- Provide cabinet with recessed dump station with stainless steel perforated screen.

- Battery to Item 253.

Item No 253 - FRYER, DEEP FAT GAS                                                1 Required

Keating of Chicago, Inc. Model No. 14AA,  Fryer, Deep Fat Gas.

- Full size baskets.

- Standard height riser.

- Swivel casters, front to be locking.

- Flexible gas disconnects per specific conditions.

- Battery to Item 252.

Item No 254 - EXHAUST HOOD, WALL MOUNTED                           1 Required

Gaylord Industries, Incorporated Model No. GX2-BDL-CA-48. Unit to be U.L. listed grease extractor, high velocity island canopy of size and shape as shown on plan x  18" high w/ tapered ends, built in one section.

One (1) lot stainless steel enclosure paneling, at all open sides, to finished ceiling (F.S.E. Contractor to verify height). Paneling must be supplied by Exhaust Hood Manufacturer and installed by F.S.E. Contractor. Adhere to Specific Conditions for installation of fabricated equipment.

Ventilator to be provided w/ full length fire damper located in the ventilator extractor plenum between the extractor inserts and the ventilator duct collar.  The damper shall be controlled by an electrically driven actuator specifically listed for such use and shall be activated by fail safe thermostats located at the duct collar.  The damper shall close each time the exhaust fan is turned off, to prevent conditioned air from escaping the occupied space via thermal drafts and then re-open when the exhaust fan is started.

Ventilator to be U.L. and NSF listed and be in full accordance with NFPA 96 and all state/local codes/ordinances applicable.

F.S.E. Contractor to supply the necessary mounting rods for hanging ventilator.

Supply and/ or exhaust air requirements as shown on plans.

Options:

- Recessed incandescent light fixtures as indicated on electrical plan.

- Pre-piping of the Item No. 156, Fire Protection System is to be provided by the Exhaust Hood manufacturer at the time of hood fabrication.

- Pre-piping to include fusible-link detector bracket assemblies and cable runs, and wet-chemical manifold piping, duct collar and plenum protection piping with termination nozzles, and cooking appliance nozzle drops with termination nozzles.

- Location of cooking appliance nozzle drops are to be fully dimensioned on hood submission drawings.

- Fusible link detector and wet chemical piping terminations to be coordinated with the FSE Contractor and clearly indicated on hood submission drawings.

- Access to the fusible links for inspection and change-outs to be provided.  The hood manufacturer and FSE Contractor to coordinate their efforts to assure the necessary access.

- Supply air plenum boxes w/ full length internal removable baffle, mounted at finished ceiling height.

FSE Contractor is responsible for fit of equipment. Prior to releasing hoods for fabrication, FSE Contractor needs to verify field conditions (existing &/or proposed) and to determine clearances to all structural items, obstructions, etc. The FSE Contractor will coordinate with other trades to confirm that the hood can be mounted as proposed and that the ductwork and final connection can be accommodated without conflict.  Failure to perform this step may result in modifications to the exhaust hood at the FSE Contractor's expense.

Refer to Gaylord preliminary shop drawing 06-0050 sheets 11 & 13 of 13.

Item No 255 - EXHAUST HOOD, ISLAND                                          1 Required

Gaylord Industries, Incorporated Model No. GX22-BDL-BBC-CL-CA-72  Unit to be U.L. listed grease extractor, high velocity island canopy of size and shape as shown on plan x  30" high, built in one section

One (1) lot stainless steel enclosure paneling, at all open sides, to finished ceiling (F.S.E. Contractor to verify height). Paneling must be supplied by Exhaust Hood Manufacturer and installed by F.S.E. Contractor. Adhere to Specific Conditions for installation of fabricated equipment.

Ventilator to be provided w/ full length fire damper located in the ventilator extractor plenum between the extractor inserts and the ventilator duct collar.  The damper shall be controlled by an electrically driven actuator specifically listed for such use and shall be activated by fail safe thermostats located at the duct collar.  The damper shall close each time the exhaust fan is turned off, to prevent conditioned air from escaping the occupied space via thermal drafts and then re-open when the exhaust fan is started.

Ventilator to be U.L. and NSF listed and be in full accordance with NFPA 96 and all state/local codes/ordinances applicable.

F.S.E. Contractor to supply the necessary mounting rods for hanging ventilator.

Supply and/ or exhaust air requirements as shown on plans.

Options:

- Recessed incandescent light fixtures as indicated on electrical plan.

- Pre-piping of the Item No. 156, Fire Protection System is to be provided by the Exhaust Hood manufacturer at the time of hood fabrication.

- Pre-piping to include fusible-link detector bracket assemblies and cable runs, and wet-chemical manifold piping, duct collar and plenum protection piping with termination nozzles, and cooking appliance nozzle drops with termination nozzles.

- Location of cooking appliance nozzle drops are to be fully dimensioned on hood submission drawings.

- Fusible link detector and wet chemical piping terminations to be coordinated with the FSE Contractor and clearly indicated on hood submission drawings.

- Access to the fusible links for inspection and change-outs to be provided.  The hood manufacturer and FSE Contractor to coordinate their efforts to assure the necessary access.

- Supply air plenum boxes w/ full length internal removable baffle, mounted at finished ceiling height.

FSE Contractor is responsible for fit of equipment. Prior to releasing hoods for fabrication, FSE Contractor needs to verify field conditions (existing &/or proposed) and to determine clearances to all structural items, obstructions, etc. The FSE Contractor will coordinate with other trades to confirm that the hood can be mounted as proposed and that the ductwork and final connection can be accommodated without conflict.  Failure to perform this step may result in modifications to the exhaust hood at the FSE Contractor's expense.

Refer to Gaylord preliminary shop drawing 06-0050 sheets 12 & 13 of 13.

Item No 256 - REFRIGERATED EQUIPMENT STAND                              1 Required

Kairak Innovations Model No. KRES-72S,  Refrigerated Equipment Stand.

- Provide five (5) year compressor warranty and one (1) year refrigeration service contract.

- Provide on 4” casters.

Item No 257 - GRIDDLE, COUNTERTOP, GAS                                       1 Required

Garland Commercial Ind., Inc. Model No. GXT3,  Griddle, Countertop, Gas.

- Natural gas.

- Flexible gas disconnect w/ restraining device per specific conditions

- Provide cutting board and support bracket per detail C-2-3B.

- 4" stainless steel legs w/ adjustable feet.

- Provide stainless steel clips, if required, to hold legs on stand at right end.

Item No 258 - BROILER, COUNTERTOP, GAS                                     1 Required

Garland Commercial Ind., Inc. Model No. GD-36RB,  Natural Gas

- Stainless Steel Sides and Back

- Broiler Splatter Shield

- Provide stainless steel clips, if required, to hold legs on stand at left end.

- Provide cutting board and support bracket per detail C-2-3B.

Item No 259 - FOOD SHIELD                                                                  1 Required

Brass Smith Inc Model No. Z9915,  Finish to be brushed aluminum.  Of size and shape as shown on plan.

- Undercounter mount, millwork counter MWU5 heavy duty flange/inverted

- 3/8” tempered glass (standard for shelves) max. 66" long.

- Lights linear halogen. Wire lights to remote switch in counter apron. Coordinate w/ Millwork Contractor.

- Stealth warmer brushed aluminum finish

- Switch option: remote infinite (standard). Switch mounted in counter apron.  Coordinate w/ Millwork Contractor.

- Wiring exit point option: right side (customer view).

- Provide shop drawings of size and scale outlined in specific conditions.

Note:  Lights to be provided w/ concealed wiring of sufficient length through protector case support posts to remote control located below counter.

Item No 260 - HOT FOOD WELL, DROP IN, 2-WELL                            1 Required

Wells Manufacturing Company Model No. MOD-200TDM,  Hot Food Well, Drop In, 2-Well.

- Plumbing Division to provide gate valve on indirect waste.

- Ship unit to fabricator for installation in counter, Item 215.

- Provide type 6 single pantry faucet.

- Drain Screen

- Wellslok Extension Kit, for UL listed 12" x 20" warmers only, to adapt wood countertop up to 1-1/2" thick (1 well requires one Kit, 2 to 3 well units require two kits, 4 to 5 well units require three kits)

- 72" Wiring (thermostatically controlled warmers) per well.

Item No 261 - FIRE SUPPRESSION SYSTEM                                       1 Required

Ansul Fire Protection Model No. PIRANHA,  Furnish a complete wet chemical fire suppression system model Piranha as manufactured by “Ansul” or equal in compliance with U.L. 300 standards. The system shall include all permits and test as required by the authority having jurisdiction.

Automatic actuation shall be by means of fusible links mounted in the exhaust plenum of the ventilator with no visible conduit.

System shall be furnished and installed by an Ansul certified distributor in accordance with manufacturer's instructions and the authority having jurisdiction.

System to include a power interruption device, (PID) to insure the solenoid gas valve will stay open in the event of a momentary power loss.

(4) Microswitches shall be furnished as part of the fire protection system for tie in of building alarm and for fire/fuel shut down.

All access openings, holes, sleeves, chases, etc., in building structure necessary to permit piping and control tubing to be run between system 	unit, ventilator and duct work are to be provided by the

General Contractor.

The Building Alarm System Contractor is to furnish a control 	relay to detect operation of the system by

connection to the microswitches supplied.

The Electrical Contractor is to furnish and install all wiring required for the system specified and include shunt trip devices required.

Notes:

(1)	All exposed piping and nozzles of fire protection system shall be chrome or Stainless steel sleeved.

(2)All horizontal piping is to be done on the top of the ventilator unless otherwise specified.

Refer to Gaylord preliminary shop drawings 06-0050, sheet 13 of 13.

Item No 262 - FOOD SHIELD, COMBINATION                                        1 Required

Brass Smith Inc Model No. Z9500,  Finish to be brushed aluminum Provide shop drawings of size and scale outlined in specific conditions. Of size and shape as shown on plan.

- Undercounter mount, millwork counter MWU5 heavy duty flange/inverted

- 3/8” tempered glass (standard for shelves) max. 66" long.

Item No 263 - FREEZER, UNDERCOUNTER                                           1 Required

Kairak Innovations Model No. KRUCF-48S,  Door hinged right. Provide five (5) year compressor warranty and one (1) year refrigeration service contract.

- Provide on 3 1/2” casters (recommended for undercounter).

- Door locks.

Item No 264 - COUNTER W/SINK & HAND SINK                                     1 Required

Custom fabrication. Unit to be of size and shape as shown on plan x 36” high w/ 6” splash per detail C-1-1A Type “A” edge, C-1-1B backsplash and C-2-1. Right end splash as shown on plan.  Provide with the following:

- 20" x 20" x 10" deep sink.

- Type 5 faucet.

- Equal double pan doors, hinged per plan and constructed per detail C-2-2.

- Padlock hasp(s) similar to detail C-1-4.

- Fixed intermediate shelf behind each door.

- Removable stainless steel kickplates.

- 9" x 12" x 6" deep hand sink per detail C-2-1E. Provide w/type 5B faucet.

- Fabricator to provide an integral side splash, xx” high x 1” thick x depth of counter top, to isolate hand sink from counter similar to C-2-1E. Soap & towel dispenser by Owner.

- Open base to floor to accommodate undercounter freezer, Item 263.

- Compartment open to floor for “Slim Jim” waste receptacle w/ hinged doors similar to detail C-2-2. Height of doors w/integral toe base to be approximately 27" and clear floor by 1/2”. Provide open front at top of door to underside of counter top for trash access.

Item No 265 - WASTE CONTAINER                                                        1 Required

This item is not in contract. The specification is for reference only. The item is shown on the drawing for informational purposes and generally represents the size/ capacity of unit to be provided by the Owner.

Rubbermaid Model No. Slim Jim, 23 gal., 20" x 11" x 30" high, plastic construction. Specify color:  light gray.

Item No 266 - HOT FOOD MERCHANDISER, TWO-TIER                     1 Required

Merco Savory Model No.  2TSW-5024,  Hot Food Merchandiser, Two-Tier.

- Furnish per manufacturer's standard specifications.

Item No 267 - FREEZER, 1-SECTION                                                    1 Required

Traulsen & Company, Inc. Model No. ALT132WUT-HHS,  Freezer, 1-Section.

- Furnish w/ cord & plug.

- Doors w/ cylinder locks, common keyed.

- Provide five (5) year compressor warranty and one (1) year refrigeration service contract.

- Half doors.

- One pair universal bottom support tray slides  for 18” x 26” or 12” x 20” (one pair per half section).

- Provide one additional chrome plated wire shelves.

Item No 268 - REFRIGERATOR, 2-SECTION ROLL-IN                             1 Required

Traulsen & Company, Inc. Model No. RRI232HUT-FHS,  Refrigerator, 2-Section Roll-In.

- Furnish w/ cord & plug.

- Doors w/ cylinder locks, common keyed.

- Provide five (5) year compressor warranty and one (1) year refrigeration service contract.

Item No 269 - MOP BUCKET                                                                    1 Required

This item is not in contract. The specification is for reference only. The item is shown on the drawing for informational purposes and generally represents the size/ capacity of unit to be provided by the Owner.

Rubbermaid Commercial Products Model No. 6186,  Mopping Combo Pack, 6185 mop bucket/6127-01 wringer, foot activated emptying system, non-marking casters, yellow.

Item No 270 - MOP SINK, HEAVY DUTY                                               1 Required

IMC/Teddy Food Service Corp. Model No. FS-H,  Mop Sink, Heavy Duty.

- Mop holder, 4 pole (MH4).

- Service sink wall faucet (SSF).

- Hose, 36" long (HA).

Item No 271 - TRASH RECEPTACLE (44 GAL.)                                   LOT Required

This item is not in contract. The specification is for reference only. The item is shown on the drawing for informational purposes and generally represents the size/ capacity of unit to be provided by the Owner.

Rubbermaid Commercial Products Model No. 2643-06,  Round, 44 gal., 24"D x 31.50"H, High-impact plastic construction Specify color: gray

Item No 272 - SALAD BAR                                                                      1 Required

This item is not in contract. It is specified by the Architect.

Item No. 273 - SPARE NUMBER -

Item No. 274 - SPARE NUMBER –

Item No 275 - ICE PAN                                                                              2 Required

Custom fabrication. Unit to be size and shape shown on the plan, drop-in type, approximately 36” diameter x 4” deep inside, stainless steel where exposed, with removable perforated false bottom in two sections, insulated, 21” high stainless steel post for mounting Item 276, Fruit Display, constructed similar to detail C-4-5.

Item No 276 – FRUIT DISPLAY                                                                 2 Required

Custom fabrication,  Eastern Tabletop (Phone: 718-522-4142), Ware-Ever, 16” diameter ivory serving bowl with chrome wire stand.  Installed on Item 275, Ice Pan.

Item No 277 - REFRIGERATOR, UNDERCOUNTER                               2 Required

Kairak Innovations Model No. KRUC-32S.

- Provide on 4" casters.

- Provide five (5) year compressor warranty and one (1) year refrigeration service contract

Item No 278 - COLD AIR PAN, REFRIGERATED                                  4 Required

ASDI, Inc. Model No. CAP-FI-87,  Cold Air Pan, Refrigerated.

- Integral “salad bar” food shield with light and non-glare glass.  Light on separate circuit from refrigeration system.  Provide light deflector on rear of fixture to prevent splash of light to customers behind unit in island application.

- Optional inset pan 4" deep for ice application

- Provide five (5) year compressor warranty and one (1) year refrigeration service contract.

- Condensate evaporator.

- Coordinate w/ Millwork Contractor for ventilation requirements of self-contained refrigeration systems.

Item No 279 - COUNTER                                                                      2 Required

This item is not in contract. It is specified by the Architect.

Item No 280 - COUNTER BY ARCHITECT                                          1 Required

This item is not in contract. It is specified by the Architect.

Item No 281 - HAND SINK, DROP-IN                                                1 Required

Eagle Model No. SR10-14-5-1 Hand Sink, Drop-In

- Furnish per manufacturer's standard specifications.

- Ship to fabricator.

- Locate in cut-out of intermediate shelf.

Item No 282 - REFRIGERATED PREP TABLE                                  1 Required

Kairak Innovations Model No. KRP-65S,  Refrigerated Prep Table.

- Stainless steel rear where exposed.

- Provide 1/2 “ thick Richlite cutting board in lieu of standard.

- Provide on 4" casters.

- Removable tray slides with spacing at 2 1/2".

- Telescoping night cover.

Item No 283 - WASTE CONTAINER                                                  LOT Required

This item is not in contract. It is provided by Owner. The specification is for reference only.

By Owner Model No. Slim Jim waste receptacle.  #3540, 20" x 11" x 30" high.  Gray.

Item No 284 - FOOD SHIELD, VERTICAL                                              1 Required

Brass Smith Inc Model No. Z9500,  Finish to be brushed aluminum. Of size and shape as shown on plan.

- Undercounter mount, millwork counter MWU5 heavy duty flange/inverted

- 3/8” tempered glass (standard for shelves) max. 66" long.

- Provide shop drawings of size and scale outlined in specific conditions.

Item No 285 - HOT FOOD WELL, DROP IN, 2-WELL                          1 Required

Wells Manufacturing Company Model No. MOD-200TDM,  Hot Food Well, Drop In, 2-Well.

- Drain manifold.

- Plumbing Division to provide gate valve on drain manifold.

- Drain Screen

- Provide type 6 single pantry faucet.

- Wellslok Extension Kit, for UL listed 12" x 20" warmers only, to adapt wood countertop up to 1-1/2" thick (1 well requires one Kit, 2 to 3 well units require two kits, 4 to 5 well units require three kits)

- 72" Wiring (thermostatically controlled warmers) per well.

Item No 286 - EXHAUST HOOD, ISLAND STYLE                               1 Required

Gaylord Industries, Incorporated Model No. GX2-BDL-48.  Unit to be U.L. listed grease extractor, high velocity island canopy of size and shape as shown on plan x  30" high, built in one section.

One (1) lot stainless steel enclosure paneling, at all open sides, to finished ceiling (F.S.E. Contractor to verify height). Paneling must be supplied by Exhaust Hood Manufacturer and installed by F.S.E. Contractor. Adhere to Specific Conditions for installation of fabricated equipment.

Ventilator to be provided w/ full length fire damper located in the ventilator extractor plenum between the extractor inserts and the ventilator duct collar.  The damper shall be controlled by an electrically driven actuator specifically listed for such use and shall be activated by fail safe thermostats located at the duct collar.  The damper shall close each time the exhaust fan is turned off, to prevent conditioned air from escaping the occupied space via thermal drafts and then re-open when the exhaust fan is started.

Ventilator to be U.L. and NSF listed and be in full accordance with NFPA 96 and all state/local codes/ordinances applicable.

F.S.E. Contractor to supply the necessary mounting rods for hanging ventilator.

Supply and/ or exhaust air requirements as shown on plans.

Options:

- Recessed incandescent light fixtures as indicated on electrical plan.

- Pre-piping of the Item No. 156, Fire Protection System is to be provided by the Exhaust Hood manufacturer at the time of hood fabrication.

- Pre-piping to include fusible-link detector bracket assemblies and cable runs, and wet-chemical manifold piping, duct collar and plenum protection piping with termination nozzles, and cooking appliance nozzle drops with termination nozzles.

- Location of cooking appliance nozzle drops are to be fully dimensioned on hood submission drawings.

- Fusible link detector and wet chemical piping terminations to be coordinated with the FSE Contractor and clearly indicated on hood submission drawings.

- Access to the fusible links for inspection and change-outs to be provided.  The hood manufacturer and FSE Contractor to coordinate their efforts to assure the necessary access.

- Supply air plenum boxes w/ full length internal removable baffle, mounted at finished ceiling height.

FSE Contractor is responsible for fit of equipment. Prior to releasing hoods for fabrication, FSE Contractor needs to verify field conditions (existing &/or proposed) and to determine clearances to all structural items, obstructions, etc. The FSE Contractor will coordinate with other trades to confirm that the hood can be mounted as proposed and that the ductwork and final connection can be accommodated without conflict.  Failure to perform this step may result in modifications to the exhaust hood at the FSE Contractor's expense.

Item No 287 - REFRIGERATED EQUIPMENT STAND                            1 Required

Kairak Innovations Model No.KRES-48S,  Refrigerated Equipment Stand.

- Provide five (5) year compressor warranty and one (1) year refrigeration service contract.

- Provide on 4" casters.

- Heat shield top.

Item No 288 – WOK RANGE, STEP UP                                                   1 Required

Rankin – Delux, Inc. ORSU-636-C,  Wok Range, Step Up

- Delete backsplash.

- Flexible disconnects per specific conditions.

Item No 289 - S/S SURROUND                                                               1 Required

Custom fabrication. Of size and shape as shown on plan fabricated per detail C-2-20.

Item No. 290 –PANELBOARD

This item is not in contract.  It is in the millwork contract.

Item No 291 - SHELVING                                                                        1 Required

Metro Equipment Corporation Model No. SUPER ERECTA,  Of size and shape as shown on plan x 2 tiers high.  34" high posts to match shelf type.

- 5" diameter polyurethane swivel casters, two (2) with brakes.

Item No 292 - TOASTER, CONVEYOR TYPE                                        1 Required

Merco Savory Model No. RT-2VSE,  Toaster, Conveyor Type.

- Furnish per manufacturer's standard specifications.

Item No 293 - FOOD SHIELD                                                                 1 Required

Brass Smith Inc Model No. Z9945 EZ,  Finish to be brushed aluminum.  Of size and shape as shown on plan.

- Undercounter mount, millwork counter MWU5 heavy duty flange/inverted

- 3/8” tempered glass (standard for shelves) max. 66" long.

- Lights linear halogen. Wire lights to remote switch in counter apron. Coordinate w/ Millwork Contractor.

- Wiring exit point option: right side  (customer view).

- Provide shop drawings of size and scale outlined in specific conditions.

Note:  Lights to be provided w/ concealed wiring of sufficient length through protector case support posts to remote control located below counter.

Item No 294 - ROUND WARMER, COUNTER TOP                               4 Required

Wells Manufacturing Company Model No. SW-10T,  Round Warmer, Counter Top.

- Inset, 11 quart capacity, with hinged lid.

- Legs, set of four 4" adjustable, plastic.

- Set on millwork shelf below counter top.

Item No 295 - ROUND WARMER, DROP IN                                        6 Required

Wells Manufacturing Company Model No. SS-10ULTD,  Round Warmer, Drop In.

- Plumbing Division to provide gate valve on indirect waste. Plumber to manifold three IW's for each set to drain w/ one gate valve.

- Ship unit to fabricator for installation in counter, Item 215.

- Provide two (2) type 6 single pantry faucet. One faucet for each set of three wells.

- Drain Screen

- Wellslok Extension Kit, for UL listed 12" x 20" warmers only, to adapt wood countertop up to 1-1/2" thick (1 well requires one Kit, 2 to 3 well units require two kits, 4 to 5 well units require three kits)

- 72" Wiring (thermostatically controlled warmers) per well.

Item No 296 - FOOD SHIELD                                                              2 Required

Brass Smith Inc Model No. Z9945 EZ,  Finish to be brushed aluminum. Of size and shape as shown on plan.

- Undercounter mount, millwork counter MWU5 heavy duty flange/inverted

- 3/8” tempered glass (standard for shelves) max. 66" long.

- Lights linear halogen. Switch mounted in counter apron.  Coordinate w/ Millwork Contractor.

- Wiring exit point option: Refer to electrical drawing.

- Provide shop drawings of size and scale outlined in specific conditions

Note:  Lights to be provided w/ concealed wiring of sufficient length through protector case support posts to remote control located below counter.

Item No 297 - SERVICE COUNTER, SOUP                                          1 Required

This item is not in contract. It is specified by the Architect.

Item No. 298 - SPARE NUMBER -

Item No. 299 - SPARE NUMBER –

Item No 300 - POS SYSTEM                                                                2 Required

- This item is not in contract. It is provided by the Owner.

- Utility requirements for provided by a product supplier or Owner will be confirmed by the FSE Contractor and represented on the dimensioned rough-in plans for use by the construction trades.

- Any utility connections indicated on drawings are representative and intended to assist the Engineer in planning base building utility systems.

Item No 301 - SCALE                                                                           2 Required

- This item is not in contract. It is provided by the Owner.

- Utility requirements for provided by a product supplier or Owner will be confirmed by the FSE Contractor and represented on the dimensioned rough-in plans for use by the construction trades.

- Any utility connections indicated on drawings are representative and intended to assist the Engineer in planning base building utility systems.

Item No 302 - CREDIT CARD TERMINAL                                         2 Required

- This item is not in contract. It is provided by the Owner.

- Utility requirements for provided by a product supplier or Owner will be confirmed by the FSE Contractor and represented on the dimensioned rough-in plans for use by the construction trades.

- Any utility connections indicated on drawings are representative and intended to assist the Engineer in planning base building utility systems.

Item No 303 - CHANGE DISPENSER                                                2 Required

By Owner Model No. CHANGER,  Change Dispenser.

- This item is not in contract. It is provided by the Owner.

- Utility requirements for provided by a product supplier or Owner will be confirmed by the FSE Contractor and represented on the dimensioned rough-in plans for use by the construction trades.

- Any utility connections indicated on drawings are representative and intended to assist the Engineer in planning base building utility systems.

Item No 304 - CASHIER STOOL                                                          6 Required

This item is not in contract. It is specified by the Architect.

Item No 305 - COUNTER, CASHIER                                                   6 Required

This item is not in contract. It is specified by the Architect.

Item No 306 - COUNTER                                                                    1 Required

This item is not in contract. It is specified by the Architect.

Item No 307 - COUNTER                                                                     1 Required

This item is not in contract. It is specified by the Architect.

Item No. 308 - SPARE NUMBER -

Item No 309 - ICE MAKER/ICE & WATER DISPENSER                    4 Required

Follett Corporation Model No. 110CT400A,  Provide five (5) year compressor warranty and one (1) year refrigeration service contract. Dispenses ice and water.

- Provide flexible water disconnects for installation by Plumbing Division

- Connect to Item 67, Central Water Treatment System.

- Chilled water option

- SensorSAFE actuation package

Item No 310 - MICROWAVE OVEN W/ DIAL TIMER                        4 Required

Amana Refrigeration, Inc. Model No. RCS10DA,  Microwave Oven W/ dial Timer.

- Furnish per manufacturer's standard specifications.

Item No 311 - FLATWARE DISPENSER                                          16 Required

Custom fabrication.  FLATWARE DISPENSER.

Item No 312 – 319  CONDIMENT COUNTER                                    1 Required

- This item is not in contract. It is specified by the Architect.

Item No 320 - POS SYSTEM                                                               4 Required

- This item is not in contract. It is provided by the Owner.

- Utility requirements for provided by a product supplier or Owner will be confirmed by the FSE Contractor and represented on the dimensioned rough-in plans for use by the construction trades.

- Any utility connections indicated on drawings are representative and intended to assist the Engineer in planning base building utility systems.

Item No 321 - SCALE                                                                           4 Required

- This item is not in contract. It is provided by the Owner.

- Utility requirements for provided by a product supplier or Owner will be confirmed by the FSE Contractor and represented on the dimensioned rough-in plans for use by the construction trades.

- Any utility connections indicated on drawings are representative and intended to assist the Engineer in planning base building utility systems.

Item No 322 - CREDIT CARD TERMINAL                                         4 Required

- This item is not in contract. It is provided by the Owner.

- Utility requirements for provided by a product supplier or Owner will be confirmed by the FSE Contractor and represented on the dimensioned rough-in plans for use by the construction trades.

- Any utility connections indicated on drawings are representative and intended to assist the Engineer in planning base building utility systems.

Item No 323 - CHANGE DISPENSER                                                4 Required

By Owner Model No. CHANGER,  Change Dispenser.

- This item is not in contract. It is provided by the Owner.

- Utility requirements for provided by a product supplier or Owner will be confirmed by the FSE Contractor and represented on the dimensioned rough-in plans for use by the construction trades.

- Any utility connections indicated on drawings are representative and intended to assist the Engineer in planning base building utility systems.