### Legend Dome Camera

- Pan/tilt/zoom (PTZ) dome receiver with options for camera, housing, and bubble.

  - The Legend camera, housing, and pan/tilt drive unit shall be as manufactured by GE Security or an approved equal.

  - The high-speed pan/tilt drive unit shall meet or exceed the following design and performance specifications:

    - The Legend shall be backward compatible and can replace domes in an existing Digiplex III or later system.

    - The unit shall use SilkTrak direct drive technology for smooth movement. No gears or belts shall be used in the drive system.

      - SilkTrak shall use closed-loop positioning technology.

        - Preset positioning accuracy shall be within ±0.015 degrees.

      - The unit shall operate at manual speeds between 0.01 and 250 degrees per second.

      - The unit shall have a maximum speed to 450 degrees per second during preset seek operations.

      - Maximum manual speed operation shall be programmable from 100 to 250 degrees per second in steps of 10 degrees per second.

    - The unit shall contain a built-in receiver.

      - The receiver shall decode operational commands, including panning, tilting, and zooming the camera, that originate from a compatible controller keypad.

      - The receiver shall feature Rotary switch-selectable addressing with settings from 0 to 1599.

    - The unit shall use a graphical menu-based system for programming the camera and pan/tilt unit.

      - A compatible keypad can be used as a simulated computer mouse to navigate through the menus.

      - The menus shall have the following options:

        - Setup

          - Memory

          - Passcode

            - Installer passcode

            - Admin passcode

            - Operator passcode

            - Operator access options

          - Title/Date

            - Camera number

            - Camera title

            - Display title

            - Title POS (position)

            - Title font color

            - Title font size

            - Title font weight

            - Block color

            - Block transparency

            - Installation date

          - Network

            - IP address

            - Net mask

            - Server

            - Gateway

            - MAC address

          - Preferences

            - Display

            - Coordinates

        - Camera

          - Privacy masks

          - Settings

            - Day/Night

            - White balance

            - Exposure settings

            - Phase lock enabled

            - Phase adjustment

            - Stabilization

            - Digital zoom limit

            - Lift

            - Gain

            - Zoom speed

            - Tilt up limit

            - Tilt down limit

            - Pan left limit

            - Pan right limit

            - Clear limits

        - Actions

          - Presets

          - ShadowTours

          - Macros

          - Areas

        - Alarms

          - Summary

          - Contact Setup

          - Priority

          - Display

          - Relay State

        - Control

          - Command map

          - Power on and Resume

          - Speeds Tracking

        - System

          - Diagnostics

          - Status

          - Logs

          - Firmware updates

        - Exit

    - The unit shall be capable of 32 macros.

      - Any macro can be linked to any other macro.

      - Each macro can include up to 16 steps.

      - Each macro step can be set as a preset position, shadow tour, jump to another step, relay action or link to another macro.

      - Individual dwell times can be set for each macro step for a preset or relay action.

      - Zoom action can be programmed for the transition to each step to include maintaining the current zoom position or zooming to the maximum wide angle position.

    - The unit shall be capable of up to 16 Shadow Tours.

      - The Shadow Tours shall have a maximum total duration of 20 minutes.

      - A Shadow Tour shall be defined as a tour that the camera learns by recording the user’s manual operation of the camera via a keypad.

    - The unit shall be capable of 127 presets that store pan, tilt, and zoom position.

      - Each preset shall include a 60-character title.

      - The presents shall include individual camera settings including backlight compensation, iris settings, and shutter settings.

    - Eight dry contact alarm inputs shall be assignable to presets, tours, or macros.

      - Each alarm input can be set to trigger any preset position, shadow tour or macro.

      - The alarm inputs shall be capable of controlling either or both of two relay outputs.

      - Each alarm input shall be configurable as normally open or normally closed.

      - Alarm inputs can be assigned in any priority order.

      - Each alarm can be programmed with a 60-character title.

    - The unit shall include 24 available privacy masks to prevent operators from viewing neighboring or sensitive areas.

      - Eight privacy masks shall be able to display simultaneously.

      - Spherical privacy masking shall use a  three-dimensional representation of the camera view and eliminate mask drift caused by camera tilt angle.

      - Privacy masks can be set in the area from horizontal to 70 degrees below the horizontal.

    - The unit shall allow passcode protection that prevents unauthorized personnel from changing programming settings while still allowing operator access.

      - The installer passcode shall allow access to all dome features.

      - The admin passcode shall allow access to all dome features except passcodes and firmware upgrades.

      - The operator passcode allows no access until it is granted access to specific features on an area-by-area basis.

    - The unit shall offer an Ethernet connection to flash software upgrades over a standard IP network using standard web browser software.

    - The camera shall have 184-degree tilt travel for above horizon viewing and electronic image flip.

      - The unit shall include an electronic image flip feature to allow the camera to continuously view a subject moving beneath it without having to spin on its pan axis.

  - The available cameras shall meet or exceed the following specifications:

    - The camera shall have the form factor as typical of a CCTV video camera for use in a dome housing.

    - The signal system shall be available in NTSC or PAL video signal format.

    - The camera shall provide a composite video output of 1.0 V p-p.

    - The image-capturing device shall be a 1/4-inch CCD image sensor.

    - The camera shall provide a horizontal resolution of:

      - 470 TV lines, NTSC (530 TV lines, NTSC for 36X)

      - 460 TV lines, PAL (530 TV lines, PAL for 36X)

    - The camera scanning system shall be 2:1 interlacing.

    - The camera shall have digital signal processing.

    - The camera shall have automatic white balance.

      - The camera shall also be capable of manual setup for white balance via the menu system.

    - The camera shall have automatic iris control and manual override.

    - The camera shall provide backlight compensation while in automatic iris control mode.

    - The following camera options shall be available with the Legend dome:

      - 18X color camera

        - Optical zoom: 18X

        - Digital zoom: 12X

        - Sensitivity

          - 0.7 lux at 1/60 second NTSC

          - 0.7 lux at 1/50 second PAL

          - 0.05 lux at 1/4 second NTSC

          - 0.05 lux at 1/3 second PAL

        - Electronic shutter speed

          - 1/1 to 10,000 seconds, 22 steps, NTSC

          - 1/1 to 10,000 seconds, 22 steps, PAL

        - Lens

          - F=4.1mm (wide) to 73.8 mm (tele)

            - F1.4 to F3.0

          - Angle of view (H): 48° (wide) to 2.8° (tele)

        - Synchronization

          - External (V-lock)

          - Internal

        - Signal-to-noise ratio: 48 dB

      - 26X day/night camera

        - Optical zoom: 26X

        - Digital zoom: 12X

        - Sensitivity with ICR in place (color)

          - 1.0 lux at 1/60 second NTSC

          - 1.0 lux at 1/50 second PAL

          - 0.07 lux at 1/4 second NTSC

          - 0.07 lux at 1/3 second PAL

        - Sensitivity with ICR off (B&W)

          - 0.15 lux at 1/60 second NTSC

          - 0.15 lux at 1/50 second PAL

          - 0.01 lux at 1/4 second NTSC

          - 0.01 lux at 1/3 second PAL

        - Electronic shutter speed

          - 1/4 to 10,000 seconds, 20 steps, NTSC

          - 1/3 to 10,000 seconds, 20 steps, PAL

        - Lens

          - F=3.5mm (wide) to 91 mm (tele)

            - F1.6 to F3.8

          - Angle of view (H): 54.2° (wide) to 2.2° (tele)

        - Synchronization

          - Internal

          - Line lock

        - Signal-to-noise ratio: 48 dB

      - 26X day/night camera with image stabilization

        - Optical zoom: 26X

        - Digital zoom: 12X

        - Sensitivity with ICR in place (color)

          - 2.0 lux at 1/60 second NTSC

          - 2.0 lux at 1/50 second PAL

          - 0.14 lux at 1/4 second NTSC

          - 0.14 lux at 1/3 second PAL

        - Sensitivity with ICR off (B&W)

          - 0.7 lux at 1/60 second NTSC

          - 0.7 lux at 1/50 second PAL

          - 0.05 lux at 1/4 second NTSC

          - 0.05 lux at 1/3 second PAL

        - Electronic shutter speed

          - 1/4 to 10,000 seconds, 20 steps, NTSC

          - 1/3 to 10,000 seconds, 20 steps, PAL

        - Lens

          - F=3.5mm (wide) to 91 mm (tele)

            - F1.6 to F3.8

          - Angle of view (H): 42.0° (wide) to 1.6° (tele)

        - Synchronization

          - Internal

          - Line lock

          - Signal-to-noise ratio: 48 dB

      - 36X day/night

        - Optical zoom: 36X

        - Digital zoom: 12X

        - Sensitivity with ICR in place (color)

          - 1.4 lux at 1/60 second NTSC

          - 1.4 lux at 1/50 second PAL

          - 0.1 lux at 1/4 second NTSC

          - 0.1 lux at 1/3 second PAL

        - Sensitivity with ICR off (B&W)

          - 0.1 lux at 1/60 second NTSC

          - 0.1 lux at 1/50 second PAL

          - 0.01 lux at 1/4 second NTSC

          - 0.01 lux at 1/3 second PAL

        - Electronic shutter speed

          - 1/4 to 1/10,000 sec, 20 steps, NTSC

          - 1/3 to 1/10,000 sec, 20 steps, PAL

        - Lens

          - F=3.4 mm (wide) to 122.4 mm (tele)

            - F1.6 to F4.5

          - Angle of view (H): 57.8° (wide) to 1.7° (tele)

        - Resolution: 530 TVL NTSC, 530 TVL PAL

  - All available Legend dome housings shall meet or exceed the following specifications:

    - All housings shall include both coaxial and UTP video transmission connections.

      - The connectors for the housing shall be as follows:

        - Coaxial

          - Video: BNC

          - Control: Four-position removable terminal strip

          - Power input: Two-position removable terminal strip

        - UTP

          - Video: Twisted-pair using removable terminal strip

          - Control: Four-position terminal strip

          - Power input: Two-position terminal strip

    - Each housing shall have site-tied memory so that cameras can be removed and switched between housings, yet the settings for a specific location shall remain the same.

    - The housing shall have the following control protocols:

      - Input signal

        - Digiplex RS422, 4800 baud

        - Impac RS485, 9600 baud

        - GE ASCII RS422, 9600 baud

        - Pelco “P” RS422, 2400 baud

        - Honeywell Ultrak RS422, 9600 baud

      - Video output: Composite, 1.0 V p-p

    - Each housing shall provide rotary switches for setting the camera’s site address and communication protocol.

      - Site addresses can be numbered from 0 to 1599.

      - Available protocol switches include:

        - 0 = Digiplex (RS-422) @ 4800 baud

        - 1 = Impac (RS-485) @ 9600 baud

        - 5 = ASCII @ 9600 baud

        - 6 = Pelco D @ 2400 baud

        - 7 = Ultrak @ 9600 baud (even parity)

        - Other protocol switches are for future use.

    - The following housing options shall be available:

      - Flush-mount housing

        - The housing shall be used indoors only.

        - The housing shall be designed for in-ceiling applications.

        - The housing shall include a T-bar support kit.

        - The housing shall have an environmental rating of IP50.

        - The housing shall operate at temperatures between 32 and 122°F (0 to 50°C).

        - The housing shall be constructed of ABS plastic.

        - The electrical specifications for the housing shall be as follows:

          - Operating voltage: 24 VAC ± 4 VAC

          - Power, startup: 30 W

          - Power, running: 24 W

        - The physical specifications for the housing shall be as follows:

          - Dimensions: 9.5 in. diameter (241 mm) at its widest point by 9.6 in. high (243 mm).

          - Weight (including housing, bubble, and pan/tilt unit): 6 lb., 8.5 oz. (2.96 kg).

      - Pendant-mount housing

        - The housing shall be weatherproof for indoor and outdoor applications.

        - The housing shall be available with an optional heater and fan unit.

        - The housing shall have an environmental rating of IP55.

        - The housing shall operate at temperatures between 32 and 122°F (0 to 50°C) without a heater/fan unit or between -40 and 122°F (-40 and 50°C) with a heater/fan unit.

        - The housing shall have a gray finish.

        - The electrical specifications for the housing shall be as follows:

          - Operating voltage: 24 VAC ± 4 VAC

          - Power, startup: 30 W

          - Power, running: 16 W

          - Power, startup, with heater and fan unit: 54 W

          - Power, running, with heater and fan unit: 48 W

        - The physical specifications for the housing shall be as follows:

          - Dimensions: 9.5 in. diameter (241 mm) at its widest point by 11.9 in. high (303 mm).

          - Weight (including housing, heater/fan unit, bubble, and pan/tilt unit): 6 lb., 8.5 oz. (2.96 kg).

      - Wall-mount housing

        - The housing shall be weatherproof for indoor and outdoor applications.

        - The housing’s wall arm shall be constructed of cast aluminum.

        - The housing shall be available with an optional heater and fan unit.

        - The housing shall have an environmental rating of IP55.

        - The housing shall operate at temperatures between 32 and 122°F (0 to 50°C) without a heater/fan unit or between -40 and 122°F (-40 and 50°C) with a heater/fan unit.

        - The housing shall have a gray finish.

        - The electrical specifications for the housing shall be as follows:

          - Operating voltage: 24 VAC ± 4 VAC

          - Power, startup: 30 W

          - Power, running: 16 W

          - Power, startup, with heater and fan unit: 54 W

          - Power, running, with heater and fan unit: 48 W

        - The physical specifications for the housing shall be as follows:

          - Dimensions: 9.5 in. diameter (241 mm) at its widest point by 9.6 in. high (243 mm).

          - Weight (including housing, heater/fan unit, bubble, and pan/tilt unit): 8 lb., 8.5 oz. (3.87 kg).

      - Heavy-duty housing

        - The housing shall be weatherproof and designed to hold up under tough outdoor use.

        - The housing shall come with adjustable wall-mount brackets.

        - The housing shall be available with an optional heater and fan unit.

        - The housing shall operate best at temperatures between 32 and 122°F (0 to 50°C) without a heater/fan unit or between -13 and 122°F (-25 and 50°C) with a heater/fan unit.

        - The housing shall be constructed of 1/4–inch aluminum.

        - The housing shall have a gray finish.

        - The electrical specifications for the housing shall be as follows:

          - Operating voltage: 24 VAC ± 4 VAC

          - Power, startup: 30 W

          - Power, running: 16 W

          - Power, startup, with heater and fan unit: 76 W

          - Power, running, with heater and fan unit: 70 W

        - The physical specifications for the housing shall be as follows:

          - Dimensions: 10.56 in. (268 mm) wide by 9.29 in. high (236 mm).

          - Weight (including housing, heater/fan unit, bubble, and pan/tilt unit): 13 lb., 8 oz. (6.12 kg).

      - Rugged pendant-mount housing

        - The housing shall be weatherproof and designed for outdoor applications that require tamper resistance.

        - The housing shall be available with an optional heater and fan unit.

        - The housing shall have an environmental rating of IP66.

        - The housing shall operate best at temperatures between 32 and 122°F (0 to 50°C) without a heater/fan unit or between -40 and 122°F (-40 and 50°C) with a heater/fan unit.

        - The housing shall be constructed of cast aluminum.

        - The housing shall have a gray finish.

        - The electrical specifications for the housing shall be as follows:

          - Operating voltage: 24 VAC ± 4 VAC

          - Power, startup: 30 W

          - Power, running: 16 W

          - Power, startup, with heater and fan unit: 76 W

          - Power, running, with heater and fan unit: 70 W

        - The physical specifications for the housing shall be as follows:

          - Dimensions: 9.5 in. diameter (241 mm) at its widest point by 11.1 inches high (283 mm).

          - Weight (including housing, heater/fan unit, bubble, and pan/tilt unit): 12 lb., 8.5 oz. (5.68 kg).

  - The Legend dome bubbles shall meet or exceed the following specifications:

    - The bubble shall be available in the following:

        - Clear acrylic (no light loss)

        - Smoke acrylic (approximately 1/2 f-stop light loss)

        - Chrome mirrored acrylic (approximately 2 f-stop light loss)

        - Gold mirrored acrylic (approximately 2 f-stop light loss)

        - Polycarbonate clear (no light loss)

          - The polycarbonate clear bubble shall be available for heavy duty and rugged housings only.

        - Polycarbonate smoke (approximately 1/2 f-stop light loss)

          - The polycarbonate smoke bubble shall be available for heavy duty and rugged housings only.

      - Each bubble shall be injection molded for clearer images.

      - Each bubble shall include interlocking clips and safety cables, which vary depending on bubble and housing.

  - The Legend dome shall have a variety of optional mounting accessories available.

    - The cast-aluminum wall-mount arm shall be used to mount a dome to a vertical surface.

      - The wall-mount arm shall be for indoor or outdoor use.

      - The wall-mount arm shall be usable with both the cast aluminum outdoor rugged pendant housing and the plastic indoor pendant housing.

        - The arm shall require a minimum load of 100 pounds when used with the rugged pendant housing.

        - The arm shall require a minimum load of 50 pounds when used with the plastic pendant housing.

      - The wall-mount arm shall be capable of being attached directly to a vertical surface or mated with a corner-mount, pole-mount, or roof-mount bracket.

      - The wall-mount arm shall have the following additional specifications:

        - Weight: 2.0 lb. (0.91 kg)

        - Construction: Aluminum

        - Finish: Gray

    - The gooseneck wall-mount arm shall be used to mount a pressurized PressurDome to a vertical surface.

      - The gooseneck wall-mount arm shall be capable of being attached directly to a vertical surface or mated with a corner-mount, pole-mount, or roof-mount adapter.

      - The gooseneck wall-mount arm shall require a 1.5-inch pipe.

      - The gooseneck wall-mount arm shall require a minimum load of 100 pounds when used with the PressurDome housing.

    - The parapet swing-arm mount shall be used to mount a dome to a roof parapet.

      - The parapet swing-arm mount shall be able to swing the dome out from a building and in above a roof for a broad range of surveillance coverage and easier servicing.

      - The parapet swing-arm mount shall require a minimum load of 100 pounds when used with any Legend housing.

      - The parapet swing-arm mount shall have the following additional specifications:

        - Weight: 7.75 lb. (3.52 kg)

        - Construction: Aluminum

        - Finish: Gray powder coat

    - The corner-mount adapter shall be used to mount a dome to the corner of a building.

      - The corner-mount adapter shall be usable with a wall-mount arm or gooseneck wall-mount arm to suspend the dome.

      - The adapter can also be used to mount a power supply.

      - The adapter shall require a minimum load of 100 pounds when used with heavy-duty, rugged, or pressurized housings and a minimum load of 50 pounds when used with all other housings.

      - The adapter shall have the following additional specifications:

        - Weight: 1.9 lb. (0.86 kg)

        - Construction: Aluminum

        - Finish: Gray powder coat

    - The pole-mount adapter shall be used to mount a dome to a pole.

      - The pole-mount adapter can be attached to the pole using bolts or three adjustable stainless-steel bands, not included with the adapter.

      - The adapter shall be usable with a wall-mount arm or gooseneck wall-mount arm to suspend the dome.

      - The adapter can also be used to mount a power supply.

      - The adapter shall require a minimum load of 100 pounds when used with heavy-duty, rugged, or pressurized housings and a minimum load of 50 pounds when used with all other housings.

      - The adapter shall have the following additional specifications:

        - Weight: 14.4 oz. (408 g)

        - Construction: Aluminum

        - Finish: Gray powder coat

    - The roof-mount adapter shall be used to hinge a dome back over a roof for safe servicing.

      - The roof-mount adapter shall be usable with a wall-mount arm or gooseneck wall-mount arm to suspend the dome.

      - The adapter shall require a minimum load of 100 pounds when used with heavy-duty, rugged, or pressurized housings and a minimum load of 50 pounds when used with all other housings.

      - The adapter shall have the following additional specifications:

        - Weight: 1.2 lb. (0.54 kg)

        - Construction: Aluminum

        - Finish: Gray powder coat

    - The T-bar ceiling panel shall be used to install flush-mount housings into paneled T-bar type ceilings, which consist of a grid of metal T-bars supporting removable panels.

      - Each T-bar ceiling panel shall measure 24 x 24 in. to replace one 24 x 24 in. removable ceiling panel or half of one 24 x 48 in. removable ceiling panel.

      - The panel shall require a minimum load of 50 pounds when used with the flush-mount housing.

      - The panel shall have the following additional specifications:

        - Weight: 3.2 oz. (91 g)

        - Construction: Steel

        - Finish: White enamel

    - The T-bar support kit shall be used to install flush-mount housings into paneled T-bar type ceilings, which consist of a grid of metal T-bars supporting removable panels.

      - The T-bar support kit shall include one ceiling ring, two angle brackets, and mounting hardware.

      - The kit shall distribute the weight of the dome between the T-bars of the ceiling for greater stability.

      - The kit shall require a minimum load of 50 pounds when used with the flush-mount housing.

      - The kit shall have the following additional specifications:

        - Weight: 6.4 oz. (181 g)

        - Construction: Steel

        - Finish: Galvanized

    - The corner parapet-capping brackets shall be used to mount a dome to a roof parapet.

      - The brackets shall provide a platform for attaching a roof-mount adapter.

      - The narrow corner parapet-capping bracket shall fit 6 to 12 in. (152 to 305 mm) parapets.

      - The wide corner parapet-capping bracket shall fit 12 to 17 in. (305 to 432 mm) parapets.

      - The narrow corner parapet-capping bracket shall weigh 7 lb. (3.18 kg).

      - The wide corner parapet-capping bracket shall weigh 8 lb. (3.63 kg).

    - The straight parapet-capping brackets shall be used to mount a dome to a roof parapet.

      - The brackets shall provide a platform for attaching a roof-mount adapter.

      - The narrow straight parapet-capping bracket shall fit 6 to 12 in. (152 to 305 mm) parapets.

      - The wide straight parapet-capping bracket shall fit 12 to 17 in. (305 to 432 mm) parapets.

      - The narrow straight parapet-capping bracket shall weigh 14 lb. (6.4 kg).

      - The wide straight parapet-capping bracket shall weigh 15 lb. (6.8 kg).

  - The camera and pan/tilt unit shall conform to these internationally recognized compliance standards:

    - FCC

    - CE

    - UL

    - C-Tick