IŞIK UNIVERSITY

EE342-MICROPROCESSORS

EXPERIMENT #3

LOOPS AND ARRAY OPERATIONS

Objective:

The goal of this experiment is to learn writing and compiling assembler files externally and to learn how to perform array operations on 68HC11. Remember indexed addressing mode.

Read the lab. Handouts: 	1) How to write and compile assembler programs externally.

2) Assembler reference manual.

Example 3.1

- Searching for an Element in an Array

We are going to search for the element $FE in locations $C150 to $C175. If we find that element, we will save it in location $C180, otherwise this location will contain zero.

START		EQU	$C150		start address of the array

END		EQU	$C176		end address+1 of the array

FOUND		EQU	$C180

ORG	$C100		start address of the program

CLR	FOUND		initialize FOUND=0

LDX	#START-1	IX is used as pointer to array

REPEAT	INX			point to next location

CPX	#END		end of array yet

BEQ	EXIT		yes, stop

LDAA	0,X		no, get the contents of this location

CMPA	#$FE		contents match FE?

BNE	REPEAT	no, search again

STAA	FOUND		yes, save element, otherwise 0

EXIT		SWI			return to monitor program

- Searching for an Element in an Array Using a Counter

We are going to write the previous example using a different technique. Now, we know the start address and the size (number of elements) of the array (but not the end address).

START		EQU	$C150		start address of the array

SIZE		EQU	$26		number of elements $26 = 38

FOUND		EQU	$C180

ORG	$C100		start address of the program

CLR	FOUND		initialize FOUND=0

LDX	#START	IX is used as pointer to array

LDAB	#SIZE		ACCB is a counter

REPEAT	DECB			Decrement counter

BMI	EXIT		If negative exit

LDAA	0,X		no, get the contents of the location

INX			point to next location

CMPA	#$FE		contents match FE?

BNE	REPEAT	no, search again

STAA	FOUND		yes, save element, otherwise 0

EXIT		SWI			return to monitor program

Example 3.2:

Writing positive and negative elements of an array into two different arrays

### The source array ($C100 to $C110) contains positive and negative signed numbers. This program tests each element of the source array and saves positive elements in array POS, and negative elements in array NEG. The program code starts at address $C000.

Algorithm Description: The 68HC11 has two index registers (IX and IY), but in this example there are three arrays (source, NEG, POS) to point to. IY will point always to POS and IX will sometimes point to source and sometimes to NEG. We will use two temporary memory locations (TEMPNEG and TEMPSRC) to save pointer (IX) values.

ORG	$C100		start address of source array

SOURCE	FCB	5,0,-3,-34,2,67,120,48,-1,-2,6,18,-4,7,10,3

POS		RMB	16		Allocate memory for array POS

NEG		RMB	16		Allocate memory for array NEG

SIZE		EQU	16		number of elements of source 16

TEMPNEG	RMB	2		Allocate 2 bytes (address is 16 bit)

TEMPSRC	RMB	2		Allocate 2 bytes (address is 16 bit)

ORG	$C000		start address of the program

LDX	#NEG

STX	TEMPNEG	save NEG start. address(pointer of NEG)

LDX	#SOURCE	IX points to source

LDY	#POS		IY points to POS

LDAB	#SIZE		ACCB is a counter

REPEAT	DECB			Decrement counter

BMI	EXIT		If negative exit

LDAA	0,X		Read next element of source

INX			point to next location

TSTA			Test this element

BPL	POSITIVE	Element is positive

STX	TEMPSRC	save the pointer of the source

LDX	TEMPNEG	Now IX points to NEG

STAA	0,X		Save negative element in NEG

INX			Increment pointer of NEG

STX	TEMPNEG	Update pointer of NEG

LDX	TEMPSRC	Now IX points to SOURCE

BRA	REPEAT	repeat

POSITIVE	STAA	0,Y		Save positive element in POS

INY			increment pointer of POS

BRA	REPEAT	repeat

EXIT		SWI			return to monitor program

### Exercise 3.1: Running the Program in Example 3.2

Assemble the program given in Example 3.2 and load the executable file into EVB. Run it and check memory locations SOURCE, POS and NEG. Change values in source array using MM command of BUFFALO and run the program again.

### Exercise 3.2:

Write a program comparing two equal-size arrays (ARRAY1 and ARRAY2) to see if the elements are identical. If they are identical, indicate that by writing $FF to a memory location wit the symbolic name RESULT. If they are not identical clear this memory location.

Homework 3: An array has 10 elements (16 bit unsigned numbers each). Write an assembler program to find out how many elements of this array are divisible by three without reminder.