Jon D. Rodder’s Guide to Using a 1999 LS1/T56 Camaro Harness in a swap

Revision 1.0 - May 2, 2007

Note: I compiled this info from several sources and modified it, and I don’t know the original sources.  If I have plagiarized your work, please let me know so I can give you credit.  If you find any problems, also let me know about that.

URL: http://www.blown.net/camaro68.html

Email: ls1harness@blown.net

Section 1: Components you’ll be omitting

- EGR system

- AIR pump

- EVAP system

- Rear O2 sensors

- Air Conditioning

- Catalytic converters

Section 2: the 5 connectors on the stock LS1 harness

If you still have everything connected on the engine, you’ll have 5 connectors to hook up.  The first three are Metripacks, and the last two are unsealed connectors that were in the passenger compartment of the 4th-gen Camaro.  Highlighted entries are ones that won’t be used in this swap.

Actually there will be a 6th connector too—it’ll be a single terminal connector on a large purple wire.  The large purple wire snakes through the harness and eventually connects to, you guessed it, the starter.  You can either splice your original START wire to this wire, or disconnect it at the starter and connect your original START wire directly to the starter.

C100 Plug - black 10 pin connector

A PNK – Connect to Fuse IGN1 - Hot in Run and Start (IGN/INJ Bank 1 15A)

B (optional) DK GRN/WHT - PCM 43 A/C Clutch Relay Control

C (optional) DK GRN - PCM 18 A/C Clutch Status

D (remove) GRY - TPS +5v to TCS

E (remove) DK BLU - TPS signal to TCS

G PNK – Connect to Fuse IGN3 - Hot with IGN 1 Relay Engaged (Tranny/EGR 15A)

H DK BLU – Connect to Relay FAN2-86 and Relay FAN3-86 - PCM 33 Cooling Fan 2 and 3 Relay Control

J DK GRN – Connect to Relay FAN1-86 - PCM 42 Cooling Fan 1 Relay Control

K (remove) BLK - TPS sensor low to TCS

C101 Plug - grey 10 pin connector

A (remove) YEL/BLK - Low Coolant Sensor

B PNK - Connect to Fuse IGN2 - Hot in Run and Start (IGN/INJ Bank 2 15A)

C (remove) BRN - PCM 36 Air Pump Relay Control

D DK GRN/WHT - Connect to Relay FUEL-85 - PCM 9 Fuel Pump Relay Control

E PNK/BLK - Connect to Fuse IGN4 - PCM 19 Ignition Positive Voltage for PCM

G ORN - Connect to Fuse BATT (10A) - PCM 57 Battery Positive Voltage for PCM

H BLK/WHT - Connect to Ground

K (remove) GRY/BLK - PCM 53 TCS Spark Retard Signal to Brake Control Module

C105 Plug - black 8 pin connector

B PNK – Connect to Fuse IGN5 - Hot with IGN 1 Relay Engaged (TCC Stop Lamp Switch/MAF/O2 20A)

D (optional) DK GRN - PCM 37 Cruise Control Inhibit

F (optional) WHT - PCM 13 Cruise Engaged Signal

G WHT PCM 10 - Connect to your tach’s signal input - Tach output to Brake Control Module

H (remove) PPL - PCM 4 AIR Solenoid Relay Control

C220 - natural color 10 pin connector.

A (optional) TAN - Oil pressure output

B (optional) GRY - clutch pedal switch

C (remove) - Power supply to TCC stop lamp switch

D (remove) - TCC stop lamp switch

E (not used) BLK/WHT - (ground in?)

F (not used)

G BRN - Connect to original backup light harness - Backup light switch 12v"+" power input

H LT GRN - Connect to original backup light harness - Backup light switch out to light

J (not needed)

K DK GRN/WHT - Connect to your speedometer’s signal input - VSS signal to speedometer (4000ppm)

C230 - blue 10 pin connector

A (not needed)

B BRN/WHT - Connect to ground side of check engine light - Check engine light

C (not used)

D (not needed)

E (remove) DK BLU - VATS fuel enable input

F (optional) DK GRN/WHT - AC Request Signal

G (not needed)

H (not needed)

I (not needed)

J (not needed)

K DK GRN - Connect to OBD-II pin 2 - serial data class 2 output from LS1 PCM to OBD-II connector terminal #2 (purple)

Section 3: Relays

My wiring uses a total of 7 relays, to control all switched power for the engine, fuel pump, and cooling fans.  The cooling fan wiring is a dual-speed setup which switches a pair of fans between series and parallel operation to yield two speeds.  This is based on the use of 30A bosch style automotive relays, relays # 1,2, and 3 could be combined into 1 relay if you could find one with a 80A rating.

Relay 1 - IGN1

Terminal 85 	to conversion car wiring (a wire that is hot in run and start modes), I would use the wire that powered the original coil (remove the ballast resistor if it was a pre HEI car)

Terminal 86 	To ground

Terminal 30 	Direct to battery voltage

Terminal 87 	to 2 15A fuses(IGN 1 and IGN 2)  IGN 1 fuse to pin A of C100 connector, and IGN 2 fuse to pin B of C101 connector.

Relay 2 - IGN2

Terminal 85 	to conversion car wiring (a wire that is hot in run and start modes), I would use the wire that powered the original coil (remove the ballast resistor if it was a pre HEI car)

Terminal 86 	To ground

Terminal 30 	Direct to battery voltage

Terminal 87 	to 2 15A fuses(IGN 3 and IGN 4)  IGN 3 fuse to pin G of C100 connector, and IGN 4 fuse to pin E of C101 connector.

Relay 3 - IGN3

Terminal 85 	to conversion car wiring (a wire that is hot in run and start modes), I would use the wire that powered the original coil (remove the ballast resistor if it was a pre HEI car)

Terminal 86 	To ground

Terminal 30 	Direct to battery voltage

Terminal 87 	to 1 20A fuse(IGN 5), IGN 5 to pin B of connector C105

Relay 4 - FUEL - fuel pump relay

Terminal 85 	to pin D of connector C101

Terminal 86 	To ground

Terminal 30 	Direct to battery voltage

Terminal 87 	to 1 20A fuse then to the fuel pump.

Relay 5 - FAN1 – Left fan

Terminal 85	Hot in start or run

Terminal 86	To pin J of connector C100

Terminal 30	To 20A fuse and battery voltage

Terminal 87	To Left cooling Fan pin B

Relay 6 - FAN2 – Right fan

Terminal 85	Hot in start or run

Terminal 86	To pin H of connector C100 AND Terminal 86 of FAN3 relay

Terminal 30	To 20A fuse and battery voltage

Terminal 87	To Right cooling Fan pin B AND Terminal 87a of FAN3 relay

Relay 7 - FAN3 – speed control

Terminal 85	Hot in start or run

Terminal 86	To pin H of connector C100 AND Terminal 86 of FAN2 relay

Terminal 30	To Left cooling Fan pin A

Terminal 87	To 20A Ground

Terminal 87a	To Right cooling Fan pin B AND Terminal 87 of FAN2 relay

Section 4: the OBDII connector

If you didn’t get an OBDII connector with your car, you should be able to grab one out of any OBDII GM car.  The power wire for OBDII connector needs to go to a battery constant fuse, and you need both ground wires in the DLC connector.

Section 5: Recommended Tuning Changes

These are the changes I made in HPTuners:

Engine

General

EGR

EGR Enable            Change to Disabled

Shift Light

Shift Light Master Enable            Change to Disabled

Tach Output

Tach Output Resolutions - High            Change to 3 for 8cyl tach

Tach Output Resolutions - Low            Change to 3 for 8cyl tach

Transmission

Manual C.A.G.S.

General

C.A.G.S. Disable TPS            Change to 0%

System

General

Security

VATS Control            Change to None

Fans

General

First Stage Fan Turn On Temp            Change to 206 °F

First Stage Fan Turn Off Temp            Change to 199 °F

Second Stage Fan Turn On Temp            Change to 226 °F

Second Stage Fan Turn Off Temp            Change to 219 °F

Speedometer

Speedometer

General

New Tire Size            Change as Needed

New Gear Ratio         Change as Needed

Disable these DTCS:

P0137, P0138, P0140, P0141, P0157, P0158, P0160, P0161, P0401, P0404, P0405, P0410, P0412, P0418, P0420, P0430, P0440, P0442, P0443, P0446, P0449, P0452, P0453, P0461, P0462, P0463, P0530, P0645, P0803, P1415, P1416, P1441, P1539, P1546, P1626

Sections 6: PCM Connector Reference

| Circuit Number C1 (Blue) | This table contains all of the Pin connections, Wire colors, and Functions of Connector C1 (Blue). There are 80 pins on this connector |
| --- | --- |

| Pin |  | Wire Color | Function |
| --- | --- | --- | --- |
| 1 |  | BLK | PCM GROUND |
| 2 |  | LT GRN | CRANK. POS. SENSOR B+ SUPPLY |
| 3 |  | PNK/BLK | INJECTOR 3 CONTROL |
| 4 |  | LT GRN/BLK | INJECTOR 2 CONTROL |
| 5 |  |  | NOT USED |
| 6 |  |  | NOT USED |
| 7 |  |  | NOT USED |
| 8 |  | GRA | TP SENSOR 5V REF |
| 9 |  |  | NOT USED |
| 10 |  |  | NOT USED |
| 11 |  | LT BLU | KNOCK SENSOR REAR |
| 12 |  | DK BLU/WHT | CRANKSHAFT POS. SIGNAL |
| 13 |  |  | NOT USED |
| 14 |  |  | NOT USED |
| 15 |  |  | NOT USED |
| 16 |  |  | NOT USED |
| 17 |  | DK BLU | TRANS. RANGE SIGNAL B |
| 18 |  | RED | TRANS. RANGE SIGNAL C |
| 19 |  | PNK | IGNITION. POS. VOLT. |
| 20 |  | ORG | BAT. POS. VOLT |
| 21 |  | YEL/BLK | CRANK POS. REF. LOW |
| 22 |  |  | NOT USED |
| 23 | remove | GRA | FUEL TANK PRESSURE SENSOR /FUEL TANK SENDER GROUND |
| 24 |  |  | NOT USED |
| 25 | remove | TAN | HO2S SIGNAL LOW BANK 2 SENSOR 2 |
| 26 |  | TAN | HO2S SIGNAL LOW BANK 2 SENSOR 1 |
| 27 |  |  | NOT USED |
| 28 | remove | TAN/WHT | HO2S SIGNAL LOW BANK 1 SENSOR 2 |
| 29 |  | TAN/WHT | HO2S SIGNAL LOW BANK 1 SENSOR 1 |
| 30 |  |  | NOT USED |
| 31 |  |  | NOT USED |
| 32 |  | GRY | CLUTCH PEDAL POSITION. SWITCH SIGNAL |
| 33 |  | PPL | TCC BRAKE SWITCH |
| 34 |  | ORN/BLK | PNP SWITCH SIGNAL |
| 35 |  |  | NOT USED |
| 36 |  | BLK | INJECTOR 1 CONTOL |
| 37 |  | YEL/BLK | INJECTOR 6 CONTROL |
| 38 |  |  | NOT USED |
| 39 |  |  | NOT USED |
| 40 |  | BLK | PCM GROUND |

Information provided by Ls1swap

Formatted by Elmer Boggs	Eboggs_jkvl	7/31/2026

Continued on next page

| Circuit Number C1 (Blue). | Table continued. |
| --- | --- |

| Pin |  | Wire Color | Function |
| --- | --- | --- | --- |
| 41 | remove | BLK | EGR PINTLE POSITION. SENSOR GROUND |
| 42 |  | DK GRN | ENG. COOLING FAN RELAY 1 CONTROL |
| 43 |  | RED/BLK | INJECTOR 7 CONTROL |
| 44 |  | LT BLU/BLK | INJECTOR 4 CONTROL |
| 45 |  | GRA | A/C REFRIG. PRESS. SENSOR 5V REF |
| 46 | remove | GRA | FUEL TANK PRESS. SENSOR 5V REF |
| 47 | remove | GRA | EGR PINTLE POS. SENSOR 5V REF |
| 48 |  | GRA | MAP SENSOR 5V REF |
| 49 |  |  | NOT USED |
| 50 |  |  | NOT USED |
| 51 |  | DK BLU | KNOCK SENSOR SIGNAL FRONT |
| 52 |  |  | NOT USED |
| 53 |  | BLK | TRAN. TEMP. SENSOR GROUND |
| 54 |  | ORG/BLK | MAP SENSOR GROUND |
| 55 |  | BRN | EGR PINTLE POSITION. SENSOR SIGNAL |
| 56 |  |  | NOT USED |
| 57 |  | ORG | BAT. POS. VOLT |
| 58 |  | DRK GRN | SERIAL DATA |
| 59 |  |  | NOT USED |
| 60 |  | BLK | TP SENSOR GROUND |
| 61 |  | PNK/BLK | CAMSHAFT POS. SENSOR REF. LOW |
| 62 |  |  | NOT USED |
| 63 |  |  | NOT USED |
| 64 |  |  | NOT USED |
| 65 | remove | PPL | HO2S SIGNAL HIGH BANK 2SENSOR2 |
| 66 |  | PPL | HO2S SIGNAL HIGH BANK 2 SENSOR 1 |
| 67 |  |  | NOT USED |
| 68 | remove | PPL/WHT | HO2S SIGNAL HIGH BANK1 SENSOR 2 |
| 69 |  | PPL/WHT | HO2S SIGNAL HIGH BANK 1 SENSOR 1 |
| 70 | remove | BRN | LOW OIL LEVEL SWITCH |
| 71 |  |  | NOT USED |
| 72 |  |  | NOT USED |
| 73 |  | BRN/WHT | CAMSHAFT POS. SENSOR SIGNAL |
| 74 |  | YEL | ENG. COOLLANT TEMP.SENSOR SIGNAL |
| 75 |  |  | NOT USED |
| 76 |  | BLK/WHT | INJECTOR 5 CONTROL |
| 77 |  | DK BLU/WHT | INJECTOR 8 CONTROL |
| 78 |  |  | NOT USED |
| 79 |  | GRA OR WHT | SKIP SHIFT SELENOID CNTROL (M/T) OR 2&3 SHIFT SELENOID CONTROL (A/T) |
| 80 |  | BLK | ENG. COLOLANT TEMP. SENSOR GROUND |

Information provided by Ls1swap

Formatted by Elmer Boggs	Eboggs_jkvl	7/31/2026

| Circuit Number C2 (Red) | This table contains all of the 1999 Pin connections, Wire colors, and Functions of Connector C2 (Red). There are 80 pins on this connector |
| --- | --- |

| Pin |  | Wire Color | Function |
| --- | --- | --- | --- |
| 1 |  | BLK | PCM GROUND |
| 2 |  | BRN | TCC CONTROL SOLENOID |
| 3 |  |  | NOT USED |
| 4 | remove | PPL | AIR SELENOID CONTROL |
| 5 |  |  | NOT USED |
| 6 |  | RED/BLK | TRANS. PRESS.CONTROL SELENOID CONTROL HIGH |
| 7 |  | RED | EGR CONTROL |
| 8 |  | LT BLU/WHT | TRANS. PRESS.CONTROL SELENOID CONTROL LOW |
| 9 |  | DK/GRN | FUEL PUMP RELAY CONTROL |
| 10 |  | WHT | ENG. SPEED (TACH) OUTPUT SIGNAL |
| 11 |  |  | NOT USED |
| 12 |  |  | NOT USED |
| 13 |  | WHT | CRUISE CONTROL ENABLE SIGNAL |
| 14 |  | RED/BLK | A/C REFRIG. PRESS. SENSOR SIGNAL |
| 15 |  | RED | ALTERNATOR TERMINAL |
| 16 |  |  | NOT USED |
| 17 |  | DRK GRN/WHT | A/C REQUEST SIGNAL |
| 18 |  | DK GRN | A/C STATUS SIGNAL |
| 19 |  |  | NOT USED |
| 20 |  | LT GRN/BLK | VSS REFERENCE LOW |
| 21 |  |  | ???? Not supplied ???? |
| 22 |  |  | NOT USED |
| 23 |  |  | NOT USED |
| 24 |  | DK BLU | TP SENSOR SIGNAL |
| 25 |  | TAN | IAT SENSOR SIGNAL |
| 26 |  | PPL | INGNITION CONTROL 1 |
| 27 |  | RED | INGNITION CONTROL 7 |
| 28 |  | LT BLU/WHT | INGNITION CONTROL 6 |
| 29 |  | DK GRN/WHT | INGNITION CONTROL 4 |
| 30 |  | DK BLU | VDT FUEL ENABLE SIGNAL |
| 31 |  | YEL | MAF SENSOR SIGNAL |
| 32 |  | LT GRN | MAP SENSOR SIGNAL |
| 33 |  | DK BLU | ENG. COOLING FAN RELAY 2 & 3 CONTROL |
| 34 | remove | DK GRN/WHT | EVAP PURGE CANISTER VALVE CONTROL |
| 35 |  |  | NOT USED |
| 36 | remove | BRN | AIR PUMP RELAY CONTROL |
| 37 |  | DK GRN | CRUISE CONTROL INHIBIT |
| 38 |  |  | NOT USED |
| 39 |  | RED | CAMSHAFT POS. SENSOR B+ SUPPLY |
| 40 |  | BLK | PCM GROUND |

Information provided by Ls1swap

Formatted by Elmer Boggs	Eboggs_jkvl	7/31/2026

Continued on next page

| Circuit Number C2 (Red). | Table continued. |
| --- | --- |

| Pin |  | Wire Color | Function |
| --- | --- | --- | --- |
| 41 |  | GRA | EGR POSITION SENSOR GROUND |
| 42 |  | TAN/BLK | TCC ENABLE CIRCUIT |
| 43 |  | DK GRN/WHT | A/C CLUTCH RELAY CONTROL |
| 44 |  | LT GRN | REV. INHIBIT SOLENOID CONTROL |
| 45 | remove | WHT | EVAP CANISTER VENT VALVE CONTROL |
| 46 |  | BRN/WHT | MALFUNCTION INDICATOR LAMP CONTROL |
| 47 |  | YEL/BLK | TRANMISSION. SHIFT SOLENOID B |
| 48 |  | LT GRN | TRANMISSION. SHIFT SOLENOID A |
| 49 |  |  | NOT USED |
| 50 |  | DK GRN/WHT | VEHICLE SPEED OUTPUT CIRCUIT |
| 51 |  | YEL/BLK | TRANS. TEMP. SENSOR SIGNAL |
| 52 |  |  | NOT USED |
| 53 |  | GRA/BLK | SPARK RETARD SIGNAL |
| 54 | remove | PPL | FUEL LEVEL SENSOR SIGNAL |
| 55 |  |  | NOT USED |
| 56 |  |  | NOT USED |
| 57 |  | PPL | IAT SENSOR GROUND |
| 58 |  |  | NOT USED |
| 59 |  |  | NOT USED |
| 60 |  | BRN | INGNITION CONTROL REFERENCE LOW BANK 1 |
| 61 |  | BRN/WHT | INGNITION CONTROL REFERENCE LOW BANK 2 |
| 62 |  |  | NOT USED |
| 63 |  | PNK | TRANS RANGE SIGNAL A |
| 64 | remove | DK GRN | FUEL TANK PRESSURE SENSOR SIGNAL |
| 65 |  |  | NOT USED |
| 66 |  | PPL/WHT | IGNITION CONTROL 8 |
| 67 |  | RED/WHT | IGNITION CONTROL 2 |
| 68 |  | DK GRN | IGNITION CONTROL 5 |
| 69 |  | LT BLU | IGNITION CONTROL 3 |
| 70 |  |  | NOT USED |
| 71 |  |  | NOT USED |
| 72 |  |  | NOT USED |
| 73 |  |  | NOT USED |
| 74 |  |  | NOT USED |
| 75 |  |  | NOT USED |
| 76 |  | LT GRN/WHT | IAC COIL B HIGH |
| 77 |  | LT GRN/BLK | IAC COIL B LOW |
| 78 |  | LT BLU/BLK | IAC COIL A LOW |
| 79 |  | LT BLU/WHT | IAC COIL A HIGH |
| 80 |  |  | NOT USED |

Information provided by Ls1swap

Formatted by Elmer Boggs	Eboggs_jkvl	7/31/2026