ICAO

Address

City, Country, Code

Technical Manual on the

Universal Access Transceiver (UAT)

Revision 0.6

| Document # | Prepared by: |
| --- | --- |
| DD MM YYYY | © 2003. |

Change Record

| Date/Version | Change |
| --- | --- |
| 15 Jan 2003/v0.1 | Original Draft |
| 17 March 2003/v0.2 | Incorporate changes from review at January Montreal meeting |
| 4 April 2003/v0.3 | Incorporate changes from review at Montreal meeting 31 March – 4 April |
| 6 May 2003/v0.4 | Added material in Section 1, clean up Emitter Category/Call Sign encoding, added sections 5.2.4 and 5.2.5, added text to Section 6 |
| 21 July 2003/v0.5 | Inputs from Brussels meeting: - remove all informative material and any material relating to equipage classes to be presented in the UAT Implementation Manual - create a global requirement concerning rounding of the LSB of a field/remove numerous repetitive notes referring to rounding of LSB - centralize “data timeout” requirement in section 3.1.2.4 - Add text on “Information Frame” definition for WG review. - Changes to Figure 2-1 on Lat/Lon encoding for clarity [to be provided] |
| 16 Sept 2003/v0.6 | Input from 12 Sept 2003 telecon - Add Section 3.1.4 on transceiver T/R switching requirements - Replace Section 4.2 to require suppression output for all classes of equipment |

ICAO

Address

City, Country, Code

Telephone:

Facsimile:

Internet:

Please call ICAO for price and ordering information

FOREWORD

This Page Intentionally Left Blank

TABLE OF CONTENTS

1	Introduction	1

1.1	Outline of the Manual	1

1.2	Objective and Scope	1

1.3	Definitions	1

2	UAT Message Payload	3

2.1	UAT ADS-B Message Payload	3

2.1.1	Payload Type	3

2.1.2	Payload Elements	3

2.1.3	ADS-B Payload Composition by Payload Type Code	3

2.1.4	General Payload Encoding Rules	4

2.1.4.1	Payload Transmission Order	4

2.1.4.2	Truncation of Data Into Payload Fields	4

2.1.5	Payload Contents	5

2.1.5.1	HEADER Element	5

2.1.5.1.1	“PAYLOAD TYPE CODE” Field Encoding	5

2.1.5.1.2	“ADDRESS QUALIFIER” Field Encoding	5

2.1.5.1.3	“ADDRESS” Field Encoding	5

2.1.5.1.3.1	ICAO 24-Bit Aircraft Address of Transmitting Aircraft	6

2.1.5.1.3.2	Reserved for National Use	6

2.1.5.1.3.3	ICAO 24-Bit Aircraft Address of TIS-B Target Aircraft	6

2.1.5.1.3.4	TIS-B Track File Identifier	6

2.1.5.1.3.5	Surface Vehicle Address	7

2.1.5.1.3.6	Fixed ADS-B Beacon Address	7

2.1.5.2	STATE VECTOR Element	7

2.1.5.2.1	“LATITUDE” and “LONGITUDE” Field Encoding	7

2.1.5.2.2	“ALTITUDE TYPE” Field Encoding	10

2.1.5.2.3	“ALTITUDE” Field Encoding	10

2.1.5.2.4	“NIC” Field Encoding	10

2.1.5.2.5	“A/G STATE” Field Encoding	11

2.1.5.2.5.1	Determination of Vertical Status	12

2.1.5.2.5.2	Validation of Vertical Status	13

2.1.5.2.6	“HORIZONTAL VELOCITY” Subfields	14

2.1.5.2.6.1	Encoding as “North Velocity” Form	14

2.1.5.2.6.2	Encoding as “Ground Speed” Form	15

2.1.5.2.6.3	Encoding as “East Velocity” Form	16

2.1.5.2.6.4	Encoding as “Track Angle/Heading” Form	17

2.1.5.2.7	“VERTICAL VELOCITY or A/V LENGTH/WIDTH CODE” Field	18

2.1.5.2.7.1	Encoding as “Vertical Velocity” Form	18

2.1.5.2.7.1.1	“Vertical Velocity Source” Subfield Encoding	18

2.1.5.2.7.1.2	“VV Sign” Subfield Encoding	19

2.1.5.2.7.1.3	“Vertical Rate” Subfield Encoding	19

2.1.5.2.7.2	Encoding as “A/V Length and Width Code” Form	19

2.1.5.2.8	“UTC” Field Encoding	20

2.1.5.2.9	Reserved Bits	20

2.1.5.2.10	Reserved Byte 18 of Payload Type Zero	21

2.1.5.3	STATE VECTOR Element (For TIS-B)	21

2.1.5.3.1	“TIS-B SITE ID” Field Encoding	22

2.1.5.3.2	Encoding for All Other Fields	22

2.1.5.4	MODE STATUS Element	22

2.1.5.4.1	“EMITTER CATEGORY AND CALL SIGN CHARACTERS #1 AND #2” Field	23

2.1.5.4.2	“CALL SIGN CHARACTERS #3, #4 AND #5” Field	24

2.1.5.4.3	“CALL SIGN CHARACTERS #6, #7 AND #8” Field	24

2.1.5.4.4	EMITTER CATEGORY	24

2.1.5.4.5	CALL SIGN	25

2.1.5.4.6	“EMERGENCY/PRIORITY STATUS” Field Encoding	26

2.1.5.4.7	“UAT MANUAL VERSION” Field Encoding	27

2.1.5.4.8	“SIL” Field Encoding	27

2.1.5.4.9	“TRANSMIT MSO” Field Encoding	28

2.1.5.4.10	Reserved Bits	28

2.1.5.4.11	“NACP” Field Encoding	28

2.1.5.4.12	“NACV” Field Encoding	29

2.1.5.4.13	“NICBARO” Field Encoding	29

2.1.5.4.14	“CAPABILITY CODES” Field Encoding	30

2.1.5.4.14.1	“CDTI Traffic Display Capability” Subfield	30

2.1.5.4.14.2	“TCAS/ACAS Installed and Operational” Subfield	30

2.1.5.4.15	“OPERATIONAL MODES” Field Encoding	31

2.1.5.4.15.1	“TCAS/ACAS Resolution Advisory Active” Flag	31

2.1.5.4.15.2	“IDENT Switch Active” Flag	31

2.1.5.4.15.3	“Receiving ATC Services” Flag	32

2.1.5.4.16	“True/Magnetic Heading” Flag	32

2.1.5.4.17	Call Sign Identification (CSID)	32

2.1.5.4.18	Reserved Bits	32

2.1.5.5	AUXILIARY STATE VECTOR Element	32

2.1.5.5.1	“SECONDARY ALTITUDE” Field Encoding	33

2.1.5.5.2	Reserved Bits	33

2.1.5.6	TARGET STATE Element (Payload Type Codes “3” and “4”)	34

2.1.5.6.1	“TARGET HEADING or TRACK ANGLE INFORMATION” Field Encoding	34

2.1.5.6.1.1	“Heading/Track Indicator” Flag Encoding	34

2.1.5.6.1.2	“Target Source Indicator (Horizontal)” Subfield Encoding	35

2.1.5.6.1.3	“Mode Indicator (Horizontal)” Subfield Encoding	35

2.1.5.6.1.4	Reserved Bits	35

2.1.5.6.1.5	“Target Heading or Track Angle” Subfield Encoding	35

2.1.5.6.2	“TARGET ALTITUDE INFORMATION” Field Encoding	36

2.1.5.6.2.1	“Target Altitude Type” Flag Encoding	36

2.1.5.6.2.2	“Target Source Indicator (Vertical)” Subfield Encoding	37

2.1.5.6.2.3	“Mode Indicator (Vertical)” Subfield Encoding	37

2.1.5.6.2.4	“Target Altitude Capability” Subfield Encoding	37

2.1.5.6.2.5	“Target Altitude” Subfield Encoding	38

2.1.5.6.3	TARGET STATE Element (Payload Type Code “6”)	38

2.1.5.7	TRAJECTORY CHANGE Element	39

2.2	UAT Ground Uplink Message Payload	39

2.2.1	UAT-Specific Header	40

2.2.1.1	“GROUND STATION LATITUDE” Field Encoding	40

2.2.1.2	“GROUND STATION LONGITUDE” Field Encoding	40

2.2.1.3	“POSITION VALID” Field Encoding	40

2.2.1.4	“UTC Coupled” Field Encoding	40

2.2.1.5	Reserved Bits	40

2.2.1.6	“APPLICATION DATA VALID” Field Encoding	41

2.2.1.7	“SLOT ID” Field Encoding	41

2.2.1.8	“TIS-B SITE ID” Field Encoding	41

2.2.1.9	Reserved Bits	42

2.2.2	Ground Uplink Application Data	42

2.2.2.1	Information Frames	42

2.2.2.1.1	“Length” subfield encoding	42

2.2.2.1.2	“Reserved” subfield encoding	42

2.2.2.1.3	“Frame Type” subfield encoding	42

2.2.2.1.4	“Frame Data” subfield encoding	42

3	System Timing and Message Transmission Procedures	43

3.1	Avionics	43

3.1.1	Procedures for Processing of Time Data	43

3.1.1.1	UTC Coupled Condition	43

3.1.1.2	Non-UTC Coupled Condition	44

3.1.2	ADS-B Media Access Timing	45

3.1.2.1	The Message Start Opportunity (MSO)	45

3.1.2.2	Relationship of the MSO to the Modulated Data	46

3.1.3	Time Registration and Latency	47

3.1.3.1	Requirements When in Non-Precision Condition and UTC Coupled	47

3.1.3.2	Requirements When in Precision Condition and UTC Coupled	48

3.1.3.3	Requirements When Non-UTC Coupled	48

3.1.3.4	Data Timeout	48

3.1.4	Special Requirements for Transceiver Implementations	49

3.1.4.1	Transmit-Receive Turnaround Time	49

3.1.4.2	Receive-Transmit Turnaround Time	50

3.2	Ground Station	50

3.2.1	Procedures for Processing of Time Data	50

3.2.1.1	UTC Coupled Condition	50

3.2.1.2	Non-UTC Coupled Condition	51

3.2.2	UAT Ground Station Media Access	51

3.2.2.1	Time Slots	51

3.2.2.2	Time Slot Rotation and “Channels”	51

3.2.2.3	Transmission of Ground Uplink Message	52

4	Interface Requirements for the Airborne Equipment	53

4.1	UAT Receiver Output Requirements (Report Generation)	53

4.1.1	Receiver Time of Message Receipt	53

4.1.2	Report Assembly on Receipt of ADS-B Message	53

4.1.3	Report Assembly on Receipt of Ground Uplink Message	54

4.1.4	Message Reception-to-Report Completion Time	54

4.2	Mutual Suppression Pulses	54

TABLE OF FIGURES

Figure 2-1: Angular Weighted Binary Encoding of Latitude and Longitude	9

Figure 3-1: GPS/GNSS Time Mark Pulse	44

Figure 3-2: Relationship of “Channel Numbers” to Time Slot Numbers	52

TABLE OF TABLES

Table 2-1: ADS-B Payload Elements	3

Table 2-2: Composition of UAT ADS-B Message Payload	4

Table 2-3: Encoding of HEADER Element into UAT ADS-B Message Payload	5

Table 2-4: “ADDRESS QUALIFIER” Encoding	5

Table 2-5: Format of STATE VECTOR Element	7

Table 2-6: Angular Weighted Binary Encoding of Latitude and Longitude	8

Table 2-7: “ALTITUDE TYPE” Encoding	10

Table 2-8: “ALTITUDE” Encoding	10

Table 2-9: “NIC” Encoding	11

Table 2-10: “A/G STATE” Field Encoding	12

Table 2-11: Determination of ON-GROUND Condition when there is no means to automatically determine Vertical Status	13

Table 2-12: Criteria for Overriding an ON-GROUND Condition Determined by Automatic Means	14

Table 2-13: “North Velocity” Format	14

Table 2-14: “North/South Sign” Encoding	15

Table 2-15: “North Velocity Magnitude” Encoding	15

Table 2-16: “Ground Speed” Format	15

Table 2-17: “Ground Speed” Encoding	16

Table 2-18: “East Velocity” Format	16

Table 2-19: “East/West Sign” Encoding	16

Table 2-20: “East Velocity Magnitude” Encoding	17

Table 2-21: “Track Angle/Heading” Format	17

Table 2-22: “Track Angle/Heading Type” Encoding	17

Table 2-23: “Track Angle/Heading” Encoding	18

Table 2-24: “Vertical Velocity” Format	18

Table 2-25: “Vertical Velocity Source” Encoding	18

Table 2-26: “Sign Bit for Vertical Rate” Encoding	19

Table 2-27: “Vertical Rate” Encoding	19

Table 2-28: “A/V Length and Width” Format	20

Table 2-29: “Aircraft/Vehicle Length and Width” Encoding	20

Table 2-30: “Position Offset Applied” Encoding	20

Table 2-31: “UTC” Encoding	20

Table 2-32: Format of STATE VECTOR Element (For TIS-B)	21

Table 2-33: Format of MODE STATUS Element	23

Table 2-34: EMITTER CATEGORY Encoding	25

Table 2-35: “Call Sign” Character Encoding	26

Table 2-36: “EMERGENCY/PRIORITY STATUS” Encoding	26

Table 2-37: UAT MANUAL Version Number	27

Table 2-38: “SIL” Encoding.	27

Table 2-39: “NACP” Encoding	28

Table 2-40: “NACV” Encoding	29

Table 2-41: “NICBARO” Encoding	29

Table 2-42: “CAPABILITY CODES” Encoding	30

Table 2-43: “OPERATIONAL MODES” Encoding	31

Table 2-44: Format of AUXILIARY STATE VECTOR Element	33

Table 2-45: Format of TARGET STATE Element (Payload Type Codes “3” and “4”)	34

Table 2-46: “TARGET HEADING or TRACK ANGLE INFORMATION” Format	34

Table 2-47: “Target Source Indicator (Horizontal) Encoding	35

Table 2-48: “Mode Indicator (Horizontal)” Subfield Encoding	35

Table 2-49: “Target Heading or Track Angle” Encoding	36

Table 2-50: “TARGET ALTITUDE INFORMATION” Format	36

Table 2-51: Target Altitude Type Values	37

Table 2-52: “Target Source Indicator (Vertical)” Encoding	37

Table 2-53: “Mode Indicator (Vertical)” Subfield Encoding	37

Table 2-54: “Target Altitude Capability” Encoding	38

Table 2-55: “Target Altitude” Encoding	38

Table 2-56: Format of TARGET STATE Element	39

Table 2-57: Format of the UAT Ground Uplink Message Payload	39

Table 2-58: Encoding of TIS-B Site ID	41

Table 2-59: Format for Information Frames	42

Table 2-60: Frame Type Encoding	42

Table 3-1: UAT ADS-B Transmitter Input Requirements	49

Table 3-1: Time Slot Definition for the UAT Ground Segment	51

This Page Intentionally Left Blank

# Introduction

## Outline of the Manual

This manual contains technical requirements related to the implementation of the Standards and Recommended Practices (SARPs) for the Universal Access Transceiver (UAT).  A companion document, the UAT Implementation Manual, provides additional guidance material of an informative nature.

Section 1 of this document presents the objectives and scope of the manual and provides definition of key terms used in the manual

Section 2 provides details on UAT message payloads and formats.

Section 3 describes procedures for avionics and ground transmitters including requirements for processing timing information

Section 4 describes interface requirements for airborne equipment.

Appendix A provides a set of Acronyms and Definitions of Terms.

## Objective and Scope

The objective of this manual (in conjunction with the SARPs) is to define a set of internationally agreed characteristics of the UAT system that accomplish the following:

- Establish a basis for RF compatibility to other systems from UAT and vice versa.

- Establish a common basis for UAT intersystem interoperability across implementations manufactured and certified in different regions of the world.

This manual alone is not considered adequate for manufacture or certification of UAT equipment and is not a replacement for local certification guidance.

## Definitions

UAT: Universal Access Transceiver (UAT) is a broadcast data link intended to operate globally on a single channel, with a channel signaling rate of 1.041667 Mbps.

UAT ADS-B Message: UAT ADS-B Messages are broadcast by each aircraft once per second to convey state vector and other information.  UAT ADS-B Messages can be in one of two forms depending on the amount of information to be transmitted in a given second: the Basic UAT ADS-B Message or the Long UAT ADS-B Message (see Section 12.4.4.1 of the UAT SARPs for definition of each).

Note: 	The format of ADS-B Messages are specific to each individual ADS-B data link system

UAT Ground Uplink message: The UAT Ground Uplink message is used by ground stations to uplink flight information such as text and graphical weather data, advisories, and other aeronautical information, to any aircraft that may be in the service volume of the ground uplink station message (see Section 12.4.4.2 for further details).

Basic Receiver: A general purpose receiver with less rejection of interference from adjacent channel DMEs than the High Performance receiver.

High Performance Receiver: A UAT receiver with additional filter selectivity to aid in the rejection of adjacent channel DME interference.

Optimum Sampling Point: The optimum sampling point of a received UAT bit stream is at the nominal center of each bit period, when the frequency offset is either plus or minus 312.5 kHz.

Additional Acronyms and Definitions of terms are provided in Appendix A.  [Action #1: Need to insure that all of the listed Acronyms and Definitions are in fact used in the technical manual, and that all Acronyms and Definitions are consistent with ICAO practice.]

# UAT Message Payload

## UAT ADS-B Message Payload

### Payload Type

The UAT ADS-B Message shall contain a “PAYLOAD TYPE CODE” encoded in the first 5 bits of the payload.

Note:	The Payload Type Code allows the receiver to interpret the contents of the UAT ADS-B Message payload per the definition contained in §2.1.2 through §2.1.5.7.

### Payload Elements

The UAT ADS-B Message payload shall be organized into payload elements as shown in Table 2-1.

Note:	These elements contain the individual message fields (e.g., LATITUDE, ALTITUDE, etc) that correspond to the various report elements issued by an UAT ADS-B Receiving Subsystem to an ADS-B application.

Table 2-1: ADS-B Payload Elements

| Payload Element | Payload Bytes | Applicable ADS-B Reports (§5.2.1) | Subparagraph References |
| --- | --- | --- | --- |
| HEADER (HDR) | 4 | All | §2.1.5.1 |
| STATE VECTOR (SV) | 13 | State Vector | §2.1.5.2 §2.1.5.3 |
| MODE STATUS (MS) | 12 | Mode Status | §2.1.5.4 |
| AUX. STATE VECTOR (AUX SV) | 5 | State Vector, Air Reference Velocity | §2.1.5.5 |
| TARGET STATE (TS) | 5 | Target State | §2.1.5.6 §2.1.5.6.3 |
| TRAJECTORY CHANGE + 0 (TC+0) | 12 | Trajectory Change | §2.1.5.7 |
| TRAJECTORY CHANGE + 1 (TC+1) | 12 | Trajectory Change | §2.1.5.7 |

### ADS-B Payload Composition by Payload Type Code

The assignment of payload elements of Table 2-1 to each Payload Type Code shall be as defined in Table 2-2.

Table 2-2: Composition of UAT ADS-B Message Payload

|  | Payload Type Code | UAT ADS-B Message Payload Byte Number |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 ---- 4 | 5 ---- 17 | 18 --------- 24 |  | 25 ---- 28 | 29 |  | 30 --- 33 | 34 |
|  | 0 (Note 1) | HDR | SV | Res | Byte 19-34 Not present in Type 0 |  |  |  |  |  |
|  | 1 | HDR | SV | MS |  |  |  | AUX SV |  |  |
|  | 2 | HDR | SV | Reserved (Note 2) |  |  |  | AUX SV |  |  |
|  | 3 | HDR | SV | MS |  |  |  | TS |  | Res |
|  | 4 | HDR | SV | Reserved for TC+0 (Note 2) |  |  |  | TS |  | Res |
|  | 5 | HDR | SV | Reserved for TC+1 (Note 2) |  |  |  | AUX SV |  |  |
|  | 6 | HDR | SV | Res. (Note 2) |  | TS | Res | AUX SV |  |  |
|  | 7 | HDR | SV |  |  |  |  |  |  |  |
|  | 8 | HDR | SV | Reserved (Note 3) |  |  |  |  |  |  |
|  | 9 | HDR | SV |  |  |  |  |  |  |  |
|  | 10 | HDR | SV |  |  |  |  |  |  |  |
|  | 11 |  |  |  |  |  |  |  |  |  |
|  | through | HDR | Reserved (Note 2) |  |  |  |  |  |  |  |
|  | 29 |  |  |  |  |  |  |  |  |  |
|  | 30, 31 | HDR | Reserved for Developmental Use (Note 4) |  |  |  |  |  |  |  |

Notes:

  - Payload Type 0 is conveyed in the Basic UAT ADS-B Message; byte 18 is reserved for future definition.

  - Not defined in this Manual.  Reserved for definition in future versions.

  - Payload Types 7 – 10 will allow a degree of backward compatibility with future message definition for receivers operating according to this Manual.

  - Payload Types 30 and 31 are intended for developmental use, such as to support on-air flight testing of new payload types, prior to their adoption in future Manual versions.  These payload types should be ignored by operational equipment.

### General Payload Encoding Rules

#### Payload Transmission Order

The UAT ADS-B Message payload shall be transmitted in byte order with byte #1 first.  Within each byte, bits shall be transmitted in order with bit #1 transmitted first.  Bit-level definition of the payload is provided in §2.1.5 through §2.1.5.7.

#### Truncation of Data Into Payload Fields

When converting raw data with more resolution than that required by a payload field, the accuracy of the data shall be maintained such that it is not worse than +/- ½ LSB where the LSB is that of the payload field.

### Payload Contents

#### HEADER Element

Format for the HEADER element is defined in Table 2-3.  This encoding shall apply to UAT ADS-B Messages with PAYLOAD TYPE CODES of “0” through “31.”  Each of the fields shown is defined in §2.1.5.1.1 through §2.1.5.1.3.6.

Table 2-3: Encoding of HEADER Element into UAT ADS-B Message Payload

| Payload Byte # | Bit 1 | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | (MSB) | PAYLOAD TYPE CODE ) |  |  | (LSB) | ADDRESS QUALIFIER |  |  |
| 2 | (MSB)A1 | A2 | A3 | . . . |  |  |  |  |
| 3 | ADDRESS |  |  |  |  |  |  |  |
| 4 |  |  |  |  | . . . | A22 | A23 | A24(LSB) |
|  |  |  |  | . . . |  |  |  |  |

##### “PAYLOAD TYPE CODE” Field Encoding

Definition of the “PAYLOAD TYPE CODE” field encoding that shall be used for all UAT ADS-B Messages is provided in Table 2-2.

Note:	The “PAYLOAD TYPE CODE” field is a 5-bit (bit 1 of byte 1 through bit 5 of byte 1) field used to identify the payload for decoding by the receiver.

##### “ADDRESS QUALIFIER” Field Encoding

Definition of the “ADDRESS QUALIFIER” field encoding that shall be used for all UAT ADS-B Messages is provided in Table 2-4.

Note:	The “ADDRESS QUALIFIER” field is a 3-bit (bit 6 of byte 1 through bit 8 of byte 1) field used to indicate what the 24-bit “ADDRESS” field represents.

Table 2-4: “ADDRESS QUALIFIER” Encoding

| Address Qualifier (binary) |  |  | Address Qualifier (decimal) | Address Type | Reference subparagraph |
| --- | --- | --- | --- | --- | --- |
| Bit 6 | Bit 7 | Bit 8 |  |  |  |
| 0 | 0 | 0 | 0 | ADS-B target with ICAO 24-bit address | §2.1.5.1.3.1 |
| 0 | 0 | 1 | 1 | Reserved for National use | §2.1.5.1.3.2 |
| 0 | 1 | 0 | 2 | TIS-B target with ICAO 24-bit address | §2.1.5.1.3.3 |
| 0 | 1 | 1 | 3 | TIS-B target with track file identifier | §2.1.5.1.3.4 |
| 1 | 0 | 0 | 4 | Surface Vehicle | §2.1.5.1.3.5 |
| 1 | 0 | 1 | 5 | Fixed ADS-B Beacon | §2.1.5.1.3.6 |
| 1 | 1 | 0 | 6 | (Reserved) |  |
| 1 | 1 | 1 | 7 | (Reserved) |  |

##### “ADDRESS” Field Encoding

The meaning of the “ADDRESS” field shall depend on the “ADDRESS QUALIFIER” field as described in §2.1.5.1.3.1 through §2.1.5.1.3.6.

Note:	The “ADDRESS” field is a 24-bit (bit 1 of byte 2 through bit 8 of byte 4) field used in conjunction with the “ADDRESS QUALIFIER” field to identify the participant.

###### ICAO 24-Bit Aircraft Address of Transmitting Aircraft

An “ADDRESS QUALIFIER” value of ZERO (binary 000) shall indicate that the message is an UAT ADS-B Message from an aircraft, and that the “ADDRESS” field holds the ICAO 24-bit address that has been assigned to that particular aircraft.  The ICAO Aircraft Address shall be stored (or “latched”) in the UAT Transmitting System upon Power Up.

The UAT ADS-B Transmitting Subsystem shall declare a device failure in the event that its own ICAO 24-bit Address  (i.e., the Mode-S Address) is invalid, unavailable, or set to all “ZEROs” or all “ONEs.”

Note:	The world-wide method for allocating and assigning the 24-bit ICAO aircraft addresses is described in Annex 10 to the Convention on International Civil Aviation, Volume III, Chapter 9. [ICAO Annex 10, Vol. III, Ch. 9].

###### Reserved for National Use

An “ADDRESS QUALIFIER” value of ONE (binary 001) shall be reserved for National use.

Note	 In one State the use of such a value will indicate that the message is an UAT ADS-B Message from an aircraft, and that the “ADDRESS” field holds the transmitting aircraft’s self-assigned ownship temporary address.

###### ICAO 24-Bit Aircraft Address of TIS-B Target Aircraft

An “ADDRESS QUALIFIER” value of TWO (binary 010) shall be used by a UAT Ground Station providing TIS-B uplinks in the UAT ADS-B Message Format to indicate that the message is for a TIS-B target and the “ADDRESS” field holds the ICAO 24-bit address that has been assigned to the target aircraft being described in the message.

Note:	The world-wide scheme for allocating and assigning the 24-bit ICAO aircraft addresses is described in Annex 10 to the Convention on International Civil Aviation, Volume III, Chapter 9. [ICAO Annex 10, Vol. III, Ch. 9].

###### TIS-B Track File Identifier

An “ADDRESS QUALIFIER” value THREE (binary 011) shall be used by a UAT Ground Station providing TIS-B uplinks in the UAT ADS-B Message Format to indicate that the message is for a TIS-B target and that the “ADDRESS” field holds a TIS-B track file identifier by which the TIS-B data source identifies the target aircraft being described in the message.

Note:	It is beyond the scope of this Manual to specify the method by which a TIS-B service provider would assign track file identifiers for those TIS-B targets for which the ICAO 24-bit address is unknown.

###### Surface Vehicle Address

An “ADDRESS QUALIFIER” value of FOUR (binary 100) shall be used by the UAT Transmitting Subsystem of a surface vehicle to indicate that the “ADDRESS” field holds the address of a surface vehicle authorized to transmit UAT ADS-B Messages.

Note:	It is beyond the scope of this Manual to specify the method by which UAT ADS-B surface vehicle addresses are assigned.

###### Fixed ADS-B Beacon Address

An “ADDRESS QUALIFIER” value of FIVE (binary 101) shall be used to indicate that the “ADDRESS” field holds the address assigned to a fixed UAT ADS-B beacon or “parrot.”

Note:	It is beyond the scope of this Manual to specify the method by which UAT ADS-B beacon addresses are assigned.

#### STATE VECTOR Element

Format for the STATE VECTOR element shall be as defined in Table 2-5.  This encoding shall apply to UAT ADS-B Messages with PAYLOAD TYPE CODES of “0” through “10,” when the ADDRESS QUALIFIER value is “0,” “4” or “5.”

Note:	Each of the fields shown is defined in §2.1.5.2.1 through §2.1.5.2.10.

Table 2-5: Format of STATE VECTOR Element

| Payload Byte # | Bit 1 | Bit 2 | Bit 3 |  | Bit 4 |  | Bit 5 |  | Bit 6 | Bit 7 | Bit 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | . |  |  |  |  |  |  |
| 5 | (MSB) |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  | LATITUDE (WGS-84) |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  | (LSB) | (MSB) |
| 8 |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  | LONGITUDE (WGS-84) |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  | (LSB) | Alt Type |
| 11 | (MSB) | ALTITUDE |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  | (LSB) |  |  | (MSB) | NIC |  | (LSB) |
| 13 | (MSB) A/G STATE (LSB) |  |  | Reserved |  |  |  |  |  |  |  |
| 14 | HORIZONTAL VELOCITY |  |  |  |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  | VERTICAL VELOCITY or A/V LENGTH/WIDTH CODE |  |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  | UTC |  | Reserved |  |  |
|  |  |  |  |  | . . |  |  |  |  |  |  |

##### “LATITUDE” and “LONGITUDE” Field Encoding

- The “LATITUDE” field is a 23-bit (bit 1 of byte 5 through bit 7 of byte 7) field used to encode the latitude provided to the UAT ADS-B Transmitting Subsystem in WGS-84 and shall be encoded as indicated in Table 2-6.

- The “LONGITUDE” field is a 24-bit (bit 8 of byte 7 through bit 7 of byte 10) field used to encode the longitude provided to the UAT ADS-B Transmitting Subsystem in WGS-84 and shall be encoded as indicated in Table 2-6.

- The encoding of ALL ZEROs in the “LATITUDE” and “LONGITUDE” and “NIC” (§2.1.5.2.4) fields shall indicate that Latitude/Longitude information is “unavailable.”

Note:	Figure 2-1 provides further guidance on the angular weighted binary encoding of Latitude and Longitude.

Note:	Since the encoding of ALL ZEROs is a valid location on the earth, UAT ADS-B Receiving Subsystems will interpret this as Latitude/Longitude information “unavailable” only if the NIC field is also set to ZERO.

Table 2-6: Angular Weighted Binary Encoding of Latitude and Longitude

| Quadrant |  | “LATITUDE” or “LONGITUDE” bits |  | Meaning LSB=360224=2.145767x10−5° |  |
| --- | --- | --- | --- | --- | --- |
|  |  | MSB | LSB | Latitude | Longitude |
|  |  | 0000 0000 0000 0000 0000 0000 |  | ZERO degrees (Equator) | ZERO degrees (Prime Meridian) |
| 1st |  | 0000 0000 0000 0000 0000 0001 |  | LSB degrees North | LSB degrees East |
| quadrant | … |  |  | … | … |
|  |  | 0011 1111 1111 1111 1111 1111 |  | (90-LSB) degrees North | (90-LSB) degrees East |
|  |  | 0100 0000 0000 0000 0000 0000 |  | 90 degrees (North Pole) | 90 degrees East |
| 2nd |  | 0100 0000 0000 0000 0000 0001 |  | <Illegal Values> | (90+LSB) degrees East |
| quadrant |  | … |  | <Illegal Values> | … |
|  |  | 0111 1111 1111 1111 1111 1111 |  | <Illegal Value> | (180-LSB) degrees East |
|  |  | 1000 0000 0000 0000 0000 0000 |  | <Illegal Value> | 180 degrees East or West |
| 3rd |  | 1000 0000 0000 0000 0000 0001 |  | <Illegal Value> | (180-LSB) degrees West |
| quadrant |  | … |  | <Illegal Values> | … |
|  |  | 1011 1111 1111 1111 1111 1111 |  | <Illegal Values> | (90-LSB) degrees West |
|  |  | 1100 0000 0000 0000 0000 0000 |  | -90 degrees (South Pole) | 90 degrees West |
| 4th |  | 1100 0000 0000 0000 0000 0001 |  | (90-LSB) degrees South | (90-LSB) degrees West |
| quadrant |  | … |  | … | … |
|  |  | 1111 1111 1111 1111 1111 1111 |  | LSB degrees South | LSB degrees West |

Notes:

- The most significant bit (MSB) of the angular weighted binary “LATITUDE” is omitted from the transmitted message.  This is because all valid Latitudes, other than the Latitude of the North pole (exactly 90 degrees North), have the same value in their 2 most significant bits.  The application using the ADS-B reports has the responsibility to differentiate the North and South Poles.

Figure 2-1: Angular Weighted Binary Encoding of Latitude and Longitude

##### “ALTITUDE TYPE” Field Encoding

- The “ALTITUDE TYPE” field is a 1-bit (bit 8 of byte 10) field used to identify the source of information in the “ALTITUDE” field and shall be encoded as reflected in Table 2-7.

Table 2-7: “ALTITUDE TYPE” Encoding

| Altitude Type | “ALTITUDE” Field (§2.1.5.2.3) | “SECONDARY ALTITUDE” Field (§2.1.5.5.1) |
| --- | --- | --- |
| 0 | Pressure Altitude | Geometric Altitude |
| 1 | Geometric Altitude | Pressure Altitude |

Note:	“Pressure Altitude” refers to “Barometric Pressure Altitude” relative to a standard atmosphere at a standard pressure of 1013.25 hectopascals (29.92 in Hg) and specifically DOES NOT refer to “Barometric Corrected Altitude.”

- A means shall be provided to operationally inhibit the broadcast of Pressure Altitude information, making it unavailable for transmission.

- A means shall be provided to operationally select the preferred ALTITUDE TYPE that is reported if more than one ALTITUDE TYPE is available.  If only one ALTITUDE TYPE is available, then that Altitude shall be indicated in the “ALTITUDE TYPE” field.

Note:	The means to operationally inhibit the broadcast of pressure altitude information can be used at the request of ATC, or when altitude is determined to be invalid by the pilot.

##### “ALTITUDE” Field Encoding

The “ALTITUDE” field is a 12-bit (bit 1 of byte 11 through bit 4 of byte 12) field used to encode the altitude of the ADS-B Transmitting Subsystem and shall be encoded as indicated in Table 2-8.

Table 2-8: “ALTITUDE” Encoding

| Coding (binary) MSB LSB | Coding (decimal) | Meaning |
| --- | --- | --- |
| 0000 0000 0000 | 0 | Altitude information unavailable |
| 0000 0000 0001 | 1 | Altitude = -1000 feet |
| 0000 0000 0010 | 2 | Altitude = -975 feet |
| … | … | … |
| 0000 0010 1000 | 40 | Altitude = -25 feet |
| 0000 0010 1001 | 41 | Altitude = ZERO feet |
| 0000 0010 1010 | 42 | Altitude = 25 feet |
| … | … | … |
| 1111 1111 1110 | 4094 | Altitude = 101,325 feet |
| 1111 1111 1111 | 4095 | Altitude > 101,337.5 feet |

##### “NIC” Field Encoding

The Navigation Integrity Category (“NIC”) field is a 4-bit (bits 5 through 8 of byte 12) field used to allow surveillance applications to determine whether the reported position has an acceptable level of integrity for the intended use and shall be encoded as indicated in Table 2-9.

Note:	The value of the NIC parameter specifies an integrity containment radius, RC.

Table 2-9: “NIC” Encoding

| NIC (binary) | NIC (decimal) | Horizontal and Vertical Containment Bounds | Comment |
| --- | --- | --- | --- |
| MSB … LSB |  |  |  |
| 0000 | 0 | RC  37.04 km (20 NM) | Unknown Integrity |
| 0001 | 1 | RC < 37.04 km (20 NM) | RNP-10 containment radius |
| 0010 | 2 | RC < 14.816 km (8 NM) | RNP-4 containment radius |
| 0011 | 3 | RC < 7.408 km (4 NM) | RNP-2 containment radius |
| 0100 | 4 | RC < 3.704 km (2 NM) | RNP-1 containment radius |
| 0101 | 5 | RC < 1852 m (1 NM) | RNP-0.5 containment radius |
| 0110 | 6 | RC < 1111.2 m (0.6 NM) | RNP-0.3 containment radius |
| 0111 | 7 | RC < 370.4 m (0.2 NM) | RNP-0.1 containment radius |
| 1000 | 8 | RC < 185.2 m (0.1 NM) | RNP-0.05 containment radius |
| 1001 | 9 | RC < 75 m and VPL < 112 m | e.g., SBAS, HPL, VPL |
| 1010 | 10 | RC < 25 m and VPL < 37.5 m | e.g., SBAS, HPL, VPL |
| 1011 | 11 | RC < 7.5 m and VPL < 11 m | e.g., GBAS, HPL, VPL |
| 1100 | 12 | (Reserved) | (Reserved) |
| 1101 | 13 | (Reserved) | (Reserved) |
| 1110 | 14 | (Reserved) | (Reserved) |
| 1111 | 15 | (Reserved) | (Reserved) |

Notes:

- The “NIC” field is closely associated with the “SIL” field (defined in §2.1.5.4.8).  The value of the “SIL” field is the probability of the true position lying outside the containment radius, RC, without alerting, including the effects of airborne equipment condition, which airborne equipment is in use, and which external signals are used.

- VPL refers to the Vertical Protection Limit, which is an indication of the vertical interval within which the actual position of the transmitting participant lies, to a probability of 1 minus 10-7.  HPL refers to the Horizontal Protection Limit.

##### “A/G STATE” Field Encoding

The Air/Ground State (“A/G STATE”) field is a 2-bit (bits 1 and 2 of byte 13) field that indicates the format used for representing horizontal velocity.  The value of this field determines the encoding of the “HORIZONTAL VELOCITY” field.  The “A/G STATE” shall be composed of two (2) 1-bit fields used as follows (see also Table 2-10):

- The Vertical Status bit (bit 1 of byte 13) shall be used to reflect the AIRBORNE or ON-GROUND condition as determined in §2.1.5.2.5.1.

- The Subsonic/Supersonic bit (bit 2 of byte 13) shall be used to indicate the scale factor for the velocity information. The Subsonic/Supersonic bit (bit 2 of byte 13) shall only be set to ONE (1) if either the East – West velocity OR the North – South velocity, exceeds 1022 knots.  The Subsonic/Supersonic bit (bit 2 of byte 13) shall be reset to ZERO (0) if the East - West and the North - South velocities, drop below 1000 knots.

Table 2-10: “A/G STATE” Field Encoding

| Ownship Conditions | “A/G STATE” Field Encoding |  |  | Resulting “HORIZONTAL VELOCITY” Subfield Formats |  |
| --- | --- | --- | --- | --- | --- |
|  | MSB LSB |  |  |  |  |
|  | Vertical Status (bit 1 of byte 13) | Subsonic/Supersonic (bit 2 of byte 13) | (decimal) | “North Velocity or Ground Speed” Subfield Meaning | “East Velocity or Track Angle/Heading” Subfield Meaning |
| AIRBORNE condition. Subsonic condition. | 0 | 0 | 0 | North Velocity (LSB = 1 kt) | East Velocity (LSB = 1 kt) |
| AIRBORNE condition. Supersonic condition. | 0 | 1 | 1 | North Velocity (LSB = 4 kts) | East Velocity (LSB = 4 kts) |
| ON GROUND condition. | 1 | 0 | 2 | Ground Speed (LSB = 1 kt) | Track/Heading |
| <Reserved> | 1 | 1 | 3 |  |  |

###### Determination of Vertical Status

The UAT ADS-B Transmitting Subsystem shall determine its Vertical Status (i.e., AIRBORNE or ON-GROUND condition) using the procedure below.

- If there is a means to automatically determine the Vertical Status of the ADS-B emitter target category, then such information shall be used to determine the Vertical Status.

Note:	An “automatic” means of determining vertical status could come from a weight-on-wheels or strut switch, etc.  Landing gear deployment is not considered a suitable automatic means.

- If there is no means to automatically determine the Vertical Status of the UAT ADS-B Transmitting Subsystem, then the UAT ADS-B Transmitting Subsystem shall assume the AIRBORNE condition except under the conditions given for each of the UAT ADS-B Emitter Category types given in Table 2-11.  If the conditions given in Table 2-11 are met for the given UAT ADS-B Emitter Category, then the UAT ADS-B Transmitting Subsystem shall be in the ON-GROUND condition.

Table 2-11: Determination of ON-GROUND Condition
when there is no means to automatically determine Vertical Status

| Emitter Category | Ground Speed |  |  | Airspeed (if available) |  | Radio Altitude (if available) |
| --- | --- | --- | --- | --- | --- | --- |
| No aircraft type information | Always declare AIRBORNE condition |  |  |  |  |  |
| Light (ICAO) < 15,500 lbs | Always declare AIRBORNE condition |  |  |  |  |  |
| Small – 15,500 to 75,000 lbs | < 100 knots |  | or | < 100 knots | or | < 50 feet |
| Large – 75,000 to 300,000 lbs | < 100 knots |  | or | < 100 knots | or | < 50 feet |
| High Vortex Large (e.g., B757) | < 100 knots |  | or | < 100 knots | or | < 50 feet |
| Heavy (ICAO) - > 300,000 lbs | < 100 knots |  | or | < 100 knots | or | < 50 feet |
| Highly Maneuverable > 5G acceleration and high speed | < 100 knots |  | or | < 100 knots | or | < 50 feet |
| Rotocraft | Always declare AIRBORNE condition (See Note 1) |  |  |  |  |  |
| Glider/sailplane | Always declare AIRBORNE condition |  |  |  |  |  |
| Lighter than air | Always declare AIRBORNE condition (See Note 2) |  |  |  |  |  |
| Parachutist/sky diver | Always declare AIRBORNE condition |  |  |  |  |  |
| Ultra light/hang glider/paraglider | Always declare AIRBORNE condition |  |  |  |  |  |
| Unmanned aerial vehicle | Always declare AIRBORNE condition |  |  |  |  |  |
| Space/trans-atmospheric vehicle | < 100 knots |  | or | < 100 knots | or | < 50 feet |
| Surface vehicle—emergency vehicle | Always declare ON-GROUND condition |  |  |  |  |  |
| Surface vehicle—service vehicle | Always declare ON-GROUND condition |  |  |  |  |  |
| Point Obstacle (includes tethered balloons) | (See Note 3) |  |  |  |  |  |
| Cluster Obstacle |  |  |  |  |  |  |
| Line Obstacle |  |  |  |  |  |  |

Action #2: Complete consultation with RTCA SC186 on logic within Table (an identical table will be used in the Mode S Manual of Specific Services maintained by SCRSP).

Notes:

- Because of the unique operating capabilities of rotorcraft, i.e., hover, etc., an operational rotorcraft will always report the AIRBORNE condition unless the ON-GROUND condition is specifically declared in compliance with subparagraph “a.” above.

- Because of the unique operating capabilities of “Lighter-than-Air” vehicles, e.g., balloons, an operational “Lighter-than-Air” vehicle will always report the AIRBORNE condition unless the ON-GROUND condition is specifically declared in compliance with subparagraph “a.” above.

- The Vertical Status reported will be appropriate to the situation.  In any case the altitude is always present in the transmitted message.

###### Validation of Vertical Status

When an automatic means of determining Vertical Status indicates ON-GROUND, the Vertical Status shall be changed to AIRBORNE under the conditions listed in Table 2-12.

Note:	The Vertical Status can be used by UAT ADS-B Transmitting Subsystems to select only the TOP antenna when in the ON-GROUND condition.  A false indication of the automatic means could therefore impact signal availability.  To minimize this possibility, this validation procedure has been established.

Table 2-12: Criteria for Overriding an ON-GROUND Condition Determined by Automatic Means

| Emitter Category | Ground Speed |  | Airspeed (if available) |  | Radio Altitude (if available) |
| --- | --- | --- | --- | --- | --- |
| No aircraft type information | No Change to condition |  |  |  |  |
| Light (ICAO) < 15,500 lbs | No Change to condition |  |  |  |  |
| Small – 15,500 to 75,000 lbs | > 100 knots | or | > 100 knots | or | > 50 feet |
| Large – 75,000 to 300,000 lbs | > 100 knots | or | > 100 knots | or | > 50 feet |
| High Vortex Large (e.g., B-757) | > 100 knots | or | > 100 knots | or | > 50 feet |
| Heavy (ICAO) - > 300,000 lbs | > 100 knots | or | > 100 knots | or | > 50 feet |
| Highly Maneuverable > 5G acceleration and high speed | > 100 knots | or | > 100 knots | or | > 50 feet |
| Rotocraft | No Change to condition |  |  |  |  |

##### “HORIZONTAL VELOCITY” Subfields

The “HORIZONTAL VELOCITY” Field shall be composed of two components:

- The “North Velocity or Ground Speed” component shall be represented by an 11-bit subfield from bit 4 of byte 13 through bit 6 of byte 14.

- The “East Velocity or Track/Heading” component shall be represented by an 11-bit subfield from bit 7 of byte 14 through bit 1 of byte 16.

Note:	Each component can assume multiple formats depending on the “A/G STATE” field.  Subparagraphs §2.1.5.2.6.1 through §2.1.5.2.6.4 describe the encoding for each form of each component.

###### Encoding as “North Velocity” Form

When the “A/G STATE” field is set to “0,” or “1,” the “North Velocity or Ground Speed” component shall assume the “North Velocity” format indicated in Table 2-13.

Table 2-13: “North Velocity” Format

| Byte 13 |  |  |  |  | Byte 14 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 | Bit 1 | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 |  |
| N/S Sign | (MSB) | --North Velocity Magnitude-- |  |  |  |  |  |  |  |  | (LSB) |

- The “N/S Sign” subfield (bit 4 of byte 13) shall be used to indicate the direction of the North/South velocity vector as shown in Table 2-14.

Table 2-14: “North/South Sign” Encoding

| Coding | Meaning |
| --- | --- |
| 0 | NORTH |
| 1 | SOUTH |

- The “North Velocity Magnitude” subfield is a 10-bit (bit 5 of byte 13 through bit 6 of byte 14) subfield that shall be used to report the magnitude of the North/South velocity of the UAT ADS-B Transmitting Subsystem.  The Range, Resolution and No Data encoding of the “North Velocity Magnitude” subfield shall be as shown in Table 2-15.

Table 2-15: “North Velocity Magnitude” Encoding

| Coding MSB(binary)LSB | Coding (decimal) | Meaning (Subsonic Scale) (A/G State = 0) | Meaning (Supersonic Scale) (A/G State = 1) |
| --- | --- | --- | --- |
| 00 0000 0000 | 0 | N/S Velocity not available | N/S Velocity not available |
| 00 0000 0001 | 1 | N/S Velocity is ZERO | N/S Velocity is ZERO |
| 00 0000 0010 | 2 | N/S Velocity = 1 knots | N/S Velocity = 4 knots |
| 00 0000 0011 | 3 | N/S Velocity = 2 knots | N/S Velocity = 8 knots |
| … | … | … | … |
| 11 1111 1110 | 1022 | N/S Velocity = 1021 knots | N/S Velocity = 4,084 knots |
| 11 1111 1111 | 1023 | N/S Velocity > 1021.5 knots | N/S Velocity > 4,086 knots |

Note:	The encoding represents Positive Magnitude data only.  Direction is given completely by the N/S Sign Bit.

###### Encoding as “Ground Speed” Form

When the “A/G STATE” field is set to “2,” the “North Velocity or Ground Speed” component shall assume the “Ground Speed” format indicated in Table 2-16.

Table 2-16: “Ground Speed” Format

| Byte 13 |  |  |  |  |  | Byte 14 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 4 | Bit 5 | Bit 6 | Bit 7 |  | Bit 8 | Bit 1 | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 |  |
| Reserved | (MSB) |  |  | --Ground Speed-- |  |  |  |  |  |  |  | (LSB) |

- The 1-bit subfield (bit 4 of byte 13) shall be “Reserved” and set to ZERO (0).

- The “Ground Speed” subfield is a 10-bit (bit 5 of byte 13 through bit 6 of byte 14) subfield that shall be used to report the Ground Speed of the UAT ADS-B Transmitting Subsystem (in knots).  The Range, Resolution and No Data encoding of the “Ground Speed” subfield shall be as shown in Table 2-17.

Table 2-17: “Ground Speed” Encoding

| Coding MSB(binary)LSB | Coding (decimal) | Meaning (A/G State = 2) |
| --- | --- | --- |
| 00 0000 0000 | 0 | Ground Speed information not available |
| 00 0000 0001 | 1 | Ground Speed is ZERO |
| 00 0000 0010 | 2 | Ground Speed = 1 knots |
| 00 0000 0011 | 3 | Ground Speed = 2 knots |
| … | … | … |
| 11 1111 1110 | 1022 | Ground Speed = 1021 knots |
| 11 1111 1111 | 1023 | Ground Speed > 1021.5 knots |

###### Encoding as “East Velocity” Form

When the “A/G STATE” field is set to “0” or “1,” the “East Velocity or Track Angle/Heading” component shall assume the “East Velocity” format indicated in Table 2-18.

Table 2-18: “East Velocity” Format

| Byte 14 |  | Byte 15 |  |  |  |  |  |  |  | Byte 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 7 | Bit 8 | Bit 1 | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 | Bit 1 |
| E/W Sign | (MSB) | --East Velocity Magnitude-- |  |  |  |  |  |  |  | (LSB) |

- The “E/W Sign” subfield (bit 7 of byte 14) shall be used to indicate the direction of the East/West velocity vector as shown in Table 2-19.

Table 2-19: “East/West Sign” Encoding

| Coding | Meaning |
| --- | --- |
| 0 | EAST |
| 1 | WEST |

- The “East Velocity Magnitude” subfield is a 10-bit (bit 8 of byte 14 through bit 1 of byte 16) subfield that shall be used to report the East/West velocity of the UAT ADS-B Transmitting Subsystem (in knots).  The Range, Resolution and No Data encoding of the “East Velocity Magnitude” subfield shall be as shown in Table 2-20.

Table 2-20: “East Velocity Magnitude” Encoding

| Coding MSB(binary)LSB | Coding (decimal) | Meaning (Subsonic Scale) (A/G State = 0) | Meaning (Supersonic Scale) (A/G State = 1) |
| --- | --- | --- | --- |
| 00 0000 0000 | 0 | E/W Velocity not available | E/W Velocity not available |
| 00 0000 0001 | 1 | E/W Velocity is ZERO | E/W Velocity is ZERO |
| 00 0000 0010 | 2 | E/W Velocity = 1 knots | E/W Velocity = 4 knots |
| 00 0000 0011 | 3 | E/W Velocity = 2 knots | E/W Velocity = 8 knots |
| … | … | … | … |
| 11 1111 1110 | 1022 | E/W Velocity = 1021 knots | E/W Velocity = 4,084 knots |
| 11 1111 1111 | 1023 | E/W Velocity > 1021.5 knots | E/W Velocity > 4,086 knots |

Note:	The encoding represents Positive Magnitude data only.  Direction is given completely by the E/W Sign Bit.

###### Encoding as “Track Angle/Heading” Form

When the “A/G STATE” field is set to “2” the “East Velocity or Track Angle/Heading” component shall assume the “Track Angle/Heading” format indicated in Table 2-21.  Heading shall be encoded if available; if not available Track Angle shall be encoded.

Table 2-21: “Track Angle/Heading” Format

| Byte 14 |  | Byte 15 |  |  |  |  |  |  |  | Byte 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 7 | Bit 8 | Bit 1 | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 | Bit 1 |
| TA/H Type |  | (MSB) |  | --Track Angle/Heading-- |  |  |  |  |  | (LSB) |

- The Track Angle/Heading Type (“TA/H Type”) is a 2-bit subfield (bit 7 and 8 of byte 14) that shall be used to distinguish Track Angle from Heading as shown in Table 2-22.

Table 2-22: “Track Angle/Heading Type” Encoding

| Coding | Meaning |
| --- | --- |
| 00 | Data Not Available |
| 01 | True Track Angle |
| 10 | Magnetic Heading |
| 11 | True Heading |

- The “Track Angle/Heading” subfield is a 9-bit (bit 1 of byte 15 through bit 1 of byte 16) subfield that shall be used to report the Track Angle or Heading of the ADS-B Transmitting Subsystem as shown in Table 2-23.

Table 2-23: “Track Angle/Heading” Encoding

| Coding MSB(binary)LSB | Coding (decimal) | Meaning |
| --- | --- | --- |
| 0 0000 0000 | 0 | Track Angle/Heading is ZERO |
| 0 0000 0001 | 1 | Track Angle/Heading = 0.703125 degrees |
| 0 0000 0010 | 2 | Track Angle/Heading = 1.406250 degrees |
| 0 0000 0011 | 3 | Track Angle/Heading = 2.109375 degrees |
| … | … | … |
| 1 1111 1110 | 510 | Track Angle/Heading = 358.593750 degrees |
| 1 1111 1111 | 511 | Track Angle/Heading = 359.296875 degrees |

##### “VERTICAL VELOCITY or A/V LENGTH/WIDTH CODE” Field

###### Encoding as “Vertical Velocity” Form

When the ADS-B Transmitting Subsystem is in the AIRBORNE condition, the format for the “VERTICAL VELOCITY or Aircraft/Vehicle (A/V) SIZE” field shall assume the “Vertical Velocity” form as shown in Table 2-24.

Table 2-24: “Vertical Velocity” Format

| Byte 16 |  |  |  |  |  |  | Byte 17 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 | Bit 1 | Bit 2 | Bit 3 | Bit 4 |
| VV Src | VV Sign | (MSB) | --Vertical Rate-- |  |  |  |  |  |  | (LSB) |

“Vertical Velocity Source” Subfield Encoding

The Vertical Velocity Source (“VV Src”) subfield is a 1-bit (bit 2 of byte 16) field that shall be used to indicate the source of Vertical Rate information as defined in Table 2-25.

Table 2-25: “Vertical Velocity Source” Encoding

| Coding | Meaning |
| --- | --- |
| 0 | Vertical Rate information from Geometric Source (GNSS or INS) |
| 1 | Vertical Rate information from Barometric Source |

Vertical rate information shall come from a Geometric source when the Precision condition is met, specifically when:

  - The “NACP” (§2.1.5.4.11) value is “10” or “11,” or

  - The “NIC” value is “9,” “10” or “11”

Otherwise, the Non-Precision condition is in effect and Vertical Rate information shall come from a barometric source, if available.

“VV Sign” Subfield Encoding

The Sign Bit for Vertical Rate (“VV Sign”) subfield is a 1-bit (bit 3 of byte 16) field used to indicate the direction of the “Vertical Rate” subfield and shall be encoded as indicated in Table 2-26.

Table 2-26: “Sign Bit for Vertical Rate” Encoding

| Coding | Meaning |
| --- | --- |
| 0 | UP |
| 1 | DOWN |

“Vertical Rate” Subfield Encoding

- The “Vertical Rate” subfield is a 9-bit (bit 4 of byte 16 through bit 4 of byte 17) field  used to report the Vertical Rate (in feet/minute) of the UAT ADS-B transmission device where the Range, Resolution, and No Data encoding of the “Vertical Rate” subfield shall be encoded as shown in Table 2-27.

Table 2-27: “Vertical Rate” Encoding

| Coding MSB(binary)LSB | Coding (decimal) | Meaning |
| --- | --- | --- |
| 0 0000 0000 | 0 | No Vertical Rate information available |
| 0 0000 0001 | 1 | Vertical Rate is ZERO |
| 0 0000 0010 | 2 | Vertical Rate = 64 feet / minute |
| 0 0000 0011 | 3 | Vertical Rate = 128 feet / minute |
| … | … | … |
| 1 1111 1110 | 510 | Vertical Rate = 32,576 feet / minute |
| 1 1111 1111 | 511 | Vertical Rate > 32,608 feet / minute |

Notes:

- The encoding shown represents Positive Magnitude data only.  Direction is given completely by the VV Sign Subfield.

- For codes “0” and “1,” the VV Sign Subfield is encoded as ZERO.

###### Encoding as “A/V Length and Width Code” Form

When the UAT ADS-B Transmitting Subsystem is in the ON-GROUND condition, the “VERTICAL VELOCITY or A/V Length and Width Code” field shall assume the “A/V Length and Width Code” form as shown in Table 2-28.

The encoding of the “A/V Length and Width Code” shall be as shown in Table 2-29, using the rule that the smallest code for which the A/V satisfies both the Length Category inequality and the Width Category inequality will be used.

The “Position Offset Applied” (POA) flag indicates whether the reported position reflects application of a position offset to normalize the ownship navigation sensor position to the ADS-B reference point and shall be encoded as indicated in Table 2-30.

Table 2-28: “A/V Length and Width” Format

| Byte 16 |  |  |  |  |  |  |  | Byte 17 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 |  | Bit 7 | Bit 8 | Bit 1 | Bit 2 | Bit 3 | Bit 4 |
| A/V Length and Width |  |  |  | POA | Reserved |  |  |  |  |  |  |

Table 2-29: “Aircraft/Vehicle Length and Width” Encoding

| A/V - L/W Code (decimal) | Length Code |  |  | Width Code | Length Category (meters) | Width Category (meters) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Bit 2 | Bit 3 | Bit 4 | Bit 5 |  |  |
| 0 | 0 | 0 | 0 | 0 | L < 15 | W < 11.5 |
| 1 |  |  |  | 1 |  | W < 23 |
| 2 | 0 | 0 | 1 | 0 | L < 25 | W < 28.5 |
| 3 |  |  |  | 1 |  | W < 34 |
| 4 | 0 | 1 | 0 | 0 | L < 35 | W < 33 |
| 5 |  |  |  | 1 |  | W < 38 |
| 6 | 0 | 1 | 1 | 0 | L < 45 | W < 39.5 |
| 7 |  |  |  | 1 |  | W < 45 |
| 8 | 1 | 0 | 0 | 0 | L < 55 | W < 45 |
| 9 |  |  |  | 1 |  | W < 52 |
| 10 | 1 | 0 | 1 | 0 | L < 65 | W < 59.5 |
| 11 |  |  |  | 1 |  | W < 67 |
| 12 | 1 | 1 | 0 | 0 | L < 75 | W < 72.5 |
| 13 |  |  |  | 1 |  | W < 80 |
| 14 | 1 | 1 | 1 | 0 | L < 200 | W < 80 |
| 15 |  |  |  | 1 |  | W > 80 |

Table 2-30: “Position Offset Applied” Encoding

| Coding | Meaning |
| --- | --- |
| 0 | Position Offset Not Applied |
| 1 | Position Offset Applied (POA) |

##### “UTC” Field Encoding

The “UTC” field is a 1-bit field (bit 5 of byte 17) that indicates whether the ADS-B Transmitting Subsystem is in the “UTC Coupled” condition or the “Non-UTC Coupled” condition (§3.1.1) and shall be encoded as indicated in Table 2-31.

Table 2-31: “UTC” Encoding

| Coding | Meaning |
| --- | --- |
| 0 | Non UTC Coupled Condition |
| 1 | UTC Coupled Condition |

##### Reserved Bits

Bits 6 through 8 of byte 17 are reserved for future use and shall be set to ZERO when the “ADDRESS QUALIFIER” field is set to “0,” “1,” “4,” or “5.”

##### Reserved Byte 18 of Payload Type Zero

Byte 18 of the UAT ADS-B Message Payload definition in Table 2-2, when the Payload Type is ZERO (0) is reserved for future use, and shall be set to ALL ZEROs.

#### STATE VECTOR Element (For TIS-B)

Format for the STATE VECTOR element used for a TIS-B shall be as defined in Table 2-32.  This encoding shall apply to UAT ADS-B Messages with PAYLOAD TYPE CODES of “0” through “10” only when a TIS-B target is being reported (ADDRESS QUALIFIER value is “2” or “3”).

Note:	Each of the fields shown is defined in §2.1.5.3.1 and §2.1.5.3.2.

Table 2-32: Format of STATE VECTOR Element (For TIS-B)

| Payload Byte # | Bit 1 | Bit 2 | Bit 3 |  | Bit 4 |  | Bit 5 |  | Bit 6 | Bit 7 | Bit 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | . . . |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  | LATITUDE (WGS-84) |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  | LONGITUDE (WGS-84) |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  | Alt Type |
| 11 |  | ALTITUDE |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |  |  | NIC |  |  |
| 13 | A/G STATE |  |  | Reserved |  |  |  |  |  |  |  |
| 14 | HORIZONTAL VELOCITY |  |  |  |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  | VERTICAL VELOCITY or A/V LENGTH/WIDTH CODE |  |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  | TIS-B SITE ID |  |  |  |  |
|  |  |  |  |  | . . . |  |  |  |  |  |  |

Note:	Design of the TIS-B Ground Subsystem is in a preliminary phase.  The message structure in Table 2-32 may be changed as this design matures.

##### “TIS-B SITE ID” Field Encoding

The “TIS-B SITE ID” field shall be a 4-bit (bits 5 through 8 of byte 17) field with the MSB as bit 5 and the LSB as bit 8 and shall be encoded as shown in Table 2-58.

Notes:

  - The “UTC” field shown in Table 2-5 for the State Vector Element is not provided for TIS-B transmissions.  The “UTC Coupled” status of the ground station transmitting TIS-B information is available in the UAT Ground Uplink Message (§2.2.1.4).

  - The application that uses TIS-B reports is assumed to make appropriate checks for a TIS-B Site ID of value ZERO.  If the Address Qualifier indicates that this is a TIS-B Message, and the TIS-B Site ID indicates a value of ZERO, an error condition is indicated.

##### Encoding for All Other Fields

The encoding of all other fields in the STATE VECTOR Element for TIS-B shown in Table 2-32 shall be consistent with that of §2.1.5.2.1 through §2.1.5.2.7.2.

#### MODE STATUS Element

Format for the MODE STATUS element shall be as defined in Table 2-33.  This encoding shall apply to UAT ADS-B Messages with PAYLOAD TYPE CODES of “1” and “3.”

Note:	Each of the fields shown is defined in the following subparagraphs.

Table 2-33: Format of MODE STATUS Element

| Payload Byte # | Bit 1 | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | . . . |  |  |  |  |  |  |  |
| 18 | (MSB) | EMITTER CATEGORY AND CALL SIGN CHARACTERS #1 AND #2 (Compressed Encoding) |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |  | (LSB) |
| 20 | (MSB) | CALL SIGN CHARACTERS #3, #4, AND #5 (Compressed Encoding) |  |  |  |  |  |  |
| 21 |  |  |  |  |  |  |  | (LSB) |
| 22 | (MSB) | CALL SIGN CHARACTERS #6, #7, AND #8 (Compressed Encoding) |  |  |  |  |  |  |
| 23 |  |  |  |  |  |  |  | (LSB) |
| 24 | EMERGENCY/PRIORITY STATUS |  |  | UAT MOPS VERSION |  |  | SIL |  |
| 25 | (MSB) | TRANSMIT MSO |  |  |  | (LSB) | Reserved |  |
| 26 | NACP |  |  |  | NACV |  |  | NICBARO |
| 27 | CAPABILITY CODES |  | OPERATIONAL MODES |  |  | TRUE/MAG | CSID |  |
| 28 | Reserved |  |  |  |  |  |  |  |
| 29 |  |  |  |  |  |  |  |  |
|  | . . . |  |  |  |  |  |  |  |

Note: 	In the above table, where MSB and LSB are not specifically noted, the MSB is the leftmost bit and the LSB is the rightmost bit.

##### “EMITTER CATEGORY AND CALL SIGN CHARACTERS #1 AND #2” Field

The “EMITTER CATEGORY AND CALL SIGN CHARACTERS #1 AND #2” field shall be a 16-bit (bit 1 of byte 18 to bit 8 of byte 19) field encoded as the binary numeral generated as:

402 B2  + 40 B1 + B0

Where the values B2, B1 and B0 are:

| B2 - | EMITTER CATEGORY (encoded as in §2.1.5.4.4) |
| --- | --- |
| B1 - | Character #1 of CALL SIGN (encoded as in §2.1.5.4.5) |
| B0 - | Character #2 of CALL SIGN (encoded as in §2.1.5.4.5) |

##### “CALL SIGN CHARACTERS #3, #4 AND #5” Field

The “CALL SIGN CHARACTERS #3, #4 AND #5” field is a 16-bit (bit 1 of byte 20 to bit 8 of byte 21) field and shall be encoded as the binary numeral generated as:

402 B2  + 40 B1 + B0

Where the values B2, B1 and B0 are.

| B2 - | Character #3 of CALL SIGN (encoded as in §2.1.5.4.5) |
| --- | --- |
| B1 - | Character #4 of CALL SIGN (encoded as in §2.1.5.4.5) |
| B0 - | Character #5 of CALL SIGN (encoded as in §2.1.5.4.5) |

##### “CALL SIGN CHARACTERS #6, #7 AND #8” Field

The “CALL SIGN CHARACTERS #6, #7 AND #8” field is a 16-bit (bit 1 of byte 22 to bit 8 of byte 23) field and shall be encoded as the binary numeral generated as:

402 B2  + 40 B1 + B0

Where the values B2, B1 and B0 are.

| B2 - | Character #6 of CALL SIGN (encoded as in §2.1.5.4.5) |
| --- | --- |
| B1 - | Character #7 of CALL SIGN (encoded as in §2.1.5.4.5) |
| B0 - | Character #8 of CALL SIGN (encoded as in §2.1.5.4.5) |

##### EMITTER CATEGORY

The EMITTER CATEGORY shall be encoded as shown in Table 2-34.

Table 2-34: EMITTER CATEGORY Encoding

| (decimal) | Meaning | (decimal) | Meaning |
| --- | --- | --- | --- |
| 0 | No aircraft type information | 20 | Cluster Obstacle |
| 1 | Light (ICAO) < 15 500 lbs | 21 | Line Obstacle |
| 2 | Small - 15 500 to 75 000 lbs | 22 | (reserved) |
| 3 | Large - 75 000 to 300 000 lbs | 23 | (reserved) |
| 4 | High Vortex Large (e.g., aircraft such as B757) | 24 | (reserved) |
| 5 | Heavy (ICAO) - > 300 000 lbs | 25 | (reserved) |
| 6 | Highly Maneuverable > 5G acceleration and high speed | 26 | (reserved) |
| 7 | Rotocraft | 27 | (reserved) |
| 8 | (Unassigned) | 28 | (reserved) |
| 9 | Glider/sailplane | 29 | (reserved) |
| 10 | Lighter than air | 30 | (reserved) |
| 11 | Parachutist/sky diver | 31 | (reserved) |
| 12 | Ultra light/hang glider/paraglider | 32 | (reserved) |
| 13 | (Unassigned) | 33 | (reserved) |
| 14 | Unmanned aerial vehicle | 34 | (reserved) |
| 15 | Space/transatmospheric vehicle | 35 | (reserved) |
| 16 | (Unassigned) | 36 | (reserved) |
| 17 | Surface vehicle — emergency vehicle | 37 | (reserved) |
| 18 | Surface vehicle — service vehicle | 38 | (reserved) |
| 19 | Point Obstacle (includes tethered balloons) | 39 | (reserved) |

##### CALL SIGN

The CALL SIGN shall consist of eight characters, which must contain only decimal digits 0-9, the capital letters A-Z, and – as trailing pad characters only – the “space” character.  Each character of the CALL SIGN shall be encoded as shown in Table 2-35.  The left-most character of the Call Sign corresponds to Character #1; the right-most corresponds to Character #8.

If the Call Sign is not available, then all eight characters of the CALL SIGN Field shall be set to the Base-40 digit code 37.

The 8 characters of the CALL SIGN field shall be encoded with an identifier appropriate for the Emitter Category, operating rules, and procedures under which the A/V is operating.

Note:	For aircraft, the Call Sign could be an abbreviation of the authorized radiotelephone Call Sign for that aircraft as assigned by ATS, the aircraft registration marking, or other authorized identifier for special operations.

Note:	 A Call Sign of less than 8 characters should be padded with spaces in the right-most (trailing) positions.  The first character should not be a space.

Table 2-35: “Call Sign” Character Encoding

| (decimal) | Character |  | (decimal) | Character |
| --- | --- | --- | --- | --- |
| 0 | 0 |  | 20 | K |
| 1 | 1 |  | 21 | L |
| 2 | 2 |  | 22 | M |
| 3 | 3 |  | 23 | N |
| 4 | 4 |  | 24 | O |
| 5 | 5 |  | 25 | P |
| 6 | 6 |  | 26 | Q |
| 7 | 7 |  | 27 | R |
| 8 | 8 |  | 28 | S |
| 9 | 9 |  | 29 | T |
| 10 | A |  | 30 | U |
| 11 | B |  | 31 | V |
| 12 | C |  | 32 | W |
| 13 | D |  | 33 | X |
| 14 | E |  | 34 | Y |
| 15 | F |  | 35 | Z |
| 16 | G |  | 36 | SPACE |
| 17 | H |  | 37 | Not Available |
| 18 | I |  | 38 | (reserved) |
| 19 | J |  | 39 | (reserved) |

##### “EMERGENCY/PRIORITY STATUS” Field Encoding

The “EMERGENCY/PRIORITY STATUS” field is a 3-bit (bits 1 through 3 of byte 24) field and shall be encoded as indicated in Table 2-36.

Table 2-36: “EMERGENCY/PRIORITY STATUS” Encoding

| Status Code bits | Status Code bits | Meaning |
| --- | --- | --- |
| MSB(Binary)LSB | (Decimal) |  |
| 000 | 0 | No emergency/Not reported |
| 001 | 1 | General emergency |
| 010 | 2 | Lifeguard/medical emergency |
| 011 | 3 | Minimum fuel |
| 100 | 4 | No communications |
| 101 | 5 | Unlawful interference |
| 110 | 6 | Downed Aircraft |
| 111 | 7 | (Reserved) |

Action #3: Verify encodings above with SCRSP documentation

##### “UAT MANUAL VERSION” Field Encoding

The “UAT MANUAL VERSION” field is a 3-bit (bits 4 through 6 of byte 24) field and shall be internally hard coded to ONE (binary 001) by all ADS-B Transmitting Subsystems for equipment complying with this Manual.

Table 2-37: UAT MANUAL Version Number

| UAT MANUAL Version # | UAT MANUAL Version # | Meaning |
| --- | --- | --- |
| MSB(Binary)LSB | (Decimal) |  |
| 000 | 0 | Reserved |
| 001 | 1 | Conformant to the first edition of the UAT MANUAL |
| 010 | 2 | Reserved |
| 011 | 3 | Reserved |
| 100 | 4 | Reserved |
| 101 | 5 | Reserved |
| 110 | 6 | Reserved |
| 111 | 7 | Reserved |

##### “SIL” Field Encoding

The Surveillance Integrity Level (“SIL”) field is a 2-bit (bits 7 and 8 of byte 24) field used to define the probability of the integrity containment radius, used in the “NIC” field, being exceeded, without alerting, and shall be encoded as indicated in Table 2-38.

Note:	This includes the effects of airborne equipment condition, which airborne equipment is in use, and which external signals are being used by the navigation source.

Table 2-38: “SIL” Encoding.

| SIL (binary) | SIL (decimal) | Probability of Exceeding the RC Integrity Containment Radius Without Detection | Exemplary Navigation Source |
| --- | --- | --- | --- |
| 00 | 0 | Unknown |  |
| 01 | 1 | 1  10-3 per flight hour or per operation |  |
| 10 | 2 | 1  10-5 per flight hour or per operation | e.g., RNP Level reporting Flight Management System |
| 11 | 3 | 1  10-7 per flight hour or per operation | e.g., GNSS Receiver |

Note:	It is assumed that SIL is a static (unchanging) value that depends on the position sensor being used.  Thus, for example, if an UAT ADS-B participant reports a NIC code of 0 because four or fewer satellites are available for a GPS fix, there would be no need to change the SIL code until a different navigation source were selected for the positions being reported in the SV report.

##### “TRANSMIT MSO” Field Encoding

The “TRANSMIT MSO” field is a 6-bit (bits 1 through 6 of byte 25) field that shall be used to encode the 6 LSBs of the Message Start Opportunity (§3.1.2.1) determined for the message transmission.

##### Reserved Bits

Bits 7 and 8 of byte 25 are reserved for future use and shall be set to ALL ZEROs for equipment conforming to this Manual.

Note: 	This field is reserved for future reporting of Barometric Altitude Quality (BAQ).

##### “NACP” Field Encoding

The Navigation Accuracy Category for Position (“NACP”) field is a 4-bit (bits 1 through 4 of byte 26) field used for applications to determine if the reported State Vector has sufficient position accuracy for the intended use and shall be encoded as indicated in Table 2-39.

Table 2-39: “NACP” Encoding

| NACP (binary) MSB LSB | NACP (decimal) | 95% Horizontal and Vertical Accuracy Bounds (EPU and VEPU) | Comment | Notes |
| --- | --- | --- | --- | --- |
| 0000 | 0 | EPU  18.52 km (10 NM) | Unknown accuracy |  |
| 0001 | 1 | EPU < 18.52 km (10 NM) | RNP-10 accuracy | 1 |
| 0010 | 2 | EPU < 7.408 km (4 NM) | RNP-4 accuracy | 1 |
| 0011 | 3 | EPU < 3.704 km (2 NM) | RNP-2 accuracy | 1 |
| 0100 | 4 | EPU < 1852 m (1NM) | RNP-1 accuracy | 1 |
| 0101 | 5 | EPU < 926 m (0.5 NM) | RNP-0.5 accuracy | 1 |
| 0110 | 6 | EPU < 555.6 m (0.3 NM) | RNP-0.3 accuracy | 1 |
| 0111 | 7 | EPU < 185.2 m (0.1 NM) | RNP-0.1 accuracy | 1 |
| 1000 | 8 | EPU < 92.6 m (0.05 NM) | e.g., GNSS (e.g. GPS with SA) |  |
| 1001 | 9 | EPU < 30 m and VEPU < 45 m | e.g., GNSS high accuracy (e.g GPS with SA off) | 2 |
| 1010 | 10 | EPU < 10 m and VEPU < 15 m | e.g., SBAS | 2 |
| 1011 | 11 | EPU < 3 m and VEPU < 4 m | e.g., GBAS | 2 |
| 1100 | 12 | (Reserved) |  |  |
| 1101 | 13 | (Reserved) |  |  |
| 1110 | 14 | (Reserved) |  |  |
| 1111 | 15 | (Reserved) |  |  |

Notes:

- RNP accuracy includes error sources other than sensor error, whereas horizontal error for NACP only refers to horizontal position error uncertainty.

- If geometric altitude is not being reported then the VEPU tests are not assessed.

- The Estimated Position Uncertainty (EPU) used in is a 95% accuracy bound on horizontal position.  EPU is defined as the radius of a circle, centered on the reported position, such that the probability of the actual position being outside the circle is 0.05.  When reported by a GNSS system, EPU is commonly called HFOM (Horizontal Figure of Merit).

- Likewise, Vertical Estimated Position Uncertainty (VEPU) is a 95% accuracy limit on the vertical position.  VEPU is defined as a vertical position limit, such that the probability of the actual vertical position differing from the reported vertical position by more than that limit is 0.05.  When reported by a GNSS system, VEPU is commonly called VFOM (Vertical Figure of Merit).

- Action #4: Verify accuracy of terminology above with GNSSP and SCRSP.

##### “NACV” Field Encoding

The Navigation Accuracy Category for Velocity (“NACV”) field is a 3-bit (bits 5 through 7 of byte 26) field used for applications to determine if the reported State Vector has sufficient velocity accuracy for the intended use and shall be encoded as indicated in Table 2-40.

Note:	The “NACV” field reflects the least accurate velocity component being transmitted.

Table 2-40: “NACV” Encoding

| NACV (binary) MSB LSB | NACV (decimal) | Horizontal Velocity Error (95%) | Vertical Geometric Velocity Error (95%) |
| --- | --- | --- | --- |
| 000 | 0 | Unknown or  10 m/s | Unknown or  50 feet (15.24 m) per second |
| 001 | 1 | < 10 m/s | < 50 feet (15.24 m) per second |
| 010 | 2 | < 3 m/s | < 15 feet (4.57 m) per second |
| 011 | 3 | < 1 m/s | < 5 feet (1.52 m) per second |
| 100 | 4 | < 0.3 m/s | < 1.5 feet (0.46 m) per second |
| 101 | 5 | (Reserved) | (Reserved) |
| 110 | 6 | (Reserved) | (Reserved) |
| 111 | 7 | (Reserved) | (Reserved) |

##### “NICBARO” Field Encoding

The Barometric Altitude Integrity Code (“NICBARO”) field is a 1-bit (bit 8 of byte 26) field that indicates whether or not the barometric pressure altitude provided in the State Vector element of the payload has been cross checked against another source of pressure altitude and shall be encoded as indicated in Table 2-41.

Table 2-41: “NICBARO” Encoding

| Coding | Meaning |
| --- | --- |
| 0 | Barometric Pressure Altitude has NOT been cross checked |
| 1 | Barometric Pressure Altitude has been cross checked |

##### “CAPABILITY CODES” Field Encoding

The “CAPABILITY CODES” field is a 2-bit (bits 1 and 2 of byte 27) field used to indicate the capability of a participant to support engagement in various operations and shall be encoded as indicated in Table 2-42.

Note:	The Target State Report Capability flag, the Trajectory Change Report capability level and the ARV Report Capability flag can be derived from the UAT transmissions of A1, A2 and A3 system participants.  Reference RTCA
DO-242A, Table 3.4(d) and 3.4(e).

Table 2-42: “CAPABILITY CODES” Encoding

| Byte # | Bit # | Encoding |
| --- | --- | --- |
| Byte 27 | Bit 1 | CDTI Traffic Display Capability. 0 = NO 1 = YES |
|  | Bit 2 | TCAS/ACAS Installed and Operational. 0 = NO 1 = YES |

###### “CDTI Traffic Display Capability” Subfield

The Capability Code for “Cockpit Display of Traffic Information (CDTI) traffic display capability” shall be set to ONE if the transmitting aircraft has the capability of displaying nearby traffic on a CDTI.  Otherwise, this code shall be ZERO.

###### “TCAS/ACAS Installed and Operational” Subfield

The Capability Code for “TCAS/ACAS installed and operational” shall be set to ONE if the transmitting aircraft is fitted with a TCAS (ACAS) computer and that computer is turned on and operating in a mode that can generate Resolution Advisory (RA) alerts.  Likewise, this Capability Code shall be set to ONE if the transmitting ADS-B equipment cannot ascertain whether or not a TCAS II or ACAS computer is installed, or cannot ascertain whether that computer, if installed, is operating in a mode that can generate RA alerts.  Otherwise, this Capability Code shall be ZERO.

Note:	A value of ONE is intended to signal a receiving ADS-B application that if it is necessary to avoid the transmitting aircraft, this should be done by horizontal rather than vertical maneuvers, because a Resolution Advisory from TCAS II or ACAS will advise the pilot to maneuver vertically.  If it is unknown whether or not the transmitting aircraft has TCAS, it should set this Capability Code to ONE so that receiving aircraft will be more likely to use horizontal than vertical maneuvers if necessary to avoid the transmitting aircraft.

Action #5a: Verify with RTCA SC-186 that the names of this field accurately reflects its meaning.

##### “OPERATIONAL MODES” Field Encoding

The “OPERATIONAL MODES” field is a 3-bit (bits 3 through 5 of byte 27) field used to indicate the capability of a participant to support engagement in various operations and shall be encoded as indicated in Table 2-43.

Table 2-43: “OPERATIONAL MODES” Encoding

| Byte # | Bit # | Encoding |
| --- | --- | --- |
| Byte 27 | Bit 3 | TCAS/ACAS Resolution Advisory Active Flag. 0 = NO 1 = YES |
|  | Bit 4 | IDENT Switch Active Flag. 0 = NOT Active (> 20 seconds since activated by pilot) 1 = Active (<= 20 seconds since activated by pilot) |
|  | Bit 5 | “Receiving ATC Services” Flag 0 = NOT Receiving ATC Services 1 = Receiving ATC Services |

###### “TCAS/ACAS Resolution Advisory Active” Flag

If the “TCAS/ACAS Installed and Operational” subfield is set to ZERO, then a transmitting ADS-B participant shall set the “TCAS/ACAS Resolution Advisory Active” Flag to ZERO.  Otherwise:

- If a Resolution Advisory is known to be in effect, a transmitting ADS-B participant shall set the “TCAS/CAS Resolution Advisory Active” Flag to ONE in the messages that it transmits to support the MS report so long as a TCAS/ACAS resolution advisory is in effect.

- Likewise this Flag shall be set to ONE if the transmitting ADS-B equipment cannot ascertain whether the TCAS II or ACAS computer is currently issuing a Resolution Advisory.

- At all other times, the transmitting ADS-B participant shall set the “TCAS/ACAS Resolution Advisory Active” Flag to ZERO.

Action #5b: Verify with RTCA SC-186 that the names of this field accurately reflects its meaning?

###### “IDENT Switch Active” Flag

The “IDENT Switch Active” Flag is activated by an IDENT switch, and upon activation of the IDENT switch, this flag shall be set to ONE in all scheduled UAT ADS-B Messages containing the MODE STATUS element for an interval of 20 seconds +/-4 seconds.  After the time interval expires, the flag shall be set to ZERO.

Note: 	This allows an ATC ground station 4-5 reception opportunities to receive the IDENT indication.

###### “Receiving ATC Services” Flag

A method shall be provided to allow the pilot to indicate whether the aircraft is receiving ATC services according to the following convention: ONE corresponds to yes, ZERO corresponds to no.

Note: 	This provides for a ground ATC system to identify an aircraft that is receiving ATC services, similar to an SSR transponder providing a squawk code of other than “1200.”

##### “True/Magnetic Heading” Flag

The “True/Magnetic Heading” Flag in the Mode-Status Element is a one-bit field (bit 6 of byte 27) which shall be set to ZERO to indicate that heading is reported referenced to true north, or set to ONE to indicate that heading is reported referenced to magnetic north.

Note:	This “True/Magnetic Heading” Flag supports the “Heading/Track Indicator” Flag in the TARGET STATE Element defined in §2.1.5.6.1.1.

##### Call Sign Identification (CSID)

The Call Sign Identification (CSID) Flag in the Mode Status Element is a one-bit field (bit 7 of byte 27) which shall be set to ONE (1) in this version of this Manual.

Action #6: Determine whether the UAT Technical Manual should include an option of setting this field to zero in the event that the Call Sign and Flight Plan ID of an aircraft are timesharing this field.

##### Reserved Bits

This Reserved Bits field is a 17-bit (bit 8 of byte 27 through bit 8 of byte 29) field used that may be used in the future to indicate the capability of a participant to support engagement in various operations and shall be set to ALL ZEROs.

#### AUXILIARY STATE VECTOR Element

The format for the AUXILIARY STATE VECTOR shall apply to UAT ADS-B Messages with “PAYLOAD TYPE CODES” of “1,” “2,” “5,” and “6” as defined in Table 2-44.

Note:	Each of the fields shown is defined in the following subparagraphs.

Table 2-44: Format of AUXILIARY STATE VECTOR Element

| Payload Byte # | Bit 1 | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | . . . |  |  |  |  |  |  |  |
| 30 | (MSB) | SECONDARY ALTITUDE |  |  |  |  |  |  |
| 31 |  |  |  | (LSB) |  |  |  |  |
| 32 |  |  |  |  |  |  |  |  |
| 33 | Reserved |  |  |  |  |  |  |  |
| 34 |  |  |  |  |  |  |  |  |
|  | . . . |  |  |  |  |  |  |  |

##### “SECONDARY ALTITUDE” Field Encoding

The “SECONDARY ALTITUDE” field is a 12-bit (bit 1 of byte 30 through bit 4 of byte 31) field shall encode either the geometric altitude or barometric pressure altitude depending on the setting of the “ALTITUDE TYPE” field (§2.1.5.2.2).

The altitude encoded in the “SECONDARY ALTITUDE” field shall be the opposite type to that specified by the “ALTITUDE TYPE” field with encoding consistent with that used in Table 2-8.

##### Reserved Bits

Bit 5 of byte 31 through bit 8 of byte 34 are reserved for future use, and shall be set to ALL ZEROs.

Note: 	This field is reserved for future definition to contain either Air-Referenced Velocity or perhaps wind vector and temperature.

#### TARGET STATE Element (Payload Type Codes “3” and “4”)

Format for the TARGET STATE element shall apply to UAT ADS-B Messages with “PAYLOAD TYPE CODES” of “3” and “4” as defined in Table 2-45.

Table 2-45: Format of TARGET STATE Element (Payload Type Codes “3” and “4”)

| Payload Byte # | Bit 1 | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | . . |  |  |  |  |  |  |  |
| 30 |  | TARGET HEADING or TRACK ANGLE INFORMATION |  |  |  |  |  |  |
| 31 |  |  |  |  |  |  |  |  |
| 32 |  |  |  |  |  |  |  |  |
| 33 | TARGET ALTITUDE INFORMATION |  |  |  |  |  |  |  |
|  | . . |  |  |  |  |  |  |  |

##### “TARGET HEADING or TRACK ANGLE INFORMATION” Field Encoding

The “TARGET HEADING or TRACK ANGLE INFORMATION” field shall be composed of subfields as indicated in Table 2-46.

Table 2-46: “TARGET HEADING or TRACK ANGLE INFORMATION” Format

| Byte 30 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 1 | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 |
| Hdg/Trk | Target Source Indicator (H) |  | Mode Indicator (H) |  | Reserved | (MSB) |  |
| Byte 31 |  |  |  |  |  |  |  |
| Bit 1 | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 |  |
| -- Track Heading or Track Angle -- |  |  |  |  |  | (LSB) |  |

###### “Heading/Track Indicator” Flag Encoding

The “Heading/Track Indicator” flag (bit 1 of byte 30) shall be set to ZERO to indicate that the “Target Heading or Track Angle” subfield conveys target heading, or ONE to indicate that it conveys target track angle.

Note:	The reference direction (true north or magnetic north) is conveyed in the “True/Magnetic Heading Flag” of the Mode Status Element referenced in §2.1.5.4.16.

###### “Target Source Indicator (Horizontal)” Subfield Encoding

The “Target Source Indicator (Horizontal)” subfield is a 2-bit (bits 2 and 3 of byte 30) field that indicates the source of Target Heading/Track Angle information and shall be encoded as indicated in Table 2-47.

Table 2-47: “Target Source Indicator (Horizontal) Encoding

| Bit 2, Byte 30 | Bit 3, Byte 30 | Encoding (decimal) | Meaning |
| --- | --- | --- | --- |
| 0 | 0 | 0 | No Valid Horizontal Target State data is available |
| 0 | 1 | 1 | Autopilot control panel selected value, such as Mode Control Panel (MCP) or Flight Control Unit (FCU) |
| 1 | 0 | 2 | Maintaining current heading or track angle (e.g., autopilot mode select) |
| 1 | 1 | 3 | FMS/RNAV system (indicates track angle specified by leg type) |

###### “Mode Indicator (Horizontal)” Subfield Encoding

The “Mode Indicator (Horizontal)” is a 2-bit (bits 4 and 5 of byte 30) subfield that reflects the aircraft’s state relative to the target heading or track angle and shall be encoded as indicated in Table 2-48.

Table 2-48: “Mode Indicator (Horizontal)” Subfield Encoding

| Coding (binary) | Coding (decimal) | Meaning |
| --- | --- | --- |
| 00 | 0 | Mode Indicator Not Available |
| 01 | 1 | Target Heading or Track Angle being Acquired |
| 10 | 2 | Target Heading or Track Angle is Captured or is being Maintained |
| 11 | 3 | Reserved |

###### Reserved Bits

Bit 6 of byte 30 is reserved for future use and shall always be set to ZERO.

###### “Target Heading or Track Angle” Subfield Encoding

The “Track Angle/Heading” subfield is a 9-bit (bit 7 of byte 30 through bit 7 of byte 31) subfield that shall be used to report the Track Angle or Heading of the ADS-B Transmitting Subsystem as shown in Table 2-49.

Table 2-49: “Target Heading or Track Angle” Encoding

| Coding MSB(binary)LSB | Coding (decimal) | Meaning |
| --- | --- | --- |
| 0 0000 0000 | 0 | Track Angle/Heading is ZERO degrees |
| 0 0000 0001 | 1 | Track Angle/Heading = 0.703125 degrees |
| 0 0000 0010 | 2 | Track Angle/Heading = 1.406250 degrees |
| … | … | … |
| 1 1111 1110 | 510 | Track Angle/Heading = 358.593750 degrees |
| 1 1111 1111 | 511 | Track Angle/Heading = 359.296875 degrees |

##### “TARGET ALTITUDE INFORMATION” Field Encoding

The “TARGET ALTITUDE INFORMATION” field is a 17-bit (bit 8 of byte 31 through bit 8 of byte 33) field composed of subfields and shall be encoded as indicated in Table 2-50.

Table 2-50: “TARGET ALTITUDE INFORMATION” Format

| Byte 31 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 1 | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 |
|  |  |  |  |  |  |  | Target Alt. Type |
| Byte 32 |  |  |  |  |  |  |  |
| Bit 1 | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 |
| Target Source Indicator (V) |  | Mode Ind. (V) |  | Target Altitude Capability |  | (MSB) |  |
| Byte 33 |  |  |  |  |  |  |  |
| Bit 1 | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 |
| -- Target Altitude -- |  |  |  |  |  |  | (LSB) |

###### “Target Altitude Type” Flag Encoding

The “Target Altitude Type” flag is a 1-bit (bit 8 of byte 31) flag that indicates whether the target altitude is barometric pressure altitude or flight level (used for target altitudes above the transition level between altitude types), or a locally corrected altitude (used for target altitudes below the transition level) and shall be encoded as indicated in Table 2-51.

Note: 	This flag is set based on the relationship of the Target Altitude to the “transition level.”

Table 2-51: Target Altitude Type Values

| Value | Meaning |
| --- | --- |
| 0 | Pressure Altitude (“Flight Level”) – target altitude is above transition level |
| 1 | Baro-Corrected Altitude (“MSL”) – target altitude is below transition level |

###### “Target Source Indicator (Vertical)” Subfield Encoding

The “Target Source Indicator (Vertical)” is a 2-bit (bits 1 and 2 of byte 32) field that indicates the source of Target Altitude information and shall be encoded as indicated in Table 2-52.

Table 2-52: “Target Source Indicator (Vertical)” Encoding

| Bit 1, Byte 32 | Bit 2, Byte 32 | (decimal) | Meaning |
| --- | --- | --- | --- |
| 0 | 0 | 0 | No Valid Vertical Target State data is available |
| 0 | 1 | 1 | Autopilot Control Panel selected value such as a Mode Control Panel (MCP) or Flight Control Unit (FCU) |
| 1 | 0 | 2 | Holding Altitude |
| 1 | 1 | 3 | FMS or RNAV system |

###### “Mode Indicator (Vertical)” Subfield Encoding

The “Mode Indicator (Vertical)” is a 2-bit (bit 3 and 4 of byte 32) subfield that reflects the aircraft’s state relative to the target altitude and shall be encoded as indicated in Table 2-53.

Table 2-53: “Mode Indicator (Vertical)” Subfield Encoding

| Coding MSB(binary)LSB | Coding (decimal) | Meaning |
| --- | --- | --- |
| 00 | 0 | Mode Indicator Not Available |
| 01 | 1 | Target Altitude being Acquired |
| 10 | 2 | Target Altitude Captured or Maintained |
| 11 | 3 | Reserved |

###### “Target Altitude Capability” Subfield Encoding

The “Target Altitude Capability” is a 2-bit (bit 5 and 6 of byte 32) field that describes the value occupying the “Target Altitude” subfield and shall be encoded as indicated in Table 2-54.

Table 2-54: “Target Altitude Capability” Encoding

| Bit 5, Byte 32 | Bit 6, Byte 32 | Meaning |
| --- | --- | --- |
| 0 | 0 | Target Altitude Capability Not Available |
| 0 | 1 | Capability for either holding altitude, or for autopilot control panel selected altitude |
| 1 | 0 | Capability for either holding altitude, for autopilot control panel selected altitude, or for any FMS/RNAV level-off altitude |
| 1 | 1 | Capability for holding altitude only |

###### “Target Altitude” Subfield Encoding

“Target Altitude” is a 10-bit (bit 7 of byte 32 through bit 8 of byte 33) field that is the aircraft’s next intended level flight altitude if in a climb or descent, or its current intended altitude if commanded to hold altitude and shall be encoded as indicated in Table 2-55.

Table 2-55: “Target Altitude” Encoding

| Coding MSB(binary)LSB | Coding (decimal) | Meaning |
| --- | --- | --- |
| 00 0000 0000 | 0 | Target Altitude information unavailable |
| 00 0000 0001 | 1 | Target Altitude = -1000 feet |
| 00 0000 0010 | 2 | Target Altitude = -900 feet |
| … | … | … |
| 00 0000 1010 | 10 | Target Altitude = -100 feet |
| 00 0000 1011 | 11 | Target Altitude = ZERO feet |
| 00 0000 1100 | 12 | Target Altitude = 100 feet |
| … | … | … |
| 11 1111 1110 | 1022 | Target Altitude = 101,100 feet |
| 11 1111 1111 | 1023 | Target Altitude > 101,150 feet |

##### TARGET STATE Element (Payload Type Code “6”)

Format for the TARGET STATE element shall apply to UAT ADS-B Messages with “PAYLOAD TYPE CODES” of “6” consistent with Table 2-56

Note:	Each of the fields shown are defined in §2.1.5.6.1 through §2.1.5.6.2.5 with the exception of the byte offset indicated in Table 2-56.

Table 2-56: Format of TARGET STATE Element

| Payload Byte # | Bit 1 | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | . . |  |  |  |  |  |  |  |
| 25 | TARGET HEADING or TRACK ANGLE INFORMATION |  |  |  |  |  |  |  |
| 26 |  |  |  |  |  |  |  |  |
| 27 |  |  |  |  |  |  |  |  |
| 28 | TARGET ALTITUDE INFORMATION |  |  |  |  |  |  |  |
|  | . . |  |  |  |  |  |  |  |

#### TRAJECTORY CHANGE Element

Equipment conforming to this Manual shall insert ALL ZEROs in bytes 18 through 29, whenever this element is present in a transmitted message.

Note:	This element contains 96 bits (bytes 18 through 29) that are reserved for future definition.  [make reference to a section of the Implementation Manual that is taken from Appendix L of DO-282 and subgroup study of 4 TCRs]

## UAT Ground Uplink Message Payload

The UAT Ground Uplink Message Payload shall consist of two components: the first eight bytes that comprise the UAT-Specific Header and bytes 9 through 432 that comprise the Application Data as shown in Table 2-57.

Bytes and bits shall be fed to the interleaving process with the most significant byte, byte #1, transmitted first, and within each byte, the most significant bit, bit #1, transmitted first.

Table 2-57: Format of the UAT Ground Uplink Message Payload

| Byte # | Bit 1 | Bit 2 | Bit 3 |  | Bit 4 | Bit 5 |  | Bit 6 | Bit 7 | Bit 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | (MSB) |  |  |  |  |  |  |  |  |  |
| 2 |  |  | GROUND STATION LATITUDE (WGS-84) |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  | (LSB) | (MSB) |
| 4 |  |  |  |  |  |  |  |  |  |  |
| 5 |  | GROUND STATION LONGITUDE (WGS-84) |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  | (LSB) | POS VALID |
| 7 | UTC Coupled | Reserved | App Data Valid |  | (MSB) |  |  | SLOT ID |  | (LSB) |
| 8 | (MSB) | TIS-B Site ID |  |  | (LSB) | Reserved |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |
|  | APPLICATION DATA |  |  |  |  |  |  |  |  |  |
| 432 |  |  |  |  |  |  |  |  |  |  |

### UAT-Specific Header

#### “GROUND STATION LATITUDE” Field Encoding

The “GROUND STATION LATITUDE” field is a 23-bit (bit 1 of byte 1 through bit 7 of byte 3) field used to identify the latitude of the ground station and shall be encoded by the ground station using the same format as defined for latitude information in the UAT ADS-B Message (§2.1.5.2.1).

Note:	The resolution of this field has been selected to support a potential passive ranging function.

#### “GROUND STATION LONGITUDE” Field Encoding

The “GROUND STATION LONGITUDE” field is a 24-bit (bit 8 of byte 3 through bit 7 of byte 6) field used to identify the longitude of the ground station and shall be encoded by the ground station using the same format as defined for longitude information in the UAT ADS-B Message (§2.1.5.2.1).

Note:	The resolution of this field has been selected to support a potential passive ranging function.

#### “POSITION VALID” Field Encoding

The “POSITION VALID” field is a 1-bit (bit 8 of byte 6) field used to indicate whether or not the position in the header is valid and shall be encoded such that a ONE represents a VALID position and a ZERO represents an INVALID position.

#### “UTC Coupled” Field Encoding

The “UTC Coupled” field is a 1-bit (bit 1 of byte 7) field used to indicate whether or not the ground station 1 Pulse Per Second timing is valid and shall be encoded such that a ONE represents that the Ground Station is UTC-Coupled and a ZERO represents that the Ground Station is not UTC-Coupled.

Note:	See Paragraph 3.2.1

#### Reserved Bits

Bit 2 of byte 7 is reserved for future use and shall always be set to ZERO.

#### “APPLICATION DATA VALID” Field Encoding

The “APPLICATION DATA VALID” field is a 1-bit (bit 3 of byte 7) field used to indicate whether or not the Application Data is valid for operational use and shall be encoded such that a ONE represents VALID Application Data and a ZERO represents INVALID Application Data.

Notes:

- Airborne applications should ignore the Application Data field when this bit is set to INVALID.

- This field will allow testing and demonstration of new products without impact to operational airborne systems.

#### “SLOT ID” Field Encoding

The “SLOT ID” field is a 5-bit (bit 4 through bit 8 of byte 7) field used to identify the time slot within which the UAT Ground Uplink Message transmission took place and shall be encoded as a 5-bit unsigned binary numeral.

Note: 	The Slot for certain ground station messages may be continually shifted for maximum interference tolerance to other users sharing the band.  Airborne receivers do not need a priori knowledge of this shifting scheme; this is for ground service providers to coordinate.  The actual Slot ID in use for each uplink message is encoded by the ground station (Section 3.2.2.2).

#### “TIS-B SITE ID” Field Encoding

The “TIS-B SITE ID” field is a 4-bit (bits 1, through 4 of byte 8) field used to convey the reusable TIS-B Site ID that shall be encoded with each TIS-B transmission as shown in Table 2-58:

Table 2-58: Encoding of TIS-B Site ID

| Encoding | Meaning |
| --- | --- |
| 0000 | No TIS-B information transmitted from this site |
| 0001 | Assigned to ground stations that provide TIS-B information by TIS-B administration authority |
| through |  |
| 1111 |  |

Note: 	This field supports TIS-B applications that verify TIS-B transmissions were transmitted from the site located at the Latitude/Longitude encoded in the UAT-Specific Header portion of the Ground Uplink payload.  The width of the field was selected based upon analysis of the needs of a potential passive ranging function.

#### Reserved Bits

Bits 5 through 8 of byte 8 are reserved for future use and shall be set to ALL ZEROs.

### Ground Uplink Application Data

The APPLICATION DATA field is a 424 byte field (byte #9 through byte # 432). The Application Data shall consist of Information Frames, and shall always be an integral number of bytes.  Any remaining unused portion of the field shall be filled with ZERO bytes.

#### Information Frames

Each Information Frame shall consist of ‘N’ bytes, comprising three fields formatted as described in Table 2-59:

Table 2-59: Format for Information Frames

| Byte # | Bit 1 | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | MSB |  |  | Length |  |  |  |  |
| 2 | LSB | Reserved |  |  | MSB | Frame Type |  | LSB |
| 3 - N |  |  |  | Frame Data |  |  |  |  |

Note:	Byte numbers in this table are relative to the beginning of the current Information Frame.

##### “Length” subfield encoding

The Length field (Byte 1: Bit 1 through Byte 2: Bit 1) is a 9-bit field that shall contain the length of the Frame Data field in bytes.  Values shall range from 0 through 422 (decimal). The Length value shall always equal to ‘N-2’.

##### “Reserved” subfield encoding

The Reserved field (Byte 2: Bits 2 through 4) is a 3-bit field that is reserved for future use, and shall be set to the ZERO value in equipment that complies with this document.

##### “Frame Type” subfield encoding

The Frame Type field (Byte 2: Bits 5 through 8) is a 4-bit field that shall contain the indication for the format of the Frame Data field.  The Frame Types are defined in Table 2-60.

Table 2-60: Frame Type Encoding

| MSB Value (binary) LSB | Frame Data Format |
| --- | --- |
| 0 0 0 0 | FIS-B APDU |
| 0001 - 1111 | Reserved |

##### “Frame Data” subfield encoding

The encoding of this field is beyond the scope of this Manual and is provided in other documentation.

Action #7: Review at Toulouse meeting based upon RTCA review during week of 15 September

# System Timing and Message Transmission Procedures

## Avionics

### Procedures for Processing of Time Data

Note:	UAT equipment derives its timing for transmitter and receiver functions from GPS/GNSS (or equivalent) time sources.  The Time of Applicability of the PVT data is presumed to be within +/- 5 milliseconds of the Leading Edge of the Time Mark signal to which it applies.

Time Mark information shall be utilized by the UAT equipment in the following ways:

- Any extrapolation of Position data shall comply with the requirements of §3.1.3.1 and §3.1.3.2.

- The UAT transmit message timing shall comply with the requirements of §3.2.1.

- The UAT receiver time processing shall comply with the requirements of §4.1.1.

Notes:

  - A possible implementation of the GPS/GNSS Time Mark pulse is illustrated in Figure 3-1, adapted from ARINC Characteristic 743A.

  - Determination of time source “equivalence” will be made by appropriate Certification Authorities.  Useful information concerning recommended accuracy of such a time source may be found in Section 5.3 of the UAT Implementation Manual.

#### UTC Coupled Condition

The “UTC Coupled” subfield shall be set to ONE, except under the conditions discussed in §3.1.1.2.

Note:	Operation of the UAT system in normal mode presumes GNSS, or equivalent, equipage on system participants to, for example, prevent media access conflict with the UAT ground up-link transmissions.  Short term GNSS outages are mitigated by UAT ground infrastructure that provides timing information and/or by the ability of UAT avionics to prevent Airborne UAT Transmitting Subsystems from transmitting in the Ground Uplink Segment for a minimum of 20 minutes in the absence of GNSS (§3.1.1.2[d]).  In areas without ground up-link transmissions, there is no media access conflict.

Figure 3-1: GPS/GNSS Time Mark Pulse

#### Non-UTC Coupled Condition

- This condition shall be entered when the ADS-B equipment has not been provided a GPS/GNSS, or equivalent, time mark.

Note:	This is not the normal condition; it is a degraded mode of operation.

- Within 2 seconds of entering the Non-UTC Coupled condition, the UAT equipment shall set the “UTC Coupled” subfield to ZERO in any transmitted messages.

- While in the non-UTC Coupled Condition, Airborne UAT equipment with operational receivers shall be capable of aligning to within +/- 6 milliseconds of UTC time based upon successful message reception of any UAT Ground Uplink Message with the “UTC Coupled” bit set.

Note: 	This assumes that the UAT Ground Uplink Message is referenced to UTC 1-second epoch.

- While in the non-UTC Coupled Condition when UAT Ground Uplink Messages cannot be received, the UAT transmitter shall estimate — or “coast” — time through the outage period such that the drift rate of estimated time, relative to actual UTC-coupled time, is no greater than 12 milliseconds in 20 minutes.

- While in the non-UTC Coupled Condition, ADS-B transmissions shall continue.

- The UAT equipment shall change state to the UTC coupled condition within 2 seconds of availability of the UTC coupled source.

Notes:

- Item “d” above is consistent with an initial drift rate of 10 PPM in the baud clock over the 12 millisecond air-ground segment guard time.  Clock drift can be compensated up to the time coasting begins.

- In the non-UTC Coupled Condition, the estimated 1 second UTC epoch signal does NOT indicate the time of validity of Position, Velocity and Time (PVT) information.

- Any installations of equipment involving separated transmitters and receivers must provide a mechanism to fulfill the requirement stated in subparagraph (c) above.

- This reversionary timing exists for the following reasons:(a) to support UAT ADS-B Message transmission using an alternate source of position and velocity, if available; (b) to support UAT ADS-B Message transmission in absence of position and velocity data in order that any available fields are conveyed (e.g., barometric altitude) and (c) to ensure that a signal is provided in the event the ground network can perform an ADS-B-independent localization of the Aircraft (e.g., multilateration).

### ADS-B Media Access Timing

#### The Message Start Opportunity (MSO)

UAT ADS-B Messages shall be transmitted at discrete Message Start Opportunities (MSO) chosen by the following pseudo-random process.

The specific pseudo-random number R(m) chosen by an aircraft shall depend on the aircraft’s current position and on the previously chosen random number R(m-1) by first letting

N(0) = 12 L.S.B.’s of the most recent valid “LATITUDE”, and

N(1) = 12 L.S.B.’s of the most recent valid “LONGITUDE”

where the “latitude” and “longitude” are as defined in §2.1.5.2.1.

Using N(0) and N(1), the following procedure shall then establish the transmission timing (MSO) for the current UAT frame m.

R(m)=4001⋅R(m−1)+N(mmod2)mod3200, with

R(0) = N(0)

- When in the first frame after power up, and whenever the Vertical Status is determined to be in the AIRBORNE condition, the transmitter shall be in the full MSO range mode, where the MSO is determined as follows:

- MSO = 752 + R(m)

- Under all other conditions the transmitter shall be in the restricted MSO range mode, where the MSO is determined as follows:

- MSO = 752 + R* + R(m)mod800

- With R* = R(k) – R(k)mod800, where “k” is the frame just prior to entering the restricted MSO range mode.

Notes:

- Retention of N(0) and N(1) in non-volatile memory is required to prevent common MSO selections amongst A/Vs when no valid latitude and longitude is currently available.

- The latitude and longitude alternate in providing a changing “seed” for the pseudo-random number generation.

- The restricted range MSO mode makes the choice of MSO more nearly periodic in order to support certain surface applications.

#### Relationship of the MSO to the Modulated Data

The optimum sample point of the first bit of the UAT synchronization sequence at the antenna terminal of the UAT equipment shall occur at TTX microseconds after the 1 second UTC epoch according to the following formula:

TTX (microseconds) = 6000 + (250*MSO)

within the following tolerances:

- +/- 500 nanoseconds for UAT equipment with an internal UTC coupled time source,

- +/- 500 nanoseconds for UAT equipment with an external UTC coupled time source.

Notes:

- This is required to support ADS-B range validation by a receiving application.  This requirement sets the ultimate timing accuracy of the transmitted messages under the UTC Coupled condition. See Section 5.3 of the UAT Implementation Manual for a discussion of UAT Timing Considerations.

- Referencing this measurement to the optimum sampling point is convenient since this is the point in time identified during the synchronization process.

- There is no requirement to demonstrate this relationship when in the non-UTC Coupled condition.

### Time Registration and Latency

Note:	This subparagraph contains requirements imposed on the ADS-B Transmitting Subsystem relative to two parameters.  The first relates to the obligation of the transmitter to ensure position data in each ADS-B Message relates to a standard time of applicability.  The second relates to the obligation of the transmitter to reflect new ADS-B Message data available at the transmitter input into the transmitted ADS-B Message itself.  This requirement is expressed as a cutoff time by which any updated data presented to the UAT transmitter should be reflected in the message output.  Rules for time of applicability and cutoff time vary depending on the quality of SV data being transmitted and whether the transmitter is in the UTC Coupled state.

The UAT transmitter shall use the Precision or Non-Precision condition for reporting SV data according to the criteria below:

- Precision condition shall be in effect when:

- The “NACP” value is “10” or “11,” or

- The “NIC” value is  “9,” “10” or “11”

- Otherwise, the Non-Precision condition shall be in effect.

#### Requirements When in Non-Precision Condition and UTC Coupled

When the UAT Transmitting Subsystem is in the Non-Precision Condition, and is UTC Coupled:

- At the time of the ADS-B Message transmission, position information encoded in the “LATITUDE,” “LONGITUDE,” and “ALTITUDE” fields shall be applicable as of the start of the current 1 second UTC Epoch.

- Any updated ADS-B Message fields, other than “LATITUDE,” “LONGITUDE” or “ALTITUDE,” that are provided at the ADS-B equipment input interface at least 200 milliseconds prior to the time of a scheduled ADS-B Message transmission that involves those fields, shall be reflected in the transmitted message.

Notes:

- Specifically, any extrapolation of position performed should be to the start of the 1-second UTC Epoch and not the time of transmission.

- Velocity information cannot be extrapolated and may therefore have additional ADS-B imposed latency (generally no more than one extra second).

#### Requirements When in Precision Condition and UTC Coupled

When the UAT Transmitting Subsystem is in the Precision Condition, and is UTC Coupled:

- At the time of the ADS-B Message transmission, the position information encoded in the “LATITUDE,” “LONGITUDE,” and “ALTITUDE” fields shall be applicable as of the start of the current 0.2 second UTC Epoch.

- Any updated ADS-B Message fields, other than “LATITUDE,” “LONGITUDE” or “ALTITUDE,” that are provided at the ADS-B equipment input interface at least 200 milliseconds prior to the time of a scheduled ADS-B Message transmission that involves those fields, shall be reflected in the transmitted message.

Notes:

- Specifically, any extrapolation of position performed should be to the start of the 0.2 second UTC Epoch and not the time of transmission.

- Operation in this condition assumes a GPS/GNSS sensor output rate of 5 Hz or greater is available to the ADS-B Transmitting Subsystem.

#### Requirements When Non-UTC Coupled

When the UAT Transmitting Subsystem is in the Non-UTC Coupled Condition:

- Any change in an ADS-B Message field provided to the transmitter shall be reflected in any transmitted message containing that message field that is transmitted more than 1.0 second after the new value is received by the transmitter.

- No extrapolation of position shall be performed in this condition.

Note:	Even though no extrapolation of position is performed when non-UTC Coupled, the UAT Transmitting Subsystem makes no adjustment to the NIC or NAC that it receives as inputs.  Also, it is not expected that a single transmitted message would ever indicate both the Non-UTC Coupled condition and a NIC or NACP consistent with the Precision condition.

#### Data Timeout

At the Time of Applicability for the ADS-B Message transmission, any ADS-B Message fields without an update provided to the transmitter within the Data Lifetime parameter (in seconds) of Table 3-1 , shall be encoded as “data unavailable” in the subsequent transmitted message containing that message field.

Table 3-1: UAT ADS-B Transmitter Input Requirements

| Element # | Input Data Element | Relevant Paragraph | Data Lifetime (seconds) |
| --- | --- | --- | --- |
| 1 | ICAO 24-bit Address |  | No limit |
| 2 | Latitude (2) |  | 2 |
| 3 | Longitude (2) |  | 2 |
| 4 | Altitude Type Selection (Barometric vs Geometric) |  | 60 |
| 5 | Barometric Pressure Altitude |  | 2 |
| 6 | Geometric Altitude |  | 2 |
| 7 | NIC |  | 2 |
| 8 | Automatic AIRBORNE / ON-GROUND Indication |  | 2 |
| 9 | North Velocity (2) |  | 2 |
| 10 | East Velocity (2) |  | 2 |
| 11 | Ground Speed |  | 2 |
| 12 | Track Angle |  | 2 |
| 13 | Heading |  | 2 |
| 14 | Barometric Vertical Rate |  | 2 |
| 15 | Geometric Vertical Rate (2) |  | 2 |
| 16 | A/V Length and Width, and POA |  | No limit |
| 17 | UTC 1 PPS Timing (2) |  | 2 |
| 18 | Emitter Category |  | No limit |
| 19 | Call Sign |  | 60 |
| 20 | Emergency / Priority Status Selection |  | 60 |
| 21 | SIL |  | 60 |
| 22 | NACP (2) |  | 2 |
| 23 | NACV (2) |  | 2 |
| 24 | NICBARO |  | 60 |
| 25 | CDTI Traffic Display Capability |  | 60 |
| 26 | TCAS Installed and Operational |  | 60 |
| 27 | TCAS/ACAS Resolution Advisory Flag |  | 60 |
| 28 | IDENT Selection |  | 60 |
| 29 | “Receiving ATC Services” Flag |  | 60 |
| 30 | “True/Magnetic Heading” Flag |  | 60 |
| 31 | Heading / Track Indicator |  | 60 |
| 32 | Target Source Indicator (Horizontal) |  | 60 |
| 33 | Horizontal Mode Indicator (Horizontal) |  | 60 |
| 34 | Target Heading or Track Angle |  | 60 |
| 35 | Target Altitude Type |  | 60 |
| 36 | Target Source Indicator (Vertical) |  | 60 |
| 37 | Mode Indicator (Vertical) |  | 60 |
| 38 | Target Altitude Capability |  | 60 |
| 39 | Target Altitude |  | 60 |
| 40 | Radio Altitude |  | 2 |
| 41 | Pressure Altitude Disable |  | No limit |

### Special Requirements for Transceiver Implementations

#### Transmit-Receive Turnaround Time

A transceiver shall be capable of switching from transmission to reception within 2 milliseconds.

Note: 	Transmit to receive switching time is defined as the time between the optimum sampling point of the last information bit of one transmit message and the optimum sampling point of the first bit of the synchronization sequence of the subsequent receive message.

#### Receive-Transmit Turnaround Time

A transceiver shall be capable of switching from reception to transmission within 2 milliseconds.

Note: 	Receive to transmit switching time is defined as the time between the optimum sampling point of the last information bit of one receive message and the optimum sampling point of the first bit of the synchronization sequence of the subsequent transmit message.

## Ground Station

### Procedures for Processing of Time Data

UAT ground station equipment shall utilize Time Mark information in the following ways:

- The UAT Ground Station transmitter message timing accuracy shall comply with the requirements of §3.1.2.2

- The UAT Ground Station receiver time processing shall comply with the requirements of §4.1.1.

Notes:

- UAT Ground Station equipment derives its timing for transmitter and receiver functions from GPS/GNSS (or equivalent) time sources.

- A possible implementation of the GPS/GNSS Time Mark pulse is illustrated in Figure 3-1, adapted from ARINC Characteristic 743A.

- Determination of time source “equivalence” will be made by appropriate Certification Authorities.  Useful information concerning recommended accuracy of such a time source may be found in Appendix E.

#### UTC Coupled Condition

The “UTC Coupled” subfield shall be set to ONE, except under the conditions discussed in §3.1.1.2.

Note:	Operation of the UAT Ground System in normal mode presumes GNSS, or equivalent timing.

#### Non-UTC Coupled Condition

- This condition shall be entered when the UAT ground station equipment has not been provided a GPS/GNSS, or equivalent, time mark.  This is not the normal condition; it is a degraded mode of operation.

- Within 2 seconds of entering the Non-UTC Coupled condition, the UAT ground station equipment shall set the “UTC Coupled” subfield to ZERO in any transmitted messages.

### UAT Ground Station Media Access

#### Time Slots

The UAT Ground Station shall establish 32 Time Slots for transmission of Ground Uplink messages as defined in Table 3-1.

Table 3-1: Time Slot Definition for the UAT Ground Segment

| Slot ID # | Time Slot Span |  | Slot ID # | Time Slot Span |  |
| --- | --- | --- | --- | --- | --- |
|  | Starting MSO | Ending MSO |  | Starting MSO | Ending MSO |
| 1 | 0 | 22 | 17 | 352 | 374 |
| 2 | 22 | 44 | 18 | 374 | 396 |
| 3 | 44 | 66 | 19 | 396 | 418 |
| 4 | 66 | 88 | 20 | 418 | 440 |
| 5 | 88 | 110 | 21 | 440 | 462 |
| 6 | 110 | 132 | 22 | 462 | 484 |
| 7 | 132 | 154 | 23 | 484 | 506 |
| 8 | 154 | 176 | 24 | 506 | 528 |
| 9 | 176 | 198 | 25 | 528 | 550 |
| 10 | 198 | 220 | 26 | 550 | 572 |
| 11 | 220 | 242 | 27 | 572 | 594 |
| 12 | 242 | 264 | 28 | 594 | 616 |
| 13 | 264 | 286 | 29 | 616 | 638 |
| 14 | 286 | 308 | 30 | 638 | 660 |
| 15 | 308 | 330 | 31 | 660 | 682 |
| 16 | 330 | 352 | 32 | 682 | 704 |

Note:	MSO’s represent discrete points in time.

#### Time Slot Rotation and “Channels”

- Time Slot resources assignable to the Ground Station shall be made on a continually shifting basis.

Note:	This assignable resource will be subsequently referred to as a “Channel” to distinguish it from a Time Slot.

- Channel numbers 1-32 shall increment by 1 Time Slot per second (modulo 32).

- The Channel number and Time Slot number shall be equal at midnight UTC time and every 32 seconds thereafter (see Figure 3-2).

Note:	With the addition of a leap second the ground station does not shift slots from the previous second.  With the subtraction of a leap second, a shift of slots is omitted.

Figure 3-2: Relationship of “Channel Numbers” to Time Slot Numbers

Note:	The reason for the Time Slot rotation is to make aircraft reception of Ground Uplink messages from a given Ground Station robust in the presence of time synchronized sources of interference in the band.

#### Transmission of Ground Uplink Message

- Ground Uplink message transmissions shall begin at the start of the Time Slot determined by the next available assigned Channel.

Note:	The duration of a Ground Uplink message is approximately 1.5 milliseconds less than the Time Slot duration.  This additional time provides a propagation guard time when adjacent Channels (Time Slots) are assigned to Ground Station ground station sites with common line of sight to the same aircraft.

# Interface Requirements for the Airborne Equipment

## UAT Receiver Output Requirements (Report Generation)

ADS-B reports are output from the receiver in response to UAT messages received.

### Receiver Time of Message Receipt

The receiver shall declare a Time of Message Receipt (TOMR) and include this as part of the report issued to the on-board application systems.  The TOMR value shall be reported to within the parameters listed below:

- Range of at least 25 seconds expressed as seconds since GPS midnight modulo the range.

- Resolution of 100 nanoseconds or less.

- Accuracy of +/- 500 nanoseconds of the actual time of receipt for UAT equipment using either an internal, or external UTC coupled time source.

- The reported TOMR will be equal to the following quantity: seconds since the previous UTC midnight modulo the specified TOMR range.

Note: 	TOMR is required to support ADS-B Time of Applicability (TOA) and range validation by a receiving application.  See Appendix I for a discussion of UAT Timing Considerations.  ADS-B applications derive the TOA from the TOMR as follows:

- If the report indicates UTC Coupled, and is in the Non-Precision Condition, the TOA is the TOMR truncated to the start of the UTC second.

- If the report indicates UTC Coupled, and is in the Precision Condition, the TOA is the TOMR truncated to the start of the 0.20 second UTC epoch containing the TOMR.

- If the report indicates Non-UTC Coupled, the TOA is the TOMR minus one (1) second.

### Report Assembly on Receipt of ADS-B Message

Reports shall contain the following information:

- All elements of the received message payload applicable to the ADS-B report type with the range, resolution and units of each payload field preserved.

Note:	Report types are State Vector, Mode Status, Air Reference Velocity, Target State, and Trajectory Change.  See RTCA DO-242A and appropriate UAT certification standards for more details on Report types.

- The Time of Message Receipt (§4.1.1) value measured by the receiver.

Note:	Time of Applicability may be derived by the receiving application from the Time of Message Receipt.

### Report Assembly on Receipt of Ground Uplink Message

Reports may contain the following information:

- The 432 bytes of unaltered received message payload.

- The Time of Message Receipt (§4.1.1) value measured by the receiver.

Note:	Time of Applicability may be derived by the receiving application from the Time of Message Receipt.

### Message Reception-to-Report Completion Time

All ADS-B Applicable Messages shall be output from the Report Assembly Function within 200 milliseconds of message input.

All Ground Uplink Applicable Messages shall be output from the Report Assembly Function within 500 milliseconds of message input.

Action #8: Confirm inclusion of this requirement from the RTCA UAT MOPS into the Technical Manual.

## Mutual Suppression

- All UAT equipment classes shall provide an output suitable for sending suppression signals.

- No UAT equipment of any equipment class shall respond to suppression signals.

Note:	UAT equipment is not to inhibit or delay its transmissions based on suppression signals.  There is no need to desensitize the UAT receiver based on suppression signals.

Reminder: Complete coordination of this requirement with SCRSPS.

Action Item Assignments:

- Action #1: Rich Jennings

- Action #2: Rapporteur

- Action #3: Bob Saffell

- Action #4: Rapporteur/Secretariat

- Action #5a/5b: Bob Saffell

- Action #6: Rich Jennings

- Action #7 and 8: UAT Subgroup @ its Sept 12 2003 teleconference

- Comment from Warren Wilson

- Replace 3.2.2.2.b with the following:

- b. Channel numbers shall increment by 1 Time Slot per second according to the following rule:

- Time Slot =1+ (Channel number +UTC second -1) mod 32